3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500556

Online, 20th Jan – 24th Jan, 2024

Source:	Huawei, HiSilicon

Title:	Discussion on Identification of non-3GPP devices behind 5G-RG

Document for:	Discussion

Agenda Item:	19.8.2

Work Item / Release:	Rel-19

Abstract: This paper is about some possible work to be done when reusing and expanding the Connectivity Group id.

1. Introduction

Agreed S2-2412558 with CR 2136 has the following:

Figure X-1: Example scenario for mapping traffic of individual non-3GPP devices to a PDU Session

As in this example, two non-3GPP devices mapped to PDU Session A initially used the default QoS Flow (QFI 1); when differentiated QoS is requested for one device, the 5G-RG binds its traffic to a Non-3GPP Device Identifier, and its traffic is mapped to a separate QoS Flow (QFI 2). Four non-3GPP devices mapped to PDU Session B based on their Connectivity Group ID X initially used the default QoS Flow (QFI 3); when differentiated QoS is requested for two of those four devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to separate QoS Flows (QFI 4 and QFI 5). Similarly, three non-3GPP devices mapped to PDU Session C based on their Connectivity Group ID Y initially used the default QoS Flow (QFI 6); when differentiated, but the same, QoS is requested for two of those three devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to a separate QoS Flow (QFI 7).

It is agreed that the traffic from a non-3GPP device with a Non-3GPP Device Identifier is mapped to a PDU Session based on the Connectivity Group ID. In this scenario, some possible solutions are proposed to achieve QoS differentiation for non-3GPP device traffic when Connectivity Group ID is reused.

2. Discussion

Observation 1: According to 23.316, “Connectivity Group ID” is defined on the 5G-RG where each Connectivity Group ID corresponds to a separate physical or virtual port on 5G-RG. The 5G-RG is configured with the port information including the Connectivity Group ID for each port.

Proposal 1: The definition of the “Connectivity Group ID” on the 5G-RG and how can 5G-RG get it needs to be clarified.

Observation 2: Based on the solution in the introduction, 5G-RG still uses the Connectivity Group ID to map the parameters of the PDU Session used to carry the traffic of Non-3GPP devices with a device id e.g. DNN, S-NSSAI, etc.

According to the S2-2412779 (CR4848), it is agreed that the Non-3GPP Device Identifier(s) and their corresponding information including the DNN and S-NSSAI associated with as 5G-RG subscription are provided into the UDR by the AF.

After the connection of the non-3GPP device and the 5G-RG, the 5G-RG binds a Non-3GPP Device Identifier #1 to the Non-3GPP device. Then the 5G-RG uses the configured Connectivity Group ID to map a PDU Session#1. After the 5G-RG sends the PDU Session Establishment Request including the Non-3GPP Device Identifier#1, the network will check the Non-3GPP Device Identifier’s information including the DNN and S-NSSAI in the 5G-RG subscription. However, the DNN and S-NSSAI related with the Non-3GPP Device Identifier#1 in the 5G-RG subscription data may be different from the parameter of the PDU Session#1.

The 5G-RG needs to get the proper association between the Connectivity Group ID and the Non-3GPP Device Identifier so that it can assure the information related to the Non-3GPP Device Identifier is consistent with the parameter of the PDU Session mapped with the Connectivity Group ID.

Proposal 2: An association between the Connectivity Group ID and the Non-3GPP Device Identifier needs to be generated and sent to the 5G-RG.

3. Conclusion and proposal(s)

In this document, we have provided our considerations if using the Connectivity Group id in the UIA scenario. Our proposals are summarized below.

Proposal 1: The definition of the “Connectivity Group ID” on the 5G-RG and how can 5G-RG get it needs to be clarified.

Proposal 2: An association between the Connectivity Group ID and the Non-3GPP Device Identifier needs to be generated and sent to the 5G-RG.

The corresponding CR includes S2-2500557 (23.316 CR 2146).