3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500557

# Online, 20th Jan – 24th Jan, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2146 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifying non-3GPP devices behind 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the discussion paper (S2-2500556), this CR would like to clarify how can the 5G-RG get the association of the Non-3GPP Device Identifier and the Connectivity Group ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define how to generate and deliver the association of Connectivity Group ID and the Non-3GPP Device Identifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The parameter of the PDU Session related with the Connectivity Group ID may be different with the subscription data related with the Non-3GPP Device Identifier, which will lead to the PDU Session establishment/modification failure. |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex C (informative):
QoS differentiation of traffic of individual non-3GPP devices behind 5G-RG

This Annex describes how the traffic of individual non-3GPP devices behind a 5G-RG can be identified and provided with differentiated QoS.

Figure X-1: Example scenario for mapping traffic of individual non-3GPP devices to a PDU Session

As in this example, two non-3GPP devices mapped to PDU Session A initially used the default QoS Flow (QFI 1); when differentiated QoS is requested for one device, the 5G-RG binds its traffic to a Non-3GPP Device Identifier, and its traffic is mapped to a separate QoS Flow (QFI 2). Four non-3GPP devices mapped to PDU Session B based on their Connectivity Group ID X initially used the default QoS Flow (QFI 3); when differentitated QoS is requested for two of those four devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers based on the association of the Connectivity Group ID and the Non-3GPP Device Identifier, and their traffic is mapped to separate QoS Flows (QFI 4 and QFI 5). Similarly, three non-3GPP devices mapped to PDU Session C based on their Connectivity Group ID Y initially used the default QoS Flow (QFI 6); when differentiated, but the same, QoS is requested for two of those three devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to a separate QoS Flow (QFI 7).

Figure X-2 illustrates a procedure which enables the 5GS to identify the traffic of individual non-3GPP devices initially using the same PDU Session behind a 5G-RG and provide differentiated QoS.

Non-3GPP

device 1

Non-3GPP

device 2

5G-RG

SMF

PCF

UDM

UDR

NEF

AF

0a. Non-3GPP device 1 connects to 5G-RG

0b. Establishment PDU Session

for non-3GPP device 1

0c. Non-3GPP device2

connects to 5G-RG

0d. Determine PDU

Session

for non-3GPP device2

1. Request service differentiation for non-3GPP device 2

2. AF provisions Non-3GPP Device Identifier Information for device 2 into UDR

3. 5G-RG configuration update procedure

4a. 5G-RG requested PDU Session Modification for non-3GPP device2

4b. 5G-RG initiated PDU Session Establishment for non-3GPP device2

5. Response to user

Figure X-2: Example procedure for enabling QoS differentiation for individual non-3GPP devices behind 5G-RG

0a. Non-3GPP device 1 is connected to the 5G-RG.

0b.	To provide connectivity to the non-3GPP device 1, the 5G-RG implements the existing behaviour of either using the URSP rule (optionally containing the Connectivity Group ID as described in clause 4.10b), or using UE Local Configuration, to map the traffic of the non-3GPP device 1 to a PDU Session.

0c. Non-3GPP device 2 is connected to the 5G-RG.

0d. To provide connectivity to the non-3GPP device 2, the 5G-RG implements the existing behaviour of either using the URSP rule (optionally containing the Connectivity Group ID as described in clause 4.10b), or using UE Local Configuration, to map the traffic of the non-3GPP device 2 to the same PDU Session as non-3GPP device 1.

1.	The 5G-RG subscription owner or an authorized user, using mechanisms out of scope of 3GPP, requests differentiated QoS for the non-3GPP device 2 through the AF.

NOTE 1: 	The request for differentiated service can be made through an operator portal hosted either in the 5G-RG or in the AF.

2.	AF provisions the Non-3GPP Device Identifier Information for the non-3GPP device 2 into the UDR, as defined in clause 4.15.6.X of TS 23.502 [3].

NOTE 2:	Provisioning of the Non-3GPP Device Identifier Information into the UDR is done only for the non-3GPP devices that require differentiated QoS. This provisioning could be done before a device is connected to the 5G-RG.

3. To assure that the DNN/S-NSSAI of the PDU Session is contained in the Non-3GPP Device Identifier Information, 5GC generates the association between the Connectivity Group ID and the Non-3GPP device Identifier and sends the association to the 5G-RG.

4.	If the PDU Session for non-3GPP device 2 is established using the Connectivity Group ID, the 5G-RG will bind the Non-3GPP device Identifier with the Non-3GPP device based on the association between the Connectivity Group ID and the Non-3GPP device Identifier. Based on the network configuration, either a 5G-RG requested PDU Session Modification, or a 5G-RG requested PDU Session Establishment procedure is triggered.

4a. (5G-RG requested PDU Session Modification) As defined in Clause 7.3.2.

4b. (5G-RG requested PDU Session Establishment) As defined in Clause 7.3.1.

Editor’s note:	Whether the PCF initiated SM Policy Association Modification procedure as defined in clause 4.16.5.2 of TS 23.502 [3] can also be used is FFS.

5.   In response to step 1, using mechanisms out of scope of 3GPP, the operator portal returns a response to the 5G-RG subscription owner or the authorized user about the completion of the differentiated QoS request for non-3GPP device 2.

# * * * * End of changes * * * *