3GPP TSG SA WG2#166-Ad Hoc-e	S2-2500580

20 – 24, January 2025, Online

Title:	Reply LS on FS_VMR_Ph2 solution impacts to RAN

Response to:	LS in R3-247910 (S2-2500053) on FS_VMR_Ph2 solution impacts to RAN

Release:	Rel-19

Study Item:	FS_VMR_Ph2

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact person:

Name:	Meng Li

Tel. Number:

E-mail Address:	raymond.limeng@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

SA2 would like to thank RAN3 for their LS on FS_VMR_Ph2 solution impacts to RAN (R3-247910/S2-2500053).

SA2 has discussed the CN impact of potential solutions that allow change of the UE’s AMF with a single logical WAB-gNB and would like to provide the feedback as following:

Solution 1: Using single WAB-gNB with single cell and changing TAC

    - SA2 has different understandings of the assumptions that whether the WAB with 2 TACs connects to two different AMF set simultaneously and identifies two alternatives:

      - Option 1A(WAB with 2 TACs connects to two different AMF sets simultaneously):

          - CN impacts: Initial AMF triggers AMF re-allocation during Mobility Registration Update based on the new TAC included in the N2 message for both UE in RRC CONNECTED state and UE in CM-IDLE state.

      - Option 1B(WAB with 2 TACs can NOT connect to two different AMF sets simultaneously):

          - WAB-gNB updates the TAC in the SI and routes the subsequent UL messages to the new AMF.

          - The UE performs Mobility Registration Update, which is forwarded by the WAB-gNB to the new AMF.

          - The leftover procedures for CN reuse the existing ones.

          - WAB-gNB rejects the DL signaling from the initial AMF with the existing cause value before the Mobility Registration Update is finished.

          - WAB-gNB releases the N2 connection with initial AMF after all connected UE are served by the new AMF.

          - CN impacts: The existing mechanism can be reused without CN impacts, for UE in RRC CONNECTED state and UE in CM-IDLE state.

    - SA2 discusses the feasibility of simultaneous NG connection towards different AMF sets for one gNB.

Solution 2: Using single WAB-gNB with two cells and different TACs

- SA2 thinks this solution assumes the WAB with 2 TACs connects to two different AMF sets simultaneously.

- CN impacts: When UE is CM-CONNECTED state, Source AMF selects the Target AMF during N2 handover based on the new TAC only included in the Target ID. When UE is in CM-IDLE state, Initial AMF triggers AMF re-allocation during Mobility Registration Update based on the new TAC.

2	Actions

To RAN3 group.

ACTION: 	SA2 would like to ask RAN3 to take the above feedback into account.

3	Dates of next TSG SA WG2 meetings

3GPPSA2#167					2025-02-17 – 2025-02-21		 			Athens, GR

3GPPSA2#168					2025-04-07 – 2025-04-11					Goteborg, SE