3GPP TSG SA WG2#166-Ad Hoc-e	S2-2500582

20 – 24, January 2025, Online

Title:	Reply LS on FS_VMR_Ph2 solution impacts to RAN

Response to:	LS in R3-247909 (S2-2500052) on FS_VMR_Ph2 solution impacts to RAN

Release:	Rel-19

Study Item:	FS_VMR_Ph2

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact person:

Name:	Meng Li

Tel. Number:

E-mail Address:	raymond.limeng@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

SA2 would like to thank RAN3 for their LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI) (see R3-247909/S2-2500052) and would like to provide the feedback as following:

Reply to Issue-1a:

RAN3 has agreed the following solution to prevent multi-hop backhauling upon WAB-MT HO:

For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

The discussion on RAN based approach for preventing multi-hop backhauling at the WAB-MT’s initial access is still ongoing in RAN3.

SA2 would like to ask RAN3 whether the above solution is applied to both Xn based handover and N2 based handover.

Reply to Issue-2:

RAN3 discussed this issue and the following WA was achieved:

Additional ULI for WAB consists of TAI and NR CGI, which are determined by the WAB-node, reflecting the WAB-node’s physical location. This solution is compliant with Option 1 and Option 3. It is up to SA2 to support one of Option 1 and Option 3 or both.

RAN3 is still discussing whether there are issues with applying this solution to WAB-MT access via NTN.

SA2 has discussed Option 1 and Option 3, and would like to clarify the following aspects:

- Analysis of different options for Additional ULI:

SA2 identifies the following scenario that both Option#1 and Option#3 cannot address:

When the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are different, the deployment and the coverage of fixed gNB of the two PLMNs could be different. In this case, there could be no mapped TAC/Cell ID between the PLMNs where the UE is geographically located.

Considering the Option#2 is already ruled out by RAN3, SA2 has a slight preference on the Option#3 “TAC/Cell ID of the cell serving the MWAB-UE in other PLMN”, considering the comparison of Option#1 and Option#3 regarding the precision in case of MWAB-UE using NR NTN to connect to the backhaul PLMN.

- Additional ULI in case of WAB-MT served by a satellite NG-RAN:

The current mechanism for NR satellite access is described in the TS 23.501:

If the AMF requests UE location, in the case of NR satellite access, the NG-RAN provides all broadcast TAIs to the AMF as part of the ULI. The NG-RAN also reports the TAI where the UE is geographically located if this TAI can be determined.

The TS 38.413 also defines the NR NTN TAI Information included in the N2 message:

9.3.3.53 NR NTN TAI Information This IE contains the TAI information for NR NTN. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Serving PLMN; M; PLMN Identity 9.3.3.5; Indicates the UE’s serving PLMN.. TAC List in NR NTN; 1..<maxnoofTACsinNTN>; Includes all TAC(s) broadcast in the cell, for the UE’s serving PLMN.. >TAC; M; 9.3.3.10. UE Location Derived TAC in NR NTN; O; TAC 9.3.3.10; This IE contains TAC information derived from the actual UE location, if available..

Considering the existing format of “NR NTN TAI information” for satellite access, it is proposed to include all broadcasted TAIs for the selected PLMN of WAB-UE in the Additional ULI.  In addition, when WAB-UE’s selected PLMN is not same as the PLMN broadcast by the MWAB-gNB, SA2 thinks the design principle of Additional ULI for the both TN and NTN should be unified, but it is under the remit of RAN3 to make the decision.

2	Actions

To RAN3 group.

ACTION: 	SA2 would like to ask RAN3 to take the above feedback into account.

3	Dates of next TSG SA WG2 meetings

3GPPSA2#167					2025-02-17 – 2025-02-21		 			Athens, GR

3GPPSA2#168					2025-04-07 – 2025-04-11					Goteborg, SE