3GPP TSG-WG SA2 Meeting #166 Ad Hoc	S2-2500586

20th – 24th January 2025, E-meeting	(revision of XXXXXX)

Source:	Lenovo, China Telecom

Title:	KI#1 Addressing EN on AIoT device related information

Document for:	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to resolve the ENs for the general part of the KI#1 conclusions, especially on the AIoT device related information.

1	Discussion

This paper mainly focuses on solving the existing EN in the General part for KI#1 conclusions, specifically regarding the AIoT device related information.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:  Where to store the AIOT device related information is FFS.

As agreed in the last meeting, the AIoT device related information contains at least two parts, the AIoT device permanent ID and the last known reader information. Compared to the AIoT device subscription-like information (e.g., credential holder information, authentication results for the AIoT devices, etc, which is assumed to be covered by other paper), the AIoT device related information is more dynamic (for the reader information part) and is assumed to be updated more frequently, which makes it more like the concept of AIoT device context. Therefore, it makes sense to store this kind of information at the AIOTF, which is the hub for handling the AIoT traffic and services in the core network. By using this AIoT device related information, the AIOTF can handle the service more efficiently when it comes to certain devices.

Proposal 1: The AIoT device related information (also known as the AIoT device context), can be stored and updated at the AIOTF.

As for how the AIoT device related information is used, the last known reader information, also known as the reader-device binding table, can be used by the AIOTF to check and find the serving reader for the specific AIoT device, and then forward the message (inventory and command) towards the device via the serving reader. With this information stored at AIOTF, it does not to initiate a new round of inventory every time when wanting to send the message to a specific AIoT device (assuming that the device and reader binding relationship has not changed after the last round of inventory). What’s more, AIOTF can update the locally stored reader information for the AIoT device, when realizing that the serving reader for the device has changed.

Proposal 2: The AIoT device related information, e.g., the last known reader information, can be used by the AIOTF to find the serving reader for the AIoT device.

With the discussion above, these two Editor’s notes can be removed accordingly.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

8.1	Interim Conclusion on Key Issue #1

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the BS readers or AIOT RAN nodes. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

Editor’s note:	Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 needs coordination between SA WG2 and RAN WG(s).

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

5.	The network, e.g., AIOTF, may store and update the AIoT device related information (also known as device context information) locally, that includes, e.g., the AIOT device permanent ID, the last known reader information of the AIoT device, and optional the AIoT device authentication result. The last known reader information can be used to support the AIOTF to select the serving reader to forward the message towards the specific AIoT device(s).

NOTE: The storage of the AIoT device authentication result at the AIOTF is up to the coordination with SA WG3 later.

# * * * * End of changes * * * *