3GPP TSG-WG2 Meeting #166-Ad Hoc-e	S2-2500591

# 20 - 24 January, 2025, Online		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0651 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of MTLF discovery and selection for LMF-based AI/ML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. The descriptions for MTLF discovery, ML model retrieving and ML model provisioning for LMF-based AI/ML positioning should be clarified and aligned with TS 23.288. 2. The name of the indication of supporting model training for LMF-based AI/ML Positioning needs to be simplified. |  |  |  |  |  |  |  |  |
| Summary of changes: |  | 1. Add the LMF-based AI/ML positioning indication in the MTLF discovery and selection for LMF-based AI/ML positioning based on the contents in S2-2412728. 2. Modify the contents of MTLF discovery, ML model retrieving and ML model provisioning for LMF-based AI/ML positioning. 3. Change the name of indication to LMF-based AI/ML positioning indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent descriptions for MTLF discovery/selection, ML model retrieving and ML model provisioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18, 6.22.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.18	Support for UE positioning based on a ML Model at the LMF

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request for a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The ML model that is used for LMF-based AI/ML Positioning in LMF may be trained by LMF. The trigger for data collection and model training in LMF is up to implementation. LMF collects input data from UE for ML model training is described in clause 6.22.2. The LMF collects input data from NG-RAN for ML model training is described in clause 6.22.3.

The LMF may also request a trained ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF as described in clause 6.22.5. The LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37] with the following considerations:

- LMF includes in the request the LMF-based AI/ML positioning indication to discover an NWDAF containing MTLF that supports model training for the LMF-based AI/ML Positioning.

- If the ML Model is for a specific positioning case, then the LMF may provide a request to retrieve an AI/ML Model for UE Positioning including the positioning case information.

- The LMF provides an Area of Interest, and may provide the ML Model Interoperability indicator to discover an NWDAF that can provide an AI/ML Model that the LMF can run.

- The LMF may requests a NWDAF with ML Model accuracy checking capability to be able to report to LMF that the model is degraded (i.e. for ML model performance monitoring).

NOTE x: Other NWDAF discovery parameters listed in clause 5.2 such as Analytics ID, FL capability and related time period, S-NSSAI or any roaming capabilities are not required by LMF.

Editor´s Note:	The types of LMF-side AI/ML positioning model (e.g., per positioning case) is FFS, which needs to be coordinated with RAN WGs.

Editor’s Note: Whether and which additional parameters are considered when LMF requesting model positioning is FFS.

The LMF requests the NWDAF to provide an AI/ML Model for LMF-based AI/ML Positioning as described in clause 6.2A of TS 23.288 [37] with the following considerations:

- LMF-based AI/ML positioning indication.

- ML Model Filter Information, which may indicate the Area of Interest.

- The Notification Target Address to receive the ML Model, the positioning case information, and the Target of ML Model Reporting set to any UE.

- If vendor specific information is needed to allow running the ML Model, then the ML Model Interoperability Information.

- If the LMF supports multiple AI/ML Models, the support for multiple AI/ML Models, optionally with the maximum number of ML Models.

Editor’s Note: whether multiple model identifiers can be provided is FFS.

The NWDAF containing MTLF collects input data to perform the ML model training as described in clause 6.22.4, and the NWDAF containing MTLF performs ML model provision to LMF as described in clause 6.2A of TS 23.288 [37] with the following considerations.

- The NWDAF provides the ML Model identifier and ML Model Information (i.e. ML Model file address) for the ML Model for UE Positioning and indication of whether the ML Model identifier is updated if model is retrained and optionally the ML Model accuracy Information.

- Parameters related to the AI/ML Model to be trained, i.e., the ML Model Target Period and desired model metrics.

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

Once the ML model for LMF-based AI/ML Positioning is trained and located in the LMF, the LMF may use it to perform UE Positioning after receiving a location determination request from AMF.

Editor´s Note: Whether user consent for UE positioning calculation is needed apart from the existing GMLC check of the LCS privacy profile is FFS

For training, performance monitoring and UE positioning calculation using LMF-based AI/ML Positioning, the LMF checks with UDM the user consent status before collecting UE related data, see clause 6.22.3 and clause 6.22.4.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. When the ML model that is used for LMF-based AI/ML Positioning is trained by LMF the LMF monitors the performance of the ML model.  When the ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF monitors the performance of the ML model.  LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result.

# * * * * Next change * * * *

6.22.5	LMF retrieval of a trained AI/ML Model for positioning

The LMF may retrieve a trained AI/ML Model from NWDAF. The LMF obtains a trained AI/ML model prior to a UE positioning request that requires a trained ML Model.

Figure 6.x.3-1: The LMF retrieves a trained AI/ML based positioning model from NWDAF

- The LMF determines using local policies, that a positioning method requires a trained ML Model, the trained ML Model is not trained locally but is retrieved from NWDAF.

- The LMF discovers the NWDAF that can provide a trained model, using Nnrf_NFdiscovery including the LMF-based AI/ML positioning indication, ML Model Filter Information that includes the positioning case information and an Area(s) of Interest, the ML Model Interoperability indicator applicable to the trained ML Model for UE Positioning, optionally the ML Model Accuracy checking capability.

NOTE 1: 	NWDAF discovery and selection is described in clause 5.2 of TS 23.288 [37], the applicable NWDAF parameters used for discovery of an NWDAF supporting an AI/ML Model for UE Positioning are described in clause 5.18.

- The LMF receives a list of candidates NWDAF instances, validity period for the discovery, per NWDAF instance the NWDAF instance ID, FQDN or IP address, and the ML Model Interoperability Indication.

- The LMF selects a NWDAF out of the list of candidates NWDAF instances, that can provide a ML Model that supports the same ML Model Interoperability Indication then request a trained AI/ML Model using Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe including the request for a trained AI/ML Model for UE Positioning according to the information listed in clause 5.18.

Editor´s Note: Whether the LMF provides Inference Input Data information to NWDAF in the Nnwdaf_MLModelProvision service is FFS.

- The NWDAF provides one or more AI/ML Models to LMF including a ML Model Identifier and the ML Model Information as described in clause 5.18.

- Same as in step 5.

The LMF updates the received trained ML Model(s) when the NWDAF provides new ones.

# * * * * End of changes * * * *