3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500611

# E-meeting, 20 – 24 January 2025		(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0288 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Clarfication on the I-SMF selected for local offloading managment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At S2#166 meeting, it is agreed that when the UE in the local offloading management service area, the AMF selects an I-SMF for local offloading. For this selected I-SMF one question may be raised whether the selected I-SMF need be adjusted per target DNAI notified by the SMF, i.e. due to dedicated AF traffic. The argument to support this adjustment is that when the AMF selects the I-SMF no data traffic has been transferred via this PDU session. So the selected I-SMF may be the one selected randomly.However this is incorrect understanding and need be clarified. For one PDU session, it may include serval different AF traffic within this PDU session. From network view it does not know when and which AF traffic will be transferred within the PDU session. However it is expected that the selected I-SMF should be able to steer the AF traffic within the PDU session to different L-PSA within the local offloading management service area. This means the selected I-SMF should support that capability regardelss when and which AF traffic is to be transferred. It is not expected that the selected I-SMF need be adjusted per different AF traffic within this PDU session. If I-SMF need adjustment, indeed this means that the selected I-SMF can not cover the whole local offloading management service area. This is not suitable network configuration as this will cause following issues: - Extral signaling are introduced. For each adjustment the extral signaling are introduced. If local offloading managment is handled like this, this will cause a lot of extra siganling at the network per ongoing AF traffics. - How to do this adjustment? If I-SMF need be adjusted, it means the selected I-SMF can not select and configure an UPF to offload the indicated AF traffic. But how to handle other ongoing AF traffic which has been steered to the L-PSA(s)? So does the I-SMF also need send the previous selected DNAI as part of the target DNAI to the AMF? Then how to consider other DNAI not selected now but maybe selected later? Also does the I-SMF need keep monitor AF traffic to avoid that the AF traffic which is not transferred anymore but the related DNAI is still be considered? Taking above into consideration, we assume that per network configuration the selected I-SMF should support to select and configure all potential UPF which supports the indicated local offloading management service area and connect to the DN. And the selected I-SMF can be identified by it can serve that dedicated local offloading management service area. Hence the discvoered and selected I-SMF is the best one for local offloading management in that dedicated local offloading area. This is independent of whether AF traffic(s) are transferring and offloaded or not. By doing above, it can be seen that after the AMF select the I-SMF for local offloading management, as the selected I-SMF(s) is the best one to serve that area, it is not expected that SMF need notify the target DNAI to the I-SMF. Even in case one dedicated DNAI not supported by the I-SMF due to synchornization issue, but no other I-SMF than the selected I-SMF(s) in the better place to serve all potential AF traffic within that area. In a short, the above analyise can be summarized as : - The selected I-SMF for local offloading management need always be ready to select a L-PSA within that area for potential AF traffic. This is independent of whether the AF traffic is ongoing. - The selected I-SMF is the best one which can be selected to serves the indicated the local offloading management service area. Any adjustment due to new AF traffic within the PDU session is not needed. The I-SMF reallocation is only triggered due to UE mobility. It is proposed to claify above two point and avoid any different understanding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The AMF selects the I-SMF which serves the local offloading service area. And by network configuration, the I-SMF is able to offload traffic within the local offloading management service area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear whether the selected I-SMF for local offloading management need be adjusted per AF traffic within the PDU session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.10	Support of the Local Offloading Management

6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects an I-SMF to support Local Offloading Management, i.e., the selected I-SMF supports the indicated Local Offloading Management service area. This also includes the case when the UE is within the SMF service area. The selected I-SMF shall be ready to select and configure UPF(s), which supports the related Local Offloading Management service area and connect to the DN.

NOTE 1:  If the I-SMF for local offloading management has been selected by the AMF, the I-SMF reallocation is only triggered due to UE mobility.

For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 2:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 3:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF. as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

# * * * * End of changes * * * *