SA WG2 Meeting #166-Ad-Hoc-e	S2-2500635

20 - 24 January, 2025, Electronic meeting                  	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1554 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Address EN on Avatar Representation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper is proposed to address the following editor’s note regarding Avatar representation. In clause AC.11.1: Editor's note: Whether the personal asset information (e.g. identities of accessories, garments) can be provided with Avatar IDs and Avatar Representation is FFS. In SA4 TR 26.813, the following terms is defined in clause 3.1: Base Avatar: A personalized avatar representation. The base avatar model conforms to a predetermined avatar representation format. Avatar Representation Format: A format for a representation that describes a model for an avatar. And in clause 6.3.4, the description for Avatar representation format is given: The representation baseline format is expected to contain the following data: - High-level avatar information: - Name - Identifier - Age - Gender - Representation data: - reference template - mesh: one or more geometries (LoDs). - body mappings: a list of string with associated geometries to identify the body parts - skeletons: one or more skeleton skins - controllers: one or more animation controllers (inc. facial blend shapes, joints, morph targets) - Additional information: - Model mappings: one or more mapping function (reference template to B) - Animations: a set of predefined animations - Processing functions - Avatar gaze - Parametric textures - Hair - Accessories It is observed that the Avatar Representation includes diverse information and ‘Accessories’ is one of them. It is proposed to change the Editor’s Note to a Note to clarify that Avatar Representation is defined by SA4. In this specification, there is no need to define and describe the Avatar representation. For the following EN in clause AC.11.1: Editor's note: Whether the Avatar Representation can be downloaded through bootstrap data channel is FFS. Since an Avatar application may use many Avatar Representations, it is reasonable to download the Avatar Representation only when running the application. It will be better to download the Avatar Representation through application data channel. It is proposed to remove the EN and modify the corresponding text in clause AC.11.1 and AC.11.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Address the EN regarding Avatar Representation in clause AC.11.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s notes exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.11.1, AC.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AC.11.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) is downloaded to the UE by following options:

-	Pre-configured in the UE: The Avatar ID List is provisioned or downloaded to the UE before a data channel for avatar call is setup. The UE and the BAR may interact by means out of the scope of 3GPP.

-	Through bootstrap data channel: The Avatar ID List is fetched by the DC AS from the BAR, and transferred from the DC AS to the DCSF, and downloaded to UE through bootstrap data channel.

-	Through application data channel: The Avatar ID List is fetched by the DC AS from the BAR, and downloaded to the UE through application data channel.

Avatar ID List contains the list of Avatar IDs that are identifying the Avatar Representation and/or the associated information. The associated information may include the information for the user selection in the avatar calls (e.g. the metadata associated with the Avatar such as thumbnails, URLs, the associated data channel applications).

NOTE X:	The Avatar Representation is defined by SA4.

For downloading of Avatar ID List through bootstrap data channel, the DC AS URL(s) may be pre-configured in the DCSF, or fetched by DCSF. When the downloading of Avatar ID List is requested through bootstrap data channel, the DCSF requests DC AS to fetch the Avatar ID List from the BAR for the authorized user of the UE.

Editor's note:	How the DCSF fetches DC AS URLs is FFS.

The BAR and/or the DC AS may be deployed either in the trusted domain or untrusted domain of operator's network.

After selection of Avatar ID to use in the avatar call by the user of the UE, the Avatar Representation identified with the selected Avatar IDs may be provided from the BAR to MF, and it may be transferred to the local UE (if not locally available) and/or to the remote UE according to the rendering mode via application data channel if needed.

The avatar media is rendered by processing the Avatar Representation with the animation data.

-	The avatar media rendering process may be performed by the sending UE, the receiving UE, or the network according to the rendering mode.

-	The animation data may be generated by the sending UE, the receiving UE, or the network (e.g. from audio, video, and/or metadata such as UE sensors collected data).

NEXT CHANGE

AC.11.2	Architecture

Figure AC.11.2-1: Architecture to support Avatar communication

To support Avatar communication, the data channel architecture and functions described in clause AC.2 is enhanced as below:

Base Avatar Repository (BAR):

-	The BAR stores Avatar Representation, its Avatar ID, and the associated information as described in clause AC.11.1. One or more Avatar Representation(s), is stored for a user, and each Avatar Representation is identified with Avatar ID.

-	The BAR may provide Avatar ID List to the DC Application Server, and the DC Application Server may further provide to DCSF.

-	The BAR provides Avatar Representation and the associated information to the MF directly or after URL redirection from the DC AS.

DCSF:

-	If the Avatar ID List is downloaded through bootstrap data channel, the DCSF may fetch the Avatar ID List from the DC AS, and send to MF via MDC1.

MF:

-	The MF supports the UE downloading the Avatar ID List and associated information through bootstrap data channel or application data channel.

-	The MF supports the UE downloading the Avatar Representation through application data channel.

-	The MF may retrieve the Avatar Representation and the associated information from BAR via MDC2.

-	The MF may support Avatar media processing and transcoding (e.g. ASR/TTS).

DC AS:

-	The DC AS may provide the DC AS URL(s) to DCSF via DC3/N33 or DC4 for downloading of Avatar ID List through bootstrap data channel.

-	The DC AS retrieves the Avatar ID List from BAR(s) via MDC2.

-	The DC AS sends the Avatar ID List either to DCSF via MDC3 for downloading through bootstrap data channel, or to MF via MDC2 for downloading through application data channel.

-	The DC AS may retrieve the Avatar Representation and the associated information from BAR and sends to MF via MDC2.

-	The DC AS supports Avatar media negotiation and provides the instruction for Avatar media processing.

END OF CHANGES