SA WG2 Meeting #166-Ad-Hoc-e	S2-2500642

20 - 24 January, 2025, Electronic meeting	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TS 23.xyz: Ambient IoT Services

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: The paper adds Ambient IoT service definition.

1. Discussion

The paper defines the Ambient IoT services which include Inventory, Command (Read, Write, or Disable) service operations.

In TR 38.769, the following definitions for AIoT services are given:

Inventory: The service provided by the network to discover and acquire the identifier of A-IoT device(s).

Command: The service provided by the network to send the operation instruction to the A-IoT device (e.g. read, write, etc.).

In TR 23.700-13, it is concluded the following Ambient IoT services are supported by the 5GC:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled.

-	Enable: request to enable a temporarily disabled AIoT Device.

NOTE:	The security aspect of Enable and Disable is to be concluded by SA WG3.

Editor's note:	It is FFS whether and how to support enabling temporarily disabled AIoT devices.

Based on the above definitions and conclusions, it is proposed to add Ambient IoT service description in TS 23.xxx which includes inventory, command (read, write, disable) services. Since the support for temporarily disable and enable services are still under discussion, an editor’s note is added to address this.

2. Proposal

It is proposed to capture the following changes into TS 23.xyz.

* * * * Start of Changes * * * *

5.2	Ambient IoT Services

In this release, the following Ambient IoT services are supported:

-	Inventory: Request to discover and acquire the identifier of AIoT Device(s).

The inventory service operation can target to one or multiple AIoT Devices, or all AIoT Devices in a target area.

-	Command: Request to send the operation instruction to the AIoT Device(s).

The command service further includes the following subtype service operations:

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

Editor's note:	It is FFS whether and how to support temporarily disable and enable service operations.

* * * * End of Changes * * * *