SA WG2 Meeting #166-Ad-Hoc-e	S2-2500643

20 - 24 January, 2025, Electronic meeting	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TS 23.xyz: Assistance Information Used by AIoT RAN

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: The paper adds description of assistance information used by AIoT RAN.

1. Discussion

In TR 23.700-13, it is concluded the following assistance information will be used by AIoT Reader for service operation:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

In RAN TR 38.769, it also states it's beneficial for the reader to know an estimate of expected D2R message size, and this D2R message size information would be beneficial but it is not essential.

The paper describes assistance information used by AIoT Reader; and describes the assistance information is provided by the AF, and the AIOTF may determine the assistance information sent to the AIoT Reader based on the assistance information received from the AF.

2. Proposal

It is proposed to capture the following changes into TS 23.xyz.

* * * * Start of Changes * * * *

5.4	Assistance Information used by AIoT RAN

Assistance information is used by AIoT RAN for performing service operations, e.g. radio resource allocation. The AIOTF provides the assistance information to the AIoT Reader together with service operation requests. The AF may provide assistance information when requesting a service operation. The AIOTF determines the assistance information provided to the AIoT Reader based on the assistance information received from the AF.

The assistance information may include the following items:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT Devices;

-	approximate D2R message size.

Editor's note:  Other assistance information may be added later if necessary.

Editor's note:  How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

* * * * End of Changes * * * *