3GPP TSG-WG2 Meeting #166-Ad-Hoc-e	S2-2500658

# 20 - 24 January, 2025, Electronic meeting				                             (revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1357 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on the contents of VFL related services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the VFL procedures and the HFL service operation description in TS 23.288, this CR is proposed to update the contents of VFL related services to align with the VFL procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 6.2H.3 by adding the parameters described in procedures. Add a new clause to describe the contents of VFL inference service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The contents of VFL related services are not clear and missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.3, 6.2H.X(new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2H.3	Contents of VFL Training service

The consumers of the VFL training services may provide the input parameters in Nnwdaf_VFLTraining service or Naf_VFLTraining service or in Nnef_VFLTraining service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	(Only for preparation phase) VFL Interoperability Information that indicates the intermediate results that the VFL Server supports (e.g. activation function, gradients, type of loss), the content of the VFL Interoperability Information is not standardized in this release.

-	(Only for preparation phase) The list of samples selected by Server.

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the VFL client with the subscription.

-	(Only at the beginning of training) The parameters negotiated during the preparation phase:

-	VFL Interoperability Information to be used for training;

-	Final list of samples

-	Feature ID per VFL client

-	VFL Correlation ID: identifies the VFL procedure (i.e. training and inference).

-	[Optional] Analytic filter information

-	[Optional] Maximum response time: indicates maximum time for waiting notifications (i.e. intermediate training results).

-	[Optional] Intermediate model training information: used for updating the models of VFL clients, e.g. gradient information or loss information.

-	[Optional] sample ID(s) (e.g. for training or for validation): indicates the object(s) for which data is requested, e.g. group of UEs identified by a list of Internal-Group-Ids or External-Group-Ids.

-	[Optional] VFL accuracy check flag: identifies that the request is for using the data corresponding to the target sample ID(s) as the validation dataset to calculate the Model accuracy. (the VFL Client do not update their local model based on the intermediate model training information of this iteration)

The VFL client provides to the consumer of the ML Model training service operations the output information in as listed below:

-	(Only for preparation phase) VFL Interoperability information supported by each VFL Clients.

-	(Only for preparation phase) The list of samples selected by VFL client.

-	(Only for preparation phase) The Feature ID(s) that the VFL client supports. A Feature ID indicates that what features the VFL client can use for an Analytics ID, the values of the Feature ID are not standardized.

-	Notification Correlation Information.

-	[Optional] Intermediate training result: the output of the client local model, which is used for the VFL server to generate the final result.

-	[Optional] VFL correlation ID

-	[Optional] Analytics ID

# * * * * Next change (all new text) * * * *

### 6.2H.X	Contents of VFL Inference service

The consumers of the VFL inference services may provide the input parameters in Nnwdaf_VFLInference service or Naf_VFLInference service or in Nnef_VFLInference service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be used for inference.

-	(Only for Inference subscribe service) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the VFL client with the subscription.

-	VFL Correlation ID: identifies the VFL procedure (i.e. training and inference).

-	Target of VFL inference: indicates the object(s) for which data for VFL inference is requested, i.e. group of UEs identified by a list of Internal-Group-Ids or External-Group-Ids.

-	(Only for Inference subscribe) Subscription Correlation ID

-	[Optional] VFL inference filter

-	[Optional] Inference target period: indicates the time interval for inference.

The VFL client provides to the consumer of the VFL inference service operations the output information in as listed below:

- (Only for Inference notify) Notification Correlation Information.

- Intermediate results: the output of the client local model, which is used for the VFL server to generate the VFL inference results.VFL correlation ID

- Analytics ID

# * * * * End of changes * * * *