3GPP TSG-WG2 Meeting #166-Ad Hoc-e	S2-2500673

# Elbonia, 20-24 January, 2025		(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1358 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the preparation procedure for VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. When the VFL Server only coordinates the VFL process and does not perform local ML model training, the VFL Server does not have local data set for the VFL process, and the VFL Server cannot provide the suggested list of sample IDs to the VFL Client(s). In this scenario, it is proposed that the VFL Client(s) provides the list of supported sample IDs to the VFL Server based on, e.g., VFL Interoperability indicator, then the VFL Server can determine the final list of sample ID(s) supported by all the selected VFL Client(s) as the samples to be used in the VFL process. 2. Similar as sample IDs, If the VFL Server received the supported feature ID(s) of VFL Client(s) during the VFL Client(s) discovery procedure, the VFL Server can determine the feature ID(s) per VFL Interoperability indicator expected to be used in VFL process for each VFL Client, and send it to the VFL Client(s) during preparation phase. If the VFL Server did not receive the supported feature ID(s) of VFL Client(s) during the VFL Client(s) discovery procedure, the VFL Client(s) can provide the list of supported feature ID(s) to the VFL Server based on the feature ID provision indication, then the VFL Server can determine the feature ID(s) per VFL Client to be used in the VFL process. 3. When the NEF is involved in the procedure, the NEF shall map the SUPI(s) to GPSI(s) before sending to the AF if the NEF receives the SUPI as sample ID(s) from NWDAF, and the NEF may send the GPSI as sample ID(s) to the NWDAF and/or AF directly if the NEF receives it from NWDAF and/or AF. 4. Considering that the VFL Server and selected VFL Client(s) may support multiple VFL Interoperability indicators, so the VFL server may need to send the VFL Interoperability indicator for the specific VFL process to the selected VFL Client(s) during preparation phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify that the suggested list of sample IDs from VFL server to VFL Client(s) is optional, the VFL Client(s) may provide the list of supported sample IDs to the VFL Server based on the sample ID provision indication from the VFL Server. 2. Clarify that the VFL Server may send the feature ID(s) per VFL Interoperability indicator expected to be used in VFL process to the VFL Client during preparation phase, or the VFL Server may send feature ID provision indication to the VFL Client(s) during preparation phase. 3. Clarify how the NEF performs the sample ID mapping. 4. Clarify that the VFL server may need to send the VFL Interoperability indicator to the selected VFL Client(s) during preparation phase. 5. Remove the following EN and clarify which parameters are optional based on the reason for change part: Editor's note: Which parameters are optional, or mandatory is FFS, the list of parameters also needs to be extended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The preparation procedure for VFL is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.2.1, 6.2H.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2H.2.2.1	Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Editor´s note:	For UEs as samples, additional discussion is needed on whether UE needs to be registered or not and whether the NWDAF as VFL server can check whether UEs are registered before interacting with VFL clients.

Editor's note:	Whether the case of trusted AF as VFL Server is included in the clause for NWDAF as VFL server or in the clause for untrusted AF as VFL server is FFS.

1.	The NWDAF as VFL Server may send a Vertical Federated Learning preparation request including the Analytics ID to each of the NWDAF VFL Client(s), using Nnwdaf_VFLTraining_Request and to each of the AF VFL Clients(s), using Naf_VFLTraining_Request possibly via NEF when the VFL Client is an untrusted AF. The NWDAF as a VFL Server may also provide, the suggested VFL Interoperability Information to negotiate the intermediate results that will be used in VFL process (e.g, VFL training and/or VFL inference), the suggested list of sample IDs (e.g, SUPI(s), GPSI(s)) expected to be used in the VFL process, the sample ID provision indication, the feature ID(s) per VFL Interoperability indicator expected to be used in VFL process for each VFL Client, and the feature ID provision indication. If the VFL Clinet(s) is AF(s), the sample ID(s) may be GPSI(s). If the VFL Client(s) is NWDAF(s), the sample ID(s) may be GPSI(s) and/or SUPI(s).

NOTE:	In step 1, the feature ID(s) will only be provided to the VFL Client(s) when the VFL Server received the supported feature ID(s) of VFL Client(s) during the VFL Client(s) discovery procedure as defined in clause 6.2H.2.1.

2.	Each VFL Client checks if it can meet the ML Model training requirement. Each VFL Client checks the list of sample IDs suggested by the VFL Server and then provides to the VFL Server the list of sample IDs that it can accept within the sample IDs suggested by the VFL Server. If the suggested list of sample IDs is not provided by the VFL Server, the VFL Client provides the list of supported sample IDs to the VFL Server. Each VFL Client checks the VFL Interoperability Information and determines which VFL Interoperability information that the VFL Client accepts. The VFL Client determines the feature ID(s) that will be used in VFL process when the feature ID(s) was provided by the VFL Server; alternatively, the VFL Client may provide the list of supported feature ID(s) to the VFL Server; alternatively, the VFL Client may not want to reveal feature ID(s) and the VFL Server may know the supported feature ID(s) for a VFL Client based on configuration.

3.	Each NWDAF VFL Client invokes Nnwdaf_VFLTraining_Response or and each AF VFL Client invokes Naf_VFLTrainingRequest_Response, possibly via NEF when the AF is untrusted, to indicate to the VFL Server whether it accepts the ML Model training requirements, the VFL Client can also indicate that it cannot join the FL process.

When NEF is involved in the procedure:

-	if the NEF receives the SUPI as sample ID(s) from NWDAF, the NEF shall map the SUPI(s) to GPSI(s) before sending to the AF.

-	if the NEF receives the GPSI as sample ID(s) from NWDAF and/or AF, the NEF may send it to the NWDAF and/or AF directly.

4.	The VFL Server maps the sample ID(s) in different formats (e.g, SUPI, GPSI) from the VFL Client(s) into same format (i.e SUPI) and determines the final list of sample ID(s) that all selected VFL Client(s) support as the samples to be used in the VFL process. The VFL Server also determines the feature ID(s) per VFL Client and VFL Interoperability Information to be used in VFL process and provides them to the selected VFL Client(s) at the start of the training phase, as described in clause 6.2H.2.3.1.

# * * * * Next change * * * *

6.2H.3	Contents of ML Model Training service for Vertical Federated Learning

The consumers of the ML Model training services may provide the input parameters in Nnwdaf_VFLTraining service or Naf_VFLTraining service or in Nnef_VFLTraining service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	[OPTIONAL] VFL Interoperability Information that indicates the intermediate results that the VFL Server supports (e.g. activation function, gradients, type of loss), the content of the VFL Interoperability Information is not standardized in this release.

-	[OPTIONAL] The suggested list of sample IDs expected to be used in the VFL process.

-	[OPTIONAL] VFL Interoperability indicator.

-	[OPTIONAL] The feature ID(s) expected to be used in the VFL process for each VFL client.

The VFL Client provides to the consumer of the ML Model training service operations the output information in as listed below:

-	[OPTIONAL] VFL Interoperability information supported by each VFL Clients.

-	[OPTIONAL] The list of sample IDs within the sample IDs suggested by the VFL Server.

-	[OPTIONAL] The list of sample IDs supported by the VFL Client.

-	[OPTIONAL] The feature ID(s) that the VFL Client supports. A feature ID indicates that what features the VFL Client can use for an Analytics ID, the values of the feature ID are not standardized.

# * * * * End of changes * * * *