3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500683

# Elbonia, 20th Jan – 24th Jan, 2025		 (revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0293 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: UE mobility for PDU session supporting local offloading management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. When a local offloading PDU session experience AMF change due to UE mobility, e.g. Handover, the new AMF needs “Local Offloading Management Allowed indication” from the old AMF to select I-SMF supporting local offloading. Otherwise, the AMF can only select the I-SMF after registration, which is too late. 2. If target I-SMF not support local offloading, new EASDF will not be selected, but the DNS server address configured on UE is still the old EASDF IP address which context has been released by old I-SMF and not available any more. In this case, an availabled default DNS server should be configured to the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify the “Local Offloading Management Allowed indication” should be transferred between old and new AMFs. 2. Clarify that a default DNS server should be configured to the UE if target I-SMF does not support local offloading. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE mobility in local offloading management scenario cannot work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5271 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.10.2.4	UE mobility for PDU session supporting Local Offloading Management

For a PDU session supporting Local Offloading Management, during mobility events, e.g. Hand-Over or AMF change, the mobility handling is executed same as the respective mobility handling defined in clause 4.23 of TS 23.502 [3] with the following difference:

- When AMF change, the Local Offloading Management Allowed indication is transferred to the (new) AMF in the UE context.

- Per the received Local Offloading Management Allowed indication or the retrieval of the Local Offloading Management Allowed indication as part of the SMF selection subscription data from UDM, the new AMF determines the new I-SMF as follows:

-	If the service area of SMF includes the new UE location and the UE location is out of local offloading management service area, the AMF removes the old I-SMF (if available).

-	If the service area of SMF include the new UE location and the UE location is within local offloading management service area, the AMF selects a new I-SMF unless the existing I-SMF can serve the new UE location. If there is an old I-SMF which cannot serve the new UE location, the AMF also removes the old I-SMF.

-	To support the local offloading management, the selected I-SMF need to support Local Offloading Management, serves the new UE location; and support the DNN and S-NSSAI associated with the PDU session.

NOTE 1:	If no I-SMF supporting Local Offloading Management can be selected, and the UE is out of SMF service area, an I-SMF not support Local Offloading Management is selected to guarantee the continutity of user plane, and the local offloading management cannot be supported.

NOTE 2:	When the UE moves from an AMF which is not upgraded to act based on the Local Offloading Management support indication to an AMF which is upgraded to act based on the Local Offloading Management support indication, the above procedure also applies.

-	For I-SMF insertion and relocation case, the target I-SMF and SMF interacts as follows:

-	The target I-SMF sends new selected EASDF IP address and Local Offloading Management allowed indication to the SMF.

-	The SMF sends the new selected EASDF IP address to the UE via PCO.

-	If Local Offloading Management is not supported by the target I-SMF, the SMF updates the address of DNS 	server in the UE.

-	For I-SMF removal case, the SMF performs as follows:

-	A new EASDF is selected by the SMF and the SMF sends the new selected EASDF IP address to the UE via PCO.

# * * * * End of changes * * * *