3GPP TSG-SA WG2#166AHE	S2-2500711

E-meeting, 20-24 January 2025	(was)

Source:	Samsung

Title:	Topic #7, Conclusions on AIoT KI#2 Device ID

Document for:	Discussion/Approval

Agenda Item:	19.14.1

Work Item / Release:	Rel-19 / FS_AmbientIoT

Abstract of the contribution: This paper proposes to update conclusions on Device ID.

1 Discussion

During SA2#166 meeting, interim conclusions on Device ID. In the conclusions, there is an editor’s note which needs to be resolved for Device ID.

Editor's note:	The length of Identifier is fixed or dynamical is FFS.

- The length of Identifier needs to be fixed for the following purposes:

- To make the device simple

- The length of Identifier needs to be dynamical for the following purposes:

- For radio delivery efficiency

- For later use not defined yet

- For flexible 3rd party information description

For the length of Identifier decision, stage 3 considerations are needed. However, considering both bullet A and bullet B above, we need to set the length of Identifier to be fixed with dynamical subfields (e.g. 3rd party information) lengths. With this we can make the device simple, use the ID for later use not defined yet, and use the ID for flexible 3rd party information description.

2 Proposal

It is proposed to capture the following changes to TR 23.700-13.

START OF CHANGES

8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier and Identification Management.

The following principles are agreed for Identifier and Identification Management:

-	An Ambient IoT Device is configured with a permanent Ambient IoT Device Identifier which can be assigned by an operator or by a third party. The Identifier is used to identify Ambient IoT Device and locate the corresponding authentication server.

NOTE 1:	How to configure Ambient IoT Device with the permanent Ambient IoT Device Identifier is out of 3GPP scope.

The permanent Ambient IoT Device Identifier includes the following information:

1)	Part1information:

-	The ID type, including

-	information indicating whether the network identifier is included or not.

-	information indicating whether information used to identify a 3rd party is included or not.

-	the Part2 type indicating EPC or the other format.

-	A network identifier (i.e. MCC+MNC and/or NID), when the ID includes the network identifier.

-	Information used to identify a 3rd party when the ID includes the information used to identify a 3rd party.

2)	Part2 information:

-	The information (e.g. EPC or others) used to distinguish different Ambient IoT Devices within the scope identified by the Part1 information.

Editor's note:	Part2 information for the operator assigned and 3rd party assigned ID needs further study.

NOTE 3:	The coding for the above information is left to stage 3.

Figure 8.2.1-1: The example of Operator allocated ID

For operator allocated Part 1 information, the network identifier is mandatory and can be used to index the authentication server or not. If it is not used to index the authentication server, the ID may further include the information used to identify a 3rd party. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the authentication server.

Figure 8.2.1-2: The example third party allocated ID

For third party allocated Part 1 information, the network identifier is not needed. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the authentication server. The length of Identifier to be fixed with dynamical length for third party related context including the information used to locate the authentication server.

With the above information, the AIoT device ID is globally unique.

Editor's note:	Whether the temporary ID in the AIoT NAS layer is required for the privacy protection is FFS and is pending SA WG3 decision.

END OF CHANGES