3GPP TSG-SA WG2#166AHE	S2-2500712

E-meeting, 20-24 January 2025	(was)

Source:	Samsung

Title:	Topic#2, Conclusions on AIoT KI#1 for Topology 1

Document for:	Discussion/Approval

Agenda Item:	19.14.1

Work Item / Release:	Rel-19 / FS_AmbientIoT

Abstract of the contribution: This paper proposes to update conclusions on AIoT Architecture to support Topology 1.

1 Discussion

During SA2#166 meeting, interim conclusions on AIoT Architecture to Support Topology 1 are agreed. In the conclusions, an editor’s note needs to be resolved for selecting BS reader / ANOT RAN node.

Editor’s note:	Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 needs coordination between SA WG2 and RAN WG(s).

As for indirect path option for Topology 1, for Inventory Request, AIOTF can select AMF considering the Area condition. As an AMF is connected to a BS Reader(s) (an AIoT RAN) regardless of whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol, AIOTF can select the BS Reader(s) connected to the AMF by selecting an AMF. AIOTF can select AIoT-RANs considering the Area condition, in case each AIoT RAN is configured its Area information. After Inventory Response, the contexts on the device ID, Area, Task, and AF are used for AIOTF to select the AIoT RAN for the next Command Request.

As for direct path option for Topology 1, for Inventory Request, AIOTF can select AIoT-RANs considering the Area condition, in case each AIoT RAN is configured its Area information. After Inventory Response, the contexts on the device ID, Area, Task, and AF are used for AIOTF to select the AIoT RAN for the next Command Request.

2 Proposal

It is proposed to capture the following changes to TR 23.700-13.

START OF CHANGES

8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the BS readers or AIOT RAN nodes. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE X:	As for indirect path option for Topology 1, for Inventory Request, AIOTF can select AMF considering the Area condition. As an AMF is connected to a BS Reader(s) regardless of whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol, AIOTF can select BS Reader(s) connected to the AMF by selecting an AMF. As for direct or indirect path option for Topology 1, for Inventory Request, AIOTF can select AIoT-RANs considering the Area condition, in case each AIoT RAN is configured its Area information.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

5.	The network may manage the AIOT device related information (e.g. device context information), includes the AIOT device permanent ID, and optionally the last known reader information.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:	Where to store the AIOT device related information is FFS.

END OF CHANGES