3GPP TSG-WG SA2 Meeting #166-Ad-Hoc-e	S2-2500715

Electronic meeting, January 20-24, 2025

Source:	NTT DOCOMO, Nokia

Title:	Discussion on whether IMS AS or P-CSCF initiates SIP re-INVITE

Document for:	Discussion

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3-ARC / Rel-19

Abstract: Discussion on whether IMS AS or P-CSCF initiates SIP re-INVITE. Impacts on handling of the SIP sequence number are discussed. It is proposed to take the IMS AS based approach as the way forward.

1. Introduction

This paper discusses whether IMS AS or P-CSCF should initiate SIP re-INVITE to complete IMS AGW relocation at change of satellite. This paper considers impacts on handling of the SIP sequence number and comes to a proposal.

2. Discussion

The following is excerpts related to the SIP sequence number from IETF RFC 3261:

-	"A dialog contains certain pieces of state needed for further message transmissions within the dialog. This state consists of the dialog ID, a local sequence number (used to order requests from the UA to its peer), a remote sequence number (used to order requests from its peer to the UA), [..]" in clause 12 Dialogs.

-	"Requests within a dialog MUST contain strictly monotonically increasing and contiguous CSeq sequence numbers (increasing-by-one) in each direction" in clause 12.2.1.1 Generating the Request.

-	"The CSeq header field [..] consists of a sequence number and a method" in clause 8.1.1.5 CSeq.

-	"If [..] the sequence number of the request is lower than the remote sequence number, the request is out of order and MUST be rejected with a 500 (Server Internal Error) response" in clause 12.2.2 UAS Behavior.

The P-CSCF based approach has issue with handling of the sequence number as is explained in the below.

It is assumed that UE, P-CSCF, and IMS AS understand the local sequence number of UE is A and the local sequence number of IMS AS is B after the call setup. Then, the mobility procedure starts. P-CSCF would use CSeq=A+1 to send re-INVITE to the remote network. Later, IMS AS responses to P-CSCF using CSeq=A+1. P-CSCF then uses CSeq=B+1 to send re-INVITE to UE. Later, UE responses to P-CSCF using CSeq=B+1. The mobility procedure ends. For some reason (e.g., when the satellite changes in the remote network), IMS AS sends re-INVTE towards P-CSCF to UE. IMS AS uses CSeq=B+1, since the latest local sequence number from IMS AS’s perspective is CSeq=B. P-CSCF and/or UE cannot treat this re-INVITE. The same issue occurs also when UE sends re-INVITE.

Solution for this issue can be complicated as is explained in the below.

A candidate solution is to have P-CSCF act as a SIP B2B UA and to let the dialog be established already between UE and P-CSCF and between P-CSCF and IMS AS separately during the call setup.

Another candidate solution (in stage 3) is to let P-CSCF manage CSeq within a dialog, i.e., P-CSCF changes CSeq to be B+2 and forwards re-INVITE to UE. When P-CSCF receives 200 OK with CSeq=B+2 from UE, P-CSCF changes CSeq to be B+1 and forwards 200 OK with CSeq=B+1 to IMS AS. In this candidate solution, P-CSCF needs to keep four sequence numbers and also to check possible mis-ordering of the received messages.

NOTE 1:	This type of issue does not happen for the existing case of P-CSCF acting as UA, which is the P-CSCF initiated release after loss of radio coverage shown in clause Y.2.4.1 of TS 23.228. In this case, the dialogue is simply released.

NOTE 2:	The IMS AS based approach requires attention in this aspect as well, while the situation is the same as that for the PLMN change detected by EPC/5GC shown in clause W 4.3 of TS 23.228 and can be treated in the same way. Even if stage 3 requires further solution, candidate solutions can be light-weight: (i)since CSeq contains a method name, P-CSCF and IMS AS may use a different set of sequence numbers or ignore sequence numbers for the MESSAGE method. (ii) the message that P-CSCF triggers IMS AS may be put outside of the dialog or not a SIP message.

3. Proposal

Considering the P-CSCF based approach requires a complicated solution for handling of the SIP sequence number,

Proposal: It is proposed to take the IMS AS based approach as the way forward.