3GPP TSG-SA WG2#166AHE	S2-2500735

E-meeting, 20-24 January 2025	(was)

Source:	Samsung

Title:	pCR to TS 23.xyz: Ambient IoT Services

Document for:	Discussion/Approval

Agenda Item:	19.14.2

Work Item / Release:	Rel-19 / AmbientIoT

Abstract of the contribution: This paper proposes text for Assistance Information used by AIoT RAN.

1 Discussion

During SA2#166 meeting, interim conclusions on AIoT Architecture to Support Topology 1 are agreed.

Considering possible agreements in SA2#166AHE, we may capture TS text for Assistance Information used by AIOT RAN.

2 Proposal

It is proposed to capture the following changes to new TS.

START OF CHANGES (All New)

5.4	Assistance Information used by AIOT RAN

The following Assistance Information used by AIOT RAN is supported by AIOT RAN:

-	Approximate number of AIoT devices.

-	Approximate D2R message size.

NOTE X:	An AMF is connected to a AIoT RAN (BS Reader) regardless of whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol. So the AIoT RAN identifies the connected AMF to which it needs to forward UL messages.

END OF CHANGES