# SA WG2 Meeting #SA2166AHE	S2-2500766

# 20 - 24 January, 2025, E-meeting

Source:	Samsung

Title:	PDU Session Modification for Port Management Information

Agenda item:	7.7 (Vertical_LAN)

Document for:	Decision

1. Introduction

TS 23.502 includes PDU Session Modification procedure which includes changes for Port Management Information. However, the details do not align with what are described in Annex F.2.

- PDU Session Modification Procedure of step 3a in the procedure in Figure 4.3.3.2-1 includes descriptions for Port Management Information update.

- PDU Session Modification Procedure of step 3b in Figure 4.3.3.2-1 does not include any Port Management Information update descriptions.

- Annex F.2 mentions PDU Session Modification Procedure of step 3b in Figure 4.3.3.2-1 is used for Port Management Information update. (see the following)

Figure F.2-1: 5GS Bridge information configuration … 7. The PCF determines the SMF based on the MAC address received in the AF request, the PCF maps the TSN QoS information provided by the AF to PCC rules as described in clause 5.28.4 of TS 23.501 [2]. The PCF includes the TSC Assistance Container received from the AF with the PCC rules and forwards it to the SMF. The PCF transparently transports the received Port Management information Container and related port number to SMF via Npcf_SMPolicyControl_UpdateNotify message. 8a. If the SMF decides the port is on DS-TT based on the received port number, the SMF transports the received Port Management information Container to the UE/DS-TT in PDU Session Modification Request message. …

This misalignment gives confusions to the readers.

2. Discussion

As Port Management Information can be updated through PCF-initiated PDU session modification as well as from UE-initiated PDU session modification, we need to clarify the normative text in clause 4.3.3.2.

- PDU Session Modification Procedure of step 3b in Figure 4.3.3.2-1 needs to include Port Management Information update descriptions, which aligns with F.2.

We need to update the procedures in clause 4.3.3.2 in Rel-16 through Rel-19.

If we do not, we will have no texts for Port Management Information update in the procedure in step 3b, which causes confusions to the readers, as there exist texts for Port Management Information update in the procedure in step 3a.

3. Conclusions

We need to add descriptions for Port Management Information update in the procedure in step 3b in Figure 4.3.3.2-1.

4. Proposal

It is proposed to agree the changes to TS 23.502 Rel-16 through Rel-19, which are presented in S2-2500770, S2-2500773, S2-2500774 and S2-2500777.