SA WG2 Meeting #S2-166-Ad-Hoc-e	S2-2500800

20 - 24 January, 2025, Electronic meeting	(revision of S2-250xxxx)

Source:	ZTE

Title:	Discussion on the QME capability issue from CT3

Document for:	Discussion

Agenda Item:	19.25

Work Item / Release:	TEI19_QME / Rel-19

Abstract of the contribution: This paper discusses the CT3 LS (S2-2500037) on Clarification of the scope of QME and give the proposal.

# 1. Discussion

In this meeting, the CT3 send the LS (S2-2500037) on Clarification of the scope of QME.

CT3 has defined the QoS Monitoring Capability Report for packet delay. CT3 would like to ask:

Question1: May all the QoS Monitoring mechanism (e.g. RT delay over two QoS flow, Packet Delay Variation, congestion, data rate) to be supported for the QoS monitoring capability report or not?

In the 23.501 clause 5.45.1,

The AF may request measurements for one or more of the following QoS monitoring parameters, which may trigger QoS monitoring for service data flow(s):

-	UL packet delay, DL packet delay, round trip packet delay for a service data flow, see clause 5.45.2.

-	Congestion, see clause 5.45.3.

-	Data Rate, see clause 5.45.4.

-	Packet Delay Variation, see clause 5.37.7.

-	Round trip packet delay considering UL on a service data flow and DL of another service data flow, see clause 5.37.4.

And the PCF behaviour is:

The QoS monitoring parameter(s) that can be measured by means of QoS monitoring are listed below. The QoS monitoring policy in PCC rule (described in clause 6.3.1 of TS 23.503 [45]) may include the following:

-	UL packet delay, DL packet delay, round trip packet delay, see clause 5.45.2.

-	Congestion, see clause 5.45.3.

-	Data Rate, see clause 5.45.4.

The PCF may calculate Packet Delay Variation (clause 5.37.7) and the round trip packet delay when UL and DL are on different QoS flows (clause 5.37.4) based on packet delay monitoring results of QoS flows.

And the SMF request the NG-RAN for Qos monitoring:

The SMF may also configure NG RAN to measure the QoS monitoring parameters by sending QoS monitoring request based on the authorized QoS Monitoring policy received from the PCF and/or local configuration. The QoS monitoring request to the NG RAN for different parameters is as defined in clause 5.45.2 and 5.45.3.

Clause 5.45.2 is Packet delay monitoring (The details of the QoS Monitoring for packet delay are described in clause 5.33.3, which is introduced in URLLC in Rel-16).

Clause 5.45.3 is Congestion information monitoring (which is introduced in XRM in Rel-18).

Since the Packet delay monitoring and Congestion information monitoring are specified in the different release, it is recommended to separate them.

Observation 1: There are two Qos monitoring capability in the RAN. i.e. Packet delay monitoring and Congestion information monitoring

In the CT4 agreed CR (29.502 CR#0796) in the CT4 LS (S2-2409591), the new IE “QosMonitoringPdSupported” is only for Packet delay monitoring.

- This IE shall be present if the QME feature is supported by AMF and SMF and if the information is available. When present, this IE shall indicate whether the NG-RAN support the QoS monitoring for packet delay feature.

Observation 2: In the CT4 spec 29.502, it only specified the Qos monitoring capability for Packet delay monitoring

For the CT3 question: May all the QoS Monitoring mechanism to be supported for the QoS monitoring capability report or not?

There are two way forward:

Way forward 1:

In this release, Qos monitoring capability in the RAN only refer to Packet delay monitoring as CT4 has specified.

In the 23.501, it needs to clarify that Qos monitoring capability in the RAN is the Packet delay monitoring. There is no additional work for CT4.

Way forward 2:

In this release, Qos monitoring capability in the RAN refers to Packet delay monitoring and Congestion information monitoring. In this way forward, CT4 need to specify a new IE for “Congestion information monitoring”.

In the 23.501, it needs to clarify that Qos monitoring capability refer to two separate capabilities

Comparing between way forward 1 and 2, it prefers way forward 2, also can live with way forward 1.

Proposal: Qos monitoring capability in the RAN refers to Packet delay monitoring and Congestion information monitoring.

# 2. Proposal

It is proposed to adopt the way forward 2 in the CR S2-2500802