3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500829

# E-meeting, Jan 20th – 24th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0176 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UAV service operation description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the last two meetings, a new NEF service “Nnef_RetrieveInfoUAVFlight” is introduced. In particular, “Get” service operation is agreed to facilitate the scenario of “pre-flight planning”. For example, xxx. However, another NEF service “Nnef_UAVFlightAssistance” was also defined for the purpose of “pre-flight planning”. As indicated in clasue 4.4.1.1.3.3, “Input, Required”: UAV's identifier (e.g. GPSI), indication about the purpose of the request (i.e. USS changeover, pre-flight planning, in-flight monitoring). It is observed the “Nnef_UAVFlightAssistance” should not be used for pre-flight planning, because the “Nnef_RetrieveInfoUAVFlight” is introduced later, specific for this purpose. In general, since there are two NEF services defined in rel-19 for exposure to USS, we need to clarify the “boundary” for the two services. It is proposed to clarify the “Nnef_RetrieveInfoUAVFlight” is for pre-flight planning, while “Nnef_UAVFlightAssistance” is for in-flight monitoring. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: update clause 4.4.1.1.3.3, Nnef_UAVFlightAssistance_Create service operation: - Remove the “pre-fligfht panning” from “Input, Required”. - Add parameters “planned flight path information for each of the segment of the paths served by different USS”, “planned flight path information for each of the segment of the paths served by different USS”, “monitoring mode”, “report requirements and assistance information” in the “Input, Optional” Change 2: update clause 4.4.1.1.4.2, Nnef_RetrieveInfoUAVFlight_Get service operation: - Add parameter “information about the starting and ending points for the flight”, in the “Input Optional” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The descriptions two NEF services “Nnef_RetrieveInfoUAVFlight” and “Nnef_UAVFlightAssistance” are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.3, 4.4.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st changes * * * *

4.4.1.1.3.3	Nnef_UAVFlightAssistance_Create service operation

Service operation name: Nnef_UAVFlightAssistance_Create

Description: Providing a NEF/UAS NF information about the planned flight path(s) and a time schedule for the UAV from a starting point to a next point in the flight plath (even when the next point is located in geographical area served by different USS) or providing a NEF/UAS NF information about the in-flight monitoring of a target UAV. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the UAS NF/NEF determines that UAV will leave a geographical area served by the USS, (2) the UAV enters a TA/Cell with a border-crossing point to another USS, and (3) the UAV deviates from the assigned flight plan.

Input, Required: UAV's identifier (e.g. GPSI), indication about the purpose of the request (i.e., USS changeover, in-flight monitoring).

Input, Optional: Flight path information, which may include planned flight path information for each of the segment of the paths served by different USS, e.g. a list of TA, UAV's speed, candidate border-crossing point(s), flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells), monitoring mode, report requirements and assistance information as described in clause 5.12.

Output, Required: Flight plan configuration ID (associated with the changeover).

Output, Optional: None.

4.4.1.1.3.4	Nnef_UAVFlightAssistance_Update service operation

Service operation name: Nnef_UAVFlightAssistance_Update

Description: Used by the service consumer to update the core network about changes in the previously provided UAV flight path(s) and/or time schedule.

Input, Required: Flight plan configuration ID.

Input, Optional: Flight path information, which may include e.g. a list of TA, UAV's speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells), Notification Target Address.

Output, Required: None.

Output, Optional: None.

4.4.1.1.3.5	Nnef_UAVFlightAssistance_Notify service operation

Service operation name: Nnef_UAVFlightAssistance_Notify

Description: Notifying a USS about UAV leaving its serviced area and which border crossing point will be used for this purpose, informing a target USS about the UAV entering its serviced area, and confirming the completion of the changeover, notifying a USS about the monitoring results for the UAV.

Input, Required: Flight plan configuration ID, status of the UAV according to the assigned flight path (leaving, entering, deviating).

Input, Optional: Information about border-crossing point that will be used for USS changeover.

Output, Required: Monitoring results as described in clause 5.12, Indication whether the USS changeover is completed successfully when used by the target USS when responding to the notification.

Output, Optional: None.

# * * * * Second changes * * * *

4.4.1.1.4.2	Nnef_RetrieveInfoUAVFlight_Get service operation

Service operation name: Nnef_ RetrieveInfoUAVFlight_Get

Description: Requesting from a NEF/UAS NF to retrieve data from 5GC NFs that can be used by USS for UAV’s flight purposes.

Input, Required: UAV's identifier (e.g. GPSI), indication about the purpose of the retrieved information (i.e., USS changeover, pre-fligh planning).

Input, Optional: list of suitable target USSes, a list of candidate border-crossing point(s), acceptable deviations for flight plan/route, information about the starting and ending points for the flight, information about candidate flight path(s), requirements on the flight path, and accuracy level of predictions relevant to the flight planning as described in clause 5.12.

Output, Required: Report containing a Report ID (e.g. Transaction Reference ID assigned by the NEF) and the retrieved information.

Output, Optional: None.

# * * * * End of changes * * * *