3GPP TSG-WG SA2 Meeting #166-AdHoc-e	S2-2500835

# E-meeting, January 20 – 24, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1365 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on model performance monitoring for LMF-based AI/ML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - During data collection procedure for model performance monitoring of LMF-side AI/ML positioning, location measurement data of the PRU(s)/UE(s) may also be provided by the LMF to the NWDAF, to assist the NWDAF in model performance monitoring and model re-training. - The ground truth data for LMF-side AI/ML positioning includes location information of PRU(s)/UE(s). - Editorial modifications are needed, e.g. correcting the reference to TS 23.273, changing “AI/ML model” to “ML model”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that location measurement data from the PRU(s)/UE(s) or from the NG-RAN node(s) serving the PRU(s)/UE(s) may also be provided to the NWDAF during ML model performance monitoring for LMF-based AI/ML Positioning procedure. - Clarify that the ground truth data includes location information of PRU(s)/UE(s). - Editorial modifications, e.g. correcting the reference to TS 23.273, changing “AI/ML model” to “ML model”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Location measurement data cannot be provided to the NWDAF to assist in model performance monitoring and model retraining. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

6.2E.4	Procedure for MTLF-based ML model performance monitoring for LMF-based AI/ML Positioning

The NWDAF containing MTLF may support ML model performance monitoring for LMF-based AI/ML Positioning. When the NWDAF containing MTLF determines to monitor the ML model performance for LMF-based AI/ML Positioning, it collects the input data of PRU(s)/UE(s) from the LMF by using the procedure of input data collection by NWDAF as specified in clause 6.22.4 of TS 23.273 [39]. The input data for ML model performance monitoring includes the ground truth data (i.e. location information of PRU(s)/UE(s)), the location information of PRU(s)/UE(s) calculated by the LMF using the ML model and the location measurement data, and optionally location measurement data from the PRU(s)/UE(s) or from the NG-RAN node(s) serving the PRU(s)/UE(s). Based on the ground truth data and the calculated location information, NWDAF containing MTLF evaluates the ML model performance to generate the performance monitoring result. The result may trigger NWDAF containing MTLF to retrain the ML model, or be sent to LMF to trigger LMF to change the positioning method, e.g. from LMF-based AI/ML Positioning to legacy positioning.

* * * End of Changes * * * *