3GPP TSG-WG SA2 Meeting #166-AdHoc-e	S2-2500836

# E-meeting, January 20 – 24, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1366 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on QoS and Policy Assistance Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There are misalignment on QoS parameters in the input parameters and the output analytics, i.e. ARP and Maximum Packet Loss Rate are missing from the input parameter, Priority Level as one of the QoS characteristics is missing from the output analytics. - The following ENs are to be resolved: Editor's note: Whether Application ID or SDF template is mandatory is FFS. Editor's note: The other filter information is FFS, e.g. PDU Session Type, SSC Modem PLMN ID, etc. SDF template includes service data flow filters or application ID (See 23.503 Table 6.3.1). It is proposed to keep Application ID or service data flow filters as optional, as the analytics can be performed e.g. for all the applications in a network slice (similar to Observed Service Experience analytics). The other filter information (e.g. PDU Session Type, SSC Mode, PLMN ID) is not quite related to QoS parameters and therefore not needed. - It is not appropriate to specify “QoS binding events” as input data for analytics. And it is unclear which Event ID the NWDAF uses to collect input data in Table 6.23.2-1 from the SMF. It is proposed to reuse existing Event ID “QFI allocation” Instead of defining a new Event ID “QoS binding events” for the SMF. - QoS profile as defined in clause 5.7.1.2 of TS 23.501 contains the same QoS parameters as in the QoS parameter set. So there is no need to collect Used QoS profile from the SMF. - There are redundant information in the output analytics, e.g. Applicable duration of QoS and Policy Assistance information, Traffic descriptor, which should be removed. - Editorial modifications are needed e.g. to remove redundant descriptions, change “may” to “can” in the NOTE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the misalignment on QoS parameters in the input parameters and the output analytics, i.e. adding ARP and Maximum Packet Loss Rate to the input parameter, adding Priority Level to the output analytics. - Remove the following ENs: Editor's note: Whether Application ID or SDF template is mandatory is FFS. Editor's note: The other filter information is FFS, e.g. PDU Session Type, SSC Modem PLMN ID, etc. - Clarify that the NWDAF uses event ID “QFI allocation” to collect input data in Table 6.23.2-1 from the SMF. And modify the description of input data, including removing “Used QoS profile”. - Remove the redundant information in the output analytics, i.e. Applicable duration of QoS and Policy Assistance information, Traffic descriptor. - Editorial modifications, e.g. removing redundant descriptions, changing “may” to “can” in the NOTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned or incorrect input parameters, input data or output analytics for QoS and Policy Assistance Analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.23.1, 6.23.2, 6.23.3, 6.23.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

6.23.1	General

Clause 6.23 describes how NWDAF can provide QoS and policy assistance analytics, based on the consumer request. The consumer can either subscribe to analytics notifications (i.e. a Subscribe-Notify model) or request a single notification (i.e. using a Request-Response model).

The service consumer may be a PCF.

The QoS and policy assistance analytics provides predictions of the QoE associated to a set of QoS parameters. When the service consumer needs assistance information to determine QoS or policy, it may send an Analytics request or subscription to NWDAF with one or multiple sets of QoS parameters and optional a requested QoE. NWDAF provides the candidate set(s) of QoS parameters and the corresponding derived predicted QoE that is expected to be achieved by applying the candidate set(s) of QoS parameters, wherein the candidate set(s) of QoS parameters are within the sets of QoS parameters provided by the consumer.

The QoS and policy assistance analytics may be provided for individual UE or group of UEs (e.g. the QoE is for the application ID(s) associated to one or more QoS flow(s) of the UE(s)), or for an application (e.g. the QoE is for the service flow of the application ID).

The consumer of these analytics indicates the following information in the request or subscription:

-	Analytics ID = " QoS and Policy Assistance".

-	Target of Analytics Reporting as defined in clause 6.1.3.

-	One or more QoS parameter set(s) and optionally each associated with a QoS parameter set ID. Each QoS parameter set includes one or more of the following individual parameters and non-empty value list(s) for individual parameter(s):

-	5QI (standardized or pre-configured) and/or QoS characteristics (as defined in clause 5.7.3 of TS 23.501 [2]): Resource Type, Priority Level, PDB, PER, Averaging Window, Maximum Data Burst Volume;

-	ARP.

-	For GBR flows: GFBR, MFBR and Maximum Packet Loss Rate (if applicable).

NOTE 1:	In the case of standardised 5QIs, the specified QoS parameters and QoS Characteristics (e.g. Table 5.7.4-1 of TS 23.501 [2]) may be pre-configured in NWDAF by operators, subject to operator policy. Therefore, the NWDAF is able to associate the QoS parameters and QoS Characteristics to the given 5QI in PCF request to derive the predicted QoE.

NOTE 2:	QoS Characteristics can be included in the QoS parameter set(s) only in the case of dynamically assigned non-standardized 5QIs.

-	Optionally, a requested QoE to indicate NWDAF to only provide the QoS parameter set(s) and their corresponding value(s) for which the predicted QoE is equal or higher than the requested QoE.

-	Analytics Filter Information optionally includes:

-	DNN;

-	Application ID or service data flow filters (i.e. Packet Filter set);

-	Area of Interest;

NOTE 3:	A service consumer can use the Area of Interest in order to reduce the amount of signalling that the analytics subscription or request generates.

-	S-NSSAI and NSI ID;

-	List of analytics subsets that are requested, (see clause 6.23.3), e.g. parameter(s) in QoS parameter set(s).

-	An Analytics target period indicates the time period over which the analytics are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Preferred order of results for the list of candidate QoS parameter sets and the predicted QoE associated to the candidate QoS parameter sets:

-	ordering criterion: "QoE" (i.e. QoE that is associated to candidate QoS parameter set) or "usage duration" or "number of usages" of a QoS flow,

-	order: ascending or descending.

-	Reporting Thresholds, which apply only for subscriptions and indicate conditions on the levels to be reached for the respective analytics subsets (see clause 6.23.3).

-	Maximum number of objects and maximum number of SUPIs.

-	Preferred granularity of location information: TA level or cell level or "longitude and latitude level".

6.23.2	Input Data

The NWDAF supporting QoS and policy assistance analytics shall be able to collect information from AF, OAM and 5GC NFs.

The following specified input data might be reused by QoS and policy assistance analytics:

-	The input data defined for Observed Service Experience related network data analytics as defined in clause 6.4.2, e.g. the information of the application, Service Experience and QoE metrics.

In addition, more information collected by the NWDAF from relevant 5GC NFs (i.e. UPF, SMF, AMF, PCF) is defined in Table 6.23.2-1.

NOTE:	The NWDAF may also collect the Observed Service Experience Analytics from other NWDAFs, based on NWDAF implementation.

Table 6.23.2-1: Input data from 5GC NF related to QoS and policy assistance information

| Information | Source | Description |
| --- | --- | --- |
| QoS binding information (1..max) | SMF | Information of specific QoS parameters applied to QoS flows. |
| > Used QoS parameter set(s) | SMF | The QoS parameter set(s) that have been already applied by SMF. As detailed in clause 6.23.1, the QoS parameter set includes QoS parameters and QoS characteristics (as defined in TS 23.501 [2]). |
| > Traffic descriptor | SMF | One of Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set |
| > S-NSSAI | SMF | Slice used to transport the QoS flow |
| > DNN | SMF | DNN associated with the QoS flows. |
| > Application ID | SMF | Identifies the application service provider and application for the QoS Flow(s) |
| > DNAI | SMF | Identifies the access to the DN to which the PDU Session connects |
| > PDU Session Type | SMF | Type of PDU Session |
| > SSC Mode | SMF | SSC Mode of the PDU Session(s) |
| UE identifier | SMF, AMF | The identifier of UE, e.g. SUPI, UE IP address, etc. |
| UE Location | AMF | The UE location information, e.g. cell ID or TAI. |
| Time stamp | SMF, AMF | The time stamp associated to the collected data. |

The NWDAF subscribes to the input data in Table 6.23.2-1 from the SMF by invoking Nsmf_EventExposure_Subscribe service operation using the Event ID "QFI allocation" as described in TS 23.502 [3].

6.23.3	Output Analytics

The NWDAF shall be able to provide prediction on QoS and Policy Assistance to consumer NFs, i.e. PCF, as defined in Table 6.23.3-1.

Table 6.23.3-1: QoS and Policy Assistance predictions

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > QoS and Policy Assistance information (1…max) (NOTE 1) | List of QoS and Policy Assistance information. Max. is the number of the candidate QoS parameter set(s), if applicable. |
| >> QoS parameter set identifier | Identifies the QoS set for which the entry applies. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> Application ID(s) | Identifies the application(s) that is associated to the QoE and QoS parameter set. |
| >> Packet Filter Set | IP Packet Filter Set or Ethernet Packet Filter Set for the QoS flow(s). |
| >> Predicted QoE (NOTE 6) | The predicted QoE or the service experience (e.g. QoE, MOS) of the corresponding QoS parameter set, e.g. average, variance, maximum, minimum of QoE. |
| >> QoS parameter values (NOTE 4) | The parameters and corresponding values for the QoS parameter set associated to the predicted QoE, |
| >>> 5QI | The reference to 5G QoS characteristics and QoS parameters. |
| >>> ARP | The QoS parameter ARP contains information about the priority level, the pre-emption capability and the pre-emption vulnerability, as defined in TS 23.501 [2]. |
| >>> Resource type | The resource type of the corresponding QoS flow, e.g. GBR QoS flow, non-GBR QoS flow, delay-critical QoS flow. |
| >>> Packet Delay Budget | Packet Delay Budget (PDB) indicates the upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF, as defined in TS 23.501 [2]. |
| >>> Priority Level | Priority Level associated with 5G QoS characteristics indicates a priority in scheduling resources among QoS Flows, as defined in TS 23.501 [2]. |
| >>> Packet Error Rate | Packet Error Rate (PER) defines an upper bound for a rate of non-congestion related packet losses, as defined in TS 23.501 [2]. |
| >>> Maximum Data Burst Volume (NOTE 3) | The Maximum Data Burst Volume (MDBV) applies to GBR QoS Flow with Delay-critical resource type. The MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB, as defined in TS 23.501 [2]. |
| >>> Averaging Window (NOTE 2) | The Averaging window is applied when the resource type is GBR QoS. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. in the (R)AN, UPF, UE), as defined in TS 23.501 [2]. |
| >>> Flow Bit Rates (NOTE 2) | The flow bit rates only applies to GBR QoS Flow. |
| >>>> GFBR | Guaranteed Flow Bit Rate (GFBR) for UL and/or DL. |
| >>>> MFBR | Maximum Flow Bit Rate (MFBR) for UL and/or DL. |
| >>> Maximum Packet Loss Rate (NOTE 2) | The Maximum Packet Loss Rate (UL, DL) indicates the maximum rate for lost packets of the QoS Flow that can be tolerated in the uplink and downlink direction. This is provided to the QoS Flow if it is compliant to the GFBR. |
| >> Validity period | The validity time period within the time slot for which the analytics on service experience associated to QoS parameters applies. |
| >> Spatial validity | Area where the analytics on service experience associated to QoS parameters applies. |
| >> Traffic descriptor | One of Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set. |
| >> Usage Duration information (NOTE 6) | Maximum/Minimum/Average usage duration of QoS Flows associated to Candidate QoS parameter set. |
| >> Number of usage (NOTE 6) | The number of times that the QoS Flows associated to Candidate QoS parameter set to be used. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Reporting Thresholds". NOTE 2: The output analytics only applies to GBR QoS Flow. NOTE 3: The output analytics only applies to GBR QoS Flow with Delay-critical resource type. NOTE 4: Only the parameters that have more than one candidate values provided by the consumer shall be included in the NWDAF output and only the values provided by the consumer are allowed. NOTE 5: The usage duration and number of usage of QoS Flow is determined by NWDAF using the SMF Events QoS Flow establishment (i.e. QFI Change) or QoS Flow termination (i.e. QFI deallocation) or Traffic binding of QoS Flow events. For example, the duration equals to the time period between the timestamp of QoS Flow establishment and QoS Flow termination events. NOTE 6: The predicted QoE is reported for the entire QoS set if only one parameter value has been provided for each parameter and otherwise separately for each combination of parameter values. |  |

Editor's note:	Clarification on Usage Duration information and Number of usage is needed.

6.23.4	Procedures

Figure 6.23.4 shows the procedure for the NWDAF to derive and provide the QoS and policy assistance analytics to a consumer 5GC NF (i.e. PCF).

Figure 6.23.4-1: Procedure for QoS and Policy Assistance Analytics

1.	The Consumer NF, i.e. PCF, requests or subscribes to QoS and policy assistance analytics from NWDAF and provides the input information as specified in clause 6.23.1 to NWDAF.

2a.	The NWDAF may subscribe to the input data from 5GC NFs, e.g. AMF, SMF, UPF, etc. as defined in clause 6.23.2 by invoking the corresponding NF event exposure service operation, e.g. using Namf_EventExposure_Subscribe service operation for collecting UE location(s); using Nsmf_EventExposure_Subscribe service operation (Event ID, parameters in QoS parameter set, SUPI(s) or Application ID), etc.

If the required UE location information is finer granularity than TA/cell level, then NWDAF collects the location data from GMLC instead of AMF by invoking the Ngmlc_Location service as defined in TS 23.273 [39] and TS 29.515 [48].

2b.	The NWDAF collects data from the OAM, following the procedure captured in clause 6.2.3.2.

2c.	The NWDAF may subscribe to the service data from AF as defined in Table 6.23-2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = QoS and Policy Assistance, Application ID, Event Filter information, Target of Event Reporting = UE ID(s)) as defined in TS 23.502 [3].

3.	The NWDAF derives the requested analytics on QoS and Policy Assistance based on NWDAF internal logic and operator policies, e.g. the NWDAF derives the analytics directly based on the input data as defined in clause 6.23.2 or the NWDAF may derive the analytics by consuming the output analytics of Observed Service Experience analytics and the input parameters in Table 6.23.2-1.

NOTE:	When requested by the NF consumer (i.e. PCF) the NWDAF can also collect data from OAM as described in clause 6.6.4 and clause 6.9.4 for the purpose of derive NW performance and QoS sustainability information associated to the QoS parameters set(s).

4.	The NWDAF provides the requested QoS and Policy Assistance to the consumer NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service operation used in step 1.

5-7.	If the consumer NF subscribes to QoS and Policy Assistance in step 1, once the NWDAF generates new analytics on QoS and Policy Assistance, it provides notification using Nnwdaf_AnalyticsSubscription_Notify to the Consumer NF.

* * * End of Changes * * * *