S3GPP TSG-WG SA2 Meeting #166 Ad Hoc-e	   S2-2500851

20th – 24th January, 2025, Online                          (revision of S2-250xxxx)

Source:	Spreadtrum, UNISOC

Title:	pCR to TS 23.xyz: AIoT Device entity

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: Introduce text providing AIoT Device functional entity in new TS 23.XYZ based on what has been agreed thus far in conclusions for Rel-19 FS_AmbientIoT.

1. Introduction/Discussion

AIoT Device is introduced in the conclusion of TR 23.700-13 for Rel-19 FS_AmbientIoT.

2. Text Proposal

It is proposed to capture the following changes in TS 23.XYZ.

# * * * * Start of changes * * * *

4.5.1	Ambient IoT Device

The Ambient IoT Device supports the following functionality:

-	Receiving AIoT Device NAS information for Inventory and command operation from AIoTF via an AIoT Reader.

-	Receiving AIoT data from AF via an AIoT Reader.

-	Being configured with an Ambient IoT Device Permanent Identifier.

-	Comparing the received A-IoT Device Identification information with its own AIoT Device Identifier to determine whether to respond to the Inventory and Command.

-	Constructing responses for inventory operation and command operation and sending the constructed response to AIoTF via an AIoT Reader.

-	Sending AIoT data to AF via an AIoT Reader.

Editor's note:	Whether and how security functionality applies to the AIoT Device depends on SA WG3.

# * * * * End of changes * * * *