S3GPP TSG-WG SA2 Meeting #166 Ad Hoc-e	   S2-2500852

20th – 24th January, 2025, Online                          (revision of S2-250xxxx)

Source:	Spreadtrum, UNISOC

Title:	pCR to TS 23.xyz: AIoTF entity

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: Introduce text providing AIoTF functional entity in new TS 23.XYZ based on what has been agreed thus far in conclusions for Rel-19 FS_AmbientIoT.

1. Introduction/Discussion

AIoTF is introduced in the conclusion of TR 23.700-13 for Rel-19 FS_AmbientIoT.

2. Text Proposal

It is proposed to capture the following changes in TS 23.XYZ.

# * * * * Start of changes * * * *

4.5.3	AIOTF

The AIoTF supports the following functionality:

-	Receiving the AIoT service request from the AF.

-	Selecting the RAN reader(s).

-	Determining and Providing the Reader Assistance information to the RAN reader(s) to help control the AIoT service operations.

-	Constructing the requests for the Inventory operation and command operation, and transporting the request to the selected RAN reader(s) in AIoT Device NAS layer.

-	Providing transport for AIoT Data between the AIoT Device and AF.

-	Receiving responses with the results of the AIoT service operation from AIoT Devices via RAN reader(s) in AIoT Device NAS layer.

-	Checking whether the AIoT Device ID from AIoT Device has a subscription.

-	Aggregating the service operation results from RAN reader(s) and providing to AF.

# * * * * End of changes * * * *