3GPP TSG-SA WG2 Meeting #166AH-e	S2-2500859

# Elbonia, 20-24 January, 2025								                            (revision of S2-xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0661 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on LMF re-selection of LCS UP positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current procedure of Modification of User Plane Connection between UE and LMF of clause 6.18.3, the logic of step 1b on how AMF reselects LMF is not clear or correct. AMF *should* perform LMF reselection if a target LMF identification received from source LMF for user plane positioning, or AMF *may* perform LMF reselection if a UE established an LCS UP connection with the source LMF and the UE moves to a new location. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in step 1b of clause 6.18.3 that AMF *should* perform LMF reselection if a target LMF identification received from source LMF for user plane positioning, or AMF *may* perform LMF reselection if a UE established an LCS UP connection with the source LMF and the UE moves to a new location. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not correct implementation of LMF reselection for LCS UP connection |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change  ---

6.18.3	Modification of User Plane Connection between UE and LMF

The figure 6.18.3-1 shows how a secure user plane connection between UE and LMF is modified. The flow describes change of LMF but applies also when source and target LMF is the same. The procedure can also be used to terminate the user plane connection to Source LMF not selecting any Target LMF.

Figure 6.18.3-1: Connection modification between UE and LMFs

1a.	[Conditional] The LMF discovers a need to change LMF or re-establish the user plane connection between UE and LMF or terminate the user plane connection, e.g. after receiving an event report via user plane from UE or after receiving an AMF relocation information from a target AMF (target AMF obtains from source AMF the UE LCS-UP context which indicates UE has maintained LCS-UP connection with the source LMF, the target AMF may inform the source LMF about the AMF change using the Nlmf_Locatoion_UPConfig request). The LMF sends an Nlmf_Location_UPNotify message that includes connection move (termination and establishment) or termination is required and if connection move is requested then message may include target LMF identification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UP Subscribe message.

NOTE 1:	The LMF change procedure is independent from the SSC mode of the PDU Session with dedicated DNN used for positioning. For SSC mode 2/3, PSA UPF connection with the LMF can be relocated with UE movement and LMF can discover the need to change LMF to reduce the user plane path latency.

1b.	[Conditional] AMF should perform LMF reselection if a target LMF identification received from source LMF for user plane positioning, or AMF may perform LMF reselection if a UE established a LCS UP connection with the source LMF moves to a new location (which may be out of serving area of source LMF and in serving area of target LMF) and select the target LMF for the current UE location based on LMF service area, LMF user plane positioning capability information and other information listed in clause 5.1. LMF needs to be capable to establish a user plane session for positioning with the UE. After AMF relocation, the target AMF may trigger the LMF reselection and LCS-UP connection modification procedure, e.g. if the source LMF does not perform LMF reselection.

2.	[Conditional] Steps 3-8 of figure 6.18.2-1 are performed between AMF (or Target AMF), UE, and Target LMF with addition that UE also terminate connection to Source LMF.

3.	The AMF sends an Nlmf_Location_UPConfig Request towards the source LMF. The message may include a request for the Source LMF to terminate a specific user plane connection to the UE and the Target LMF identification. Alternatively, it may include information about AMF reallocation.

NOTE 2:	AMF relocation does not necessarily always cause LMF reselection, the AMF relocation information can keep the LMF informed.

4-5.[Conditional] The source LMF may invoke an Nlmf_Location_LocationContextTransfer Request service operation towards the target LMF to provide the current location context(s) of the UE, if there is user plane based periodic and triggered UE location events report context(s). The target LMF informs source LMF of the location context transfer operation results.

6.	[Conditional] If the user plane connection to source LMF is still active, the source LMF terminates the connection to the UE.

7.	The LMF sends Nlmf_Location_UPConfig Response message to AMF to confirm connection termination or acknowledge change of AMF. If this procedure is used for termination, the AMF will release the LCS-UP context after receiving the response message.

## ---End of the Change---