3GPP TSG-WG SA2 Meeting #166-AHE                                       	S2-2500865

# Online, January 20-24, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5294 | rev | --- | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_MVOSNS |  |  |  |  |  | Date: |  |  | 2025-1-20 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To use a more precise wording in clause 4.15.10B in order to refer to the UE Identifier when it is in the GPSI form of MSISDN. Currently the word “GPSI” is missing and by adding the word GPSI, the statement becomes consistent with such a statement already used in 4.15.10A. Additionally, throughout clause 5.2.6.2, the word “service” was missing from many service operation in the clause titles. To make the service operation names consistent throughout the spec. the word “service was added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to make editorial changes for consistently purposes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent usage of terms in the spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.10B and many subclauses within clause 5.2.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

### 4.15.10B	MSISDN verification

This clause contains the detailed description and procedures for the NEF verification (matching) of a retrieved MSISDN from the UDM against an AF-provided MSISDN. Depending on the operator policy and local regulation, the NEF verification result of the MSISDN may also be exposed to an authenticated and authorized AF. Depending on operator policy and local regulation, user consent may be required for MSISDN verification as described in clause 4.15.10A.

The figure and procedures of clause 4.15.10 shall be applicable for MSISDN verification exposure with the following differences:

-	description of the AF specific UE ID retrieval is replaced as the MSISDN verification exposure;

-	description of the AF specific UE Identifier represented as an External Identifier is replaced as the UE Identifier in the GPSI form of MSISDN; and

-	in step 1, the AF request message is replaced by Nnef_UEId_VerificationGet service operation and it shall also include GPSI in the form of MSISDN; and

-	in step 11, the NEF responds with Nnef_UEId_VerificationGet response indicating "true" or "false" depending on whether the retrieved MSISDN from the UDM and the AF-provided MSISDN match or not (for additional information see clause 5.2.6.27.7).

# *** 1st Change ***

#### 5.2.6.2	Nnef_EventExposure service

##### 5.2.6.2.1	General

See clause 4.15.3.1 and clause 4.15.2a.

In order to support the NWDAF to collect data from the AF via the NEF, the Event IDs associated with available data to be collected from the AF are defined in clause 5.2.19.2.1, which can be subscribed by the NWDAF. The Event filters for the specific Event IDs as defined in clause 5.2.19.2.1 can also be used for collecting data from the AF via the NEF.

##### 5.2.6.2.2	Nnef_EventExposure_Subscribe service operation

Service operation name: Nnef_EventExposure_Subscribe

Description: the consumer subscribes to receive an event, or if the event is already defined in NEF, then the subscription is updated.

Inputs, Required: (Set of) Event ID(s) as specified in clause 4.15.3.1 or Npcf_PolicyAuthorization_Notify and Naf_EventExposure_Subscribe service operation, Target of Event Reporting (GPSI, SUPI, UE IPv4 address(es), UE IPv6 prefix(es), External Group Identifier, S-NSSAI, Internal Group Identifier, UE addressing information (IP or MAC address), or indication that any UE is targeted), Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID), MTC Provider Information.

Inputs, Optional: Event Filter, (set of) External Application Identifier(s), Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, list of group member UE(s) whose subscription to event notification(s) are removed or added for a group-based event notification subscription, operation indication (cancellation or addition), DNN, S-NSSAI, Idle Status Indication request (if UE reachability or Availability after DDN failure reporting is requested).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available (see clause 4.15.1). External Identifier (representing an AF specific UE Identifier).

##### 5.2.6.2.3	Nnef_EventExposure_Unsubscribe service operation

Service operation name: Nnef_EventExposure_Unsubscribe

Description: the NF consumer deletes an event if already defined in NEF.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

##### 5.2.6.2.4	Nnef_EventExposure_Notify service operation

Service operation name: Nnef_EventExposure_Notify

Description: NEF reports the event to the consumer that has previously subscribed.

Inputs, Required: Event ID, Notification Correlation Information, time stamp.

Inputs, Optional: Event information (defined on a per Event ID basis), Event Removal Indication, list of group member UE(s) whose subscription to event notification(s) are removed from a group-based event notification subscription, UE(s) added/removed to/from the group.

Outputs, Required: Operation execution result indication.

#### 5.2.6.3	Nnef_PFDManagement service

##### 5.2.6.3.1	General

The service provides the capability to create, update or remove PFDs via the NEF (PFDF). See clause 4.18 for the detailed procedures.

##### 5.2.6.3.2	Nnef_PFDManagement_Fetch service operation

Service operation name: Nnef_PFDManagement_Fetch

Description: Provides the PFDs for Application Identifier to the NF Consumer.

Inputs, Required: Application Identifier(s).

Inputs, Optional: None.

Outputs, Required: Application Identifier, PFDs.

##### 5.2.6.3.3	Nnef_PFDManagement_Subscribe service operation

Service operation name: Nnef_PFDManagement_Subscribe

Description: provided by the NEF (PFDF) for NF consumers to explicitly subscribe the notification of changes of PFDs for Application Identifier.

Inputs, Required: Application Identifier(s).

Inputs, Optional: None.

Outputs, Required: None.

##### 5.2.6.3.4	Nnef_PFDManagement_Notify service operation

Service operation name: Nnef_PFDManagement_Notify

Description: Provides Update PFDs for Application Identifier to the NF Consumer.

Inputs, Required: Application Identifier(s), PFDs.

Inputs, Optional: None.

Outputs, Required: None.

##### 5.2.6.3.5	Nnef_PFDManagement_Unsubscribe service operation

Service operation name: Nnef_PFDManagement_Unsubscribe

Description: Provides by the NEF (PFDF) for NF Consumer to explicitly unsubscribe the notification of events.

Inputs, Required: Application Identifier(s).

Inputs, Optional: None.

Outputs, Required: None

##### 5.2.6.3.6	Nnef_PFDManagement_Create service operation

Service operation name: Nnef_PFDManagement_Create

Description: The consumer requests PFD management to add PFDs.

Inputs, Required: AF Identifier, External Application Identifier and one or more PFDs.

Inputs, Optional: Allowed Delay.

Outputs, Required: Transaction Reference ID.

##### 5.2.6.3.7	Nnef_PFDManagement_Update service operation

Service operation name: Nnef_PFDManagement_Update

Description: The consumer requests PFD management to update the set of PFDs by adding, modifying or deleting individual PFDs.

Inputs, Required: Transaction Reference ID, one or more PFDs.

NOTE:	The usage of the different components of the PFD information (especially the PFD id) to create, update and delete PFDs is defined in TS 29.522 [87].

Inputs, Optional: Allowed Delay.

Outputs, Required: None.

##### 5.2.6.3.8	Nnef_PFDManagement_Delete service operation

Service operation name: Nnef_PFDManagement_Delete

Description: The consumer requests PFD management to delete the PFDs.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: None.

#### 5.2.6.4	Nnef_ParameterProvision service

##### 5.2.6.4.1	General

This service is for allowing external party to provision of information which can be used for the UE in 5GS.

In all operations UE addressing information may correspond either to an UE IP address or to a UE MAC address.

##### 5.2.6.4.2	Nnef_ParameterProvision_Update service operation

Service operation name: Nnef_ParameterProvision_Update

Description: The consumer updates the UE related information (e.g. Expected UE Behaviour, Network Configuration parameters, Location Privacy Indication parameters, ECS Address Configuration Information, CAG Information provisioning parameters), Static IP address assignment parameters or 5G VN Group related information (e.g. 5G VN group data, 5G VN membership management), or for Multicast MBS related information or DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI.

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional: GPSI or UE addressing information, External Group ID, at least one of the Expected UE Behaviour parameters (optionally with associated confidence and/or accuracy levels) or at least one of the Application-Specific Expected UE Behaviour parameters (optionally with associated confidence and/or accuracy levels) or at least one of the Network Configuration parameters or 5G VN related information or ECS Address Configuration Information, Static IP address assignment parameters, Validity Time or Location Privacy Indication parameters, MTC Provider Information, or Multicast MBS related information, DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI, PLMN IDs, CAG Information provisioning parameters.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available. External Identifier (representing an AF specific UE Identifier).

For Multicast MBS related information, refer to TS 23.247 [78].

##### 5.2.6.4.3	Nnef_ParameterProvision_Create service operation

Service operation name: Nnef_ParameterProvision_Create

Description: The consumer provisions UE related information or creates a 5G VN group, or Multicast MBS related information group, or CAG Information.

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional: GPSI or UE addressing information, one or multiple Expected UE Behaviour parameters (optionally with associated confidence and/or accuracy levels) or one or multiple Application-Specific Expected UE Behaviour parameters (optionally with associated confidence and/or accuracy levels), External Group ID for 5G VN group creation or for multicast MBS group creation, External Group ID, 5G VN group related information (e.g. 5G VN group data, 5G VN membership management), MTC Provider Information, Multicast MBS related information, DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI, PLMN IDs, ECS Address Configuration Information, Static IP address assignment parameters, CAG Information provisioning parameters.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available. External Identifier (representing an AF specific UE Identifier).

For Multicast MBS related information, refer to TS 23.247 [78].

##### 5.2.6.4.4	Nnef_ParameterProvision_Delete service operation

Service operation name: Nnef_ParameterProvision_Delete

Description: The consumer deletes a 5G VN group or deletes a Multicast MBS related information or deletes Static IP address assignment parameters, or deletes CAG Information.

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional: External Group ID, Static IP address assignment parameters.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

For Multicast MBS related information, refer to TS 23.247 [78].

##### 5.2.6.4.5	Nnef_ParameterProvision_Get service operation

Service operation name: Nnef_ParameterProvision_Get

Description: The consumer gets the UE related information (e.g. Expected UE Behaviour, Network Configuration parameters, ECS Address Configuration Information).

Inputs, Required: GPSI or UE addressing information, AF Identifier, requested information (e.g. Expected UE Behaviour, Network Configuration parameters, ECS Address Configuration Information).

Inputs, Optional: None.

Outputs, Required: Requested data, Operation execution result indication.

Outputs, Optional: External Identifier (representing an AF specific UE Identifier).

#### 5.2.6.5	Nnef_Trigger service

##### 5.2.6.5.1	General

See clause 4.13.2.

##### 5.2.6.5.2	Nnef_Trigger_Delivery service operation

Service operation name: Nnef_Trigger_Delivery

Description: the consumer requests that a trigger be sent to an application on a UE and subscribes to be notified about result of the trigger delivery attempt.

Inputs, Required: GPSI, AF Identifier, Trigger Reference Number, Application Port ID

Inputs, Optional: Validity Period, Priority, Trigger Payload.

Outputs, Required: Transaction Reference ID, Cause.

##### 5.2.6.5.3	Nnef_Trigger_DeliveryNotify service operation

Service operation name: Nnef_Trigger_DeliveryNotify

Description: NEF reports the status of the trigger delivery to the consumer (failure or success).

NOTE:	This notification corresponds to an implicit subscription by Nnef_Trigger_Delivery service operation.

Inputs, Required: Transaction Reference ID, Delivery Report.

Inputs, Optional: None.

Outputs, Required: None.

#### 5.2.6.6	Nnef_BDTPNegotiation service

##### 5.2.6.6.1	General

See clause 4.16.7.

##### 5.2.6.6.2	Nnef_BDTPNegotiation_Create service operation

Service operation name: Nnef_BDTPNegotiation_Create.

Description: The consumer requests a background data transfer policy.

Inputs, Required: ASP Identifier, Volume per UE, Number of UEs, Desired time window.

Inputs, Optional: External Group Identifier, Network Area Information, Request for notification, MAC address or IP 3-tuple of Application server.

Outputs, Required: Background Data Transfer Reference ID, one or more background data transfer policies.

Output, Optional: None.

##### 5.2.6.6.3	Nnef_BDTPNegotiation_Update service operation

Service operation name: Nnef_BDTPNegotiation Update.

Description: the consumer requests the selected background data transfer policy to be set.

Inputs, Required: Background Data Transfer Reference ID, background data transfer policy.

Inputs, Optional: Stop notification.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.6.6.4	Nnef_BDTPNegotiation_Notify service operation

Service operation name: Nnef_BDTPNegotiation_Notify

Description: NEF sends the BDT warning notification to the NF consumer.

Inputs, Required: Background data transfer reference ID.

Inputs, Optional: Network Area Information, Time window, list of candidate Background DataTransfer policies.

Outputs, Required: None.

Outputs, Optional: None.

#### 5.2.6.7	Nnef_TrafficInfluence service

##### 5.2.6.7.1	General

Service description: This service provides:

-	Request authorization of NF Service Consumer requests.

-	Request parameter mapping from NF Service Consumer requests to 5GC parameters and vice versa as described in clause 5.6.7 of TS 23.501 [2].

-	NF Service Consumer request routing (forwarding) to actual NF Service Producer to influence traffic routing decisions as described in clause 5.6.7 of TS 23.501 [2].

##### 5.2.6.7.2	Nnef_TrafficInfluence_Create service operation

Service operation name: Nnef_TrafficInfluence_Create

Description: Authorize the request and forward the request for traffic influence.

Inputs, Required: AF Transaction Id, AF Identifier.

The AF Transaction Id refers to the request.

Inputs, Optional: The address (IP or Ethernet) of one or more UE(s) if available, one or more GPSIs if available, DNN if available, S-NSSAI if available, External Group Identifier(s) if available, External Application Identifier or traffic filtering information, AF-Service-Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, Indication of traffic correlation, Indication of application relocation possibility, Indication of UE IP address preservation, Early and/or late notifications about UP path management events, Notification Target Address, immediate reporting flag, Temporal validity condition, Spatial validity condition, User Plane Latency Requirements, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation and AF indication for simultaneous connectivity over source and target PSA at edge relocation, EAS Correlation indication, External Subscriber Category(s), SFC Identifier(s), Metadata, Common EAS IP address, Traffic Correlation ID, FQDN(s), Indication of considering N6 delay and Header Handling Control information as described in clauses 5.6.7, 5.6.16 and 5.6.17 of TS 23.501 [2], HPLMN of the UE. If an IP address of one or more UE(s) is provided, a corresponding port number (e.g. TCP or UPDP) for each IP address.

The IP address of one or more UE(s) may be provided together with the corresponding port number allowing to support the case where the PSA UPF is carrying out NAT on the traffic exchanged with the EAS.

NOTE 1:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

NOTE 2:	In this Release the following cannot be used by AFs in VPLMN influencing traffic on Home Routed PDU sessions:

-	External Group Identifier;

-	External Subscriber Category.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.7.3	Nnef_TrafficInfluence_Update service operation

Service operation name: Nnef_TrafficInfluence_Update

Description: Authorize the request and forward the request to update the traffic influence.

Inputs, Required: AF Transaction Id.

The AF Transaction Id identifies the NF Service Consumer request to be updated.

Inputs, Optional: Same optional information as in Nnef_TrafficInfluence_Create Input, AF Identifier.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.7.4	Nnef_TrafficInfluence_Delete service operation

Service operation name: Nnef_TrafficInfluence_Delete

Description: Authorize the request and forward the request to delete(s) request for traffic influence.

Inputs, Required: AF Transaction Id.

The AF Transaction Id identifies the NF Service Consumer request for traffic influence to be deleted.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.7.4A	Nnef_TrafficInfluence_Get service operation

Service operation name: Nnef_TrafficInfluence_Get

Description: Get the current traffic influence parameters.

Inputs, Required: AF Transaction Id.

The AF Transaction Id refers to the request.

Inputs, Optional: The address (IP or Ethernet) of the UE if available, GPSI if available, DNN if available, S-NSSAI if available, External Group Identifier if available, External Application Identifier or traffic filtering information, AF-Service-Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information.

Outputs, Required: Operation execution result indication, requested data.

Outputs, Optional: None.

##### 5.2.6.7.5	Nnef_TrafficInfluence_Notify service operation

Service operation name: Nnef_TrafficInfluence_Notify

Description: Forward the notification of UP path management event report to AF.

Known NF Service Consumers: AF.

Inputs, Required: AF Transaction Id, Event ID.

The AF Transaction Id identifies the AF request for traffic influence that the event report is related to. The event may be the UP path management event defined in clause 5.6.7 of TS 23.501 [2].

Inputs, Optional: Event information (defined on a per Event ID basis), capability of supporting EAS IP replacement in 5GC.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.7.6	Nnef_TrafficInfluence_AppRelocationInfo service operation

Service operation name: Nnef_TrafficInfluence_AppRelocationInfo

Description: Forward the acknowledgement to the notification of UP path management event report to SMF.

Inputs, Required: Notification Correlation Information, cause code.

Cause code indicates whether the acknowledgement is a positive response or a negative response.

Inputs, Optional: N6 traffic routing information as described in clause 5.6.7 of TS 23.501 [2], Indication that buffering of uplink traffic should start, Information for EAS IP Replacement in 5GC.

Outputs, Required: None.

Outputs, Optional: None.

See clause 4.3.6.3 for details on usage of this service operation for example for the usage of the Indication that buffering of uplink traffic should start.

#### 5.2.6.8	Nnef_ChargeableParty service

##### 5.2.6.8.1	General

See clauses 4.15.6.4 and 4.15.6.5.

##### 5.2.6.8.2	Nnef_ChargeableParty_Create service operation

Service operation name: Nnef_ChargeableParty Create

Description: The consumer requests to become the chargeable party for a data session for a UE.

Inputs, Required: AF Identifier, UE address (i.e. IP address or MAC address), Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, ASP Identifier, Sponsor Information, Sponsoring Status.

Inputs, Optional: Time period, traffic volume, Background Data Transfer Reference ID, DNN if available, S-NSSAI if available.

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.8.3	Nnef_ChargeableParty_Update service operation

Service operation name: Nnef_ChargeableParty Update

Description: The consumer can change the chargeable party of a data session for a UE.

Inputs, Required: AF Identifier, Transaction Reference ID, Sponsoring Status.

Inputs, Optional: Time period, traffic volume, Background Data Transfer Reference ID.

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.8.4	Nnef_ChargeableParty_Notify service operation

Service operation name: Nnef_ChargeableParty Notify

Description: NEF reports the bearer level event(s) to the consumer.

Inputs, Required: Event reports.

Inputs, Optional: None.

Outputs, Required: None.

Output (optional): None.

#### 5.2.6.9	Nnef_AFsessionWithQoS service

##### 5.2.6.9.1	General

See clauses 4.15.6.6, 4.15.6.13, 4.15.6.14.

This service is also used to support subscription and notification of QoS Monitoring for the QoS parameter(s) to be measured defined in clause 5.45 of TS 23.501 [2].

This service is also used to support subscription and notification of BAT offset for the AF that supports adjusting burst sending time based on RAN feedback, as described in clause 5.27.2.5 of TS 23.501 [2].

This service is also used to support the QoS resource allocation for a list of UEs.

This service is also used to support subscription and notification of QoS Monitoring event for Data Rate Monitoring for a list of QoS flows, as described in clause 5.2.26.

##### 5.2.6.9.2	Nnef_AFsessionWithQoS_Create service operation

Service operation name: Nnef_AFsessionWithQoS Create

Description: The consumer requests the network to provide a specific QoS for an AF session for a UE or a list of UEs.

Inputs, Required: AF Identifier, UE address (i.e. IP address or MAC address and only applicable for a single UE AF session), a list of UE addresses (as described in clause 4.15.6.13, and only applicable for a Multi-member AF session). Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

NOTE 1:	In this Release, when a list of UE addresses is provided, the Flow description information is common for all UE addresses in the list. Further details are described in clause 4.15.6.13.2.

Inputs, Optional: Time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS Monitoring parameter(s) as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available.

Only applicable for a single UE AF session: flow direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Periodicity as described in clause 5.27.2 or clause 5.37.8.2 of TS 23.501 [2], Time domain, Survival Time, BAT Window or Capability for BAT adaptation, Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], Periodicity Range, RT Latency Indication as described in clause 6.1.3.22 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description (as described in clause 5.37.5, 5.37.8.3 and other (sub)clauses in clause 5.37 of TS 23.501 [2]), Indication of ECN marking for L4S as described in clause 6.1.3.22 of TS 23.503 [20], Data Burst Size Marking Support Indication as described in clause 6.1.3.27.5 of TS 23.503 [20], Time to Next Burst Support Indication as described in clause 6.1.3.27.5 of TS 23.503 [20], QoS duration, QoS inactivity interval, Multi-Modal Service ID together with Multi-modal Service Requirements information for each data flow as described in clause 6.1.3.27.3 of TS 23.503 [20].

Only applicable for a Multi-member AF session: Consolidated Data Rate Threshold, a list of UE addresses subject to Consolidated Data Rate monitoring.

NOTE 2:	If Consolidated Data Rate Threshold is provided, the QoS Monitoring parameter(s) indicates the Guaranteed Bitrate shall be provided.

NOTE 3:	When the AF request is for Consolidated Data Rate monitoring is set for event reporting, the QoS Flow data rate reporting for the list of UEs provided to the AF by the NEF only when the Consolidated Data Rate threshold is exceeded.

NOTE 4:	When the Consolidated Data Rate threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subject for Consolidated Data Rate monitoring is also provided, then such list has to be the subset of the list of UE addresses.

Outputs, Required: Transaction Reference ID, result (result as described in clause 4.15.6.13 if a list of UE is targeted).

Output (optional): None.

##### 5.2.6.9.3	Nnef_AFsessionWithQoS_Notify service operation

Service operation name: Nnef_AFsessionWithQoS Notify

Description: NEF reports the QoS Flow level event(s) to the consumer.

Inputs, Required: Reports of the events as defined in clause 6.1.3.18 of TS 23.503 [20], Transaction Reference ID if a list of UE is targeted.

NOTE:	When the event report is for Consolidated Data Rate monitoring, the QoS Flow data rate reporting for the list of UEs provided to the AF by the NEF only when the Consolidated Data Rate threshold is exceeded.

Inputs, Optional: When the event report is for QoS Monitoring, includes QoS Monitoring report for the QoS parameter(s) to be measured defined in clause 5.45 of TS 23.501 [2], e.g. UL packet delay, DL packet delay, or round trip packet delay, UL and/or DL data rate of the single UP path or two UP paths in the case of redundant transmission, as defined in clause 5.33.3.2 of TS 23.501 [2]. When the AF has provided Capability for BAT Adaptation or BAT Window, can include BAT offset as described in clause 5.27.2.5 of TS 23.501 [2]. When the delivery report is for Consolidated Data Rate Monitoring, includes Consolidated Data Rate Monitoring report.

Outputs, Required: None.

Output (optional): None.

##### 5.2.6.9.4	Nnef_AFsessionWithQoS_Revoke service operation

Service operation name: Nnef_AFsessionWithQoS Revoke

Description: The consumer requests the network to revoke the AF session with requested QoS or the AF session with requested QoS including Alternative Service Requirements for a UE or a list of UEs.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Transaction Reference ID, result (result as described in clause 4.15.6.3 if a list of UE is targeted).

Output (optional): None.

##### 5.2.6.9.5	Nnef_AFsessionWithQoS_Update service operation

Service operation name: Nnef_AFsessionWithQoS Update

Description: The consumer requests the network to update the parameters for an AF session for a UE or a list of UEs.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: Flow description information (as described in clause 6.1.3.6 of TS 23.503 [20]) or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS Monitoring parameter(s) as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20].

Only applicable for a single UE AF session: flow direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Periodicity as described in clause 5.27.2 or clause 5.37.8.2 of TS 23.501 [2], Time domain, Survival Time, Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], BAT Window or Capability for BAT adaptation, Periodicity Range, PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description (as described in clause 5.37.5, 5.37.8.3 and other (sub)clauses in clause 5.37 of TS 23.501 [2]), Data Burst Size Marking Support Indication as described in clause 6.1.3.27.5 of TS 23.503 [20], Time to Next Burst Support Indication as described in clause 6.1.3.27.5 of TS 23.503 [20], updated information for the Multi-modal Service Requirements information as described in clause 6.1.3.27.3 of TS 23.503 [20].

Only applicable for a Multi-member AF session: a list of UE addresses (as described in clause 4.15.6.13), Consolidated Data Rate Threshold, a list of UE addresses subject to Consolidated Data Rate monitoring.

NOTE 1:	In this Release, when a list of UE addresses is provided, the Flow description information is common for all UE addresses in the list. Further details are described in clause 4.15.6.13.2.

NOTE 2:	If Consolidated Data Rate Threshold is provided, the QoS Monitoring parameter(s) indicates the Guaranteed Bitrate shall be provided.

NOTE 3:	When the AF request is for Consolidated Data Rate monitoring is set for event reporting, the QoS Flow data rate reporting for the list of UE addresses is provided to the AF by the NEF only when the Consolidated Data Rate threshold is exceeded.

NOTE 4:	When the Consolidated Data Rate threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subject for Consolidated Data Rate monitoring is also provided, then such list has to be the subset of the list of UE addresses.

NOTE 5:	If AF wants to terminate the Consolidated Data Rate monitoring, AF does not include the Consolidated Data Rate threshold in the AF request.

Outputs, Required: Success or Failure. Failure Cause in case of Failure, Transaction Reference ID if a list of UE is targeted.

Output (optional): None.

#### 5.2.6.10	Nnef_MSISDN-less_MO_SMS service

##### 5.2.6.10.1	General

See clause 4.13.7.

##### 5.2.6.10.2	Nnef_MSISDN-less_MO_SMSNotify service operation

Service operation name: Nnef_MSISDN-less_MO_SMSNotify

Description: NEF delivers to the SMS Payload to the AF. The NEF shall determine the notification destination URL of the AF based on configured information on the mapping of SME addresses to destination URLs.

Inputs, Required: SMS payload, GPSI and Application Port ID.

Inputs, Optional: None.

Outputs, Required: None.

#### 5.2.6.11	Nnef_ServiceParameter service

##### 5.2.6.11.1	General

This service is for allowing external party to provision of service specific parameters which can be used for the UE in 5GS. The detailed information is described in clause 4.15.6.7 and 4.15.6.15.

##### 5.2.6.11.2	Nnef_ServiceParameter_Create service operation

Service operation name: Nnef_ServiceParameter_Create

Description: The consumer stores service specific parameters in the UDR via the NEF.

Inputs, Required: Service Descriptor (e.g. the combination of DNN and S-NSSAI, an AF-Service-Identifier or an External Application Identifier)

Inputs, Optional: Service Parameters and Target UE identifiers (e.g. the address (IP or Ethernet) of the UE if available, GPSI if available, External Group Identifier if available), "PLMN ID(s) of inbound roamers", subscribedEvents, notificationDestination, Non-3GPP Device Identifier Information(s) as defined in the clause 4.15.6.15.

If identifiers of target UE(s) or a group of UEs or "PLMN ID(s) of inbound roamers" are not provided, then the Service Parameters shall correspond to any UEs of the PLMN of the NEF using the service identified by the Service Description.

Outputs, Required: Transaction Reference ID, operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.11.3	Nnef_ServiceParameter_Update service operation

Service operation name: Nnef_ServiceParameter_Update

Description: The consumer updates service specific parameters in the UDR via the NEF.

Inputs, Required: Service Descriptor (e.g. the combination of DNN and S-NSSAI, an AF-Service-Identifier or an External Application Identifier), Transaction Reference ID.

Inputs, Optional: Service Parameters and Target UE identifiers (e.g. the address (IP or Ethernet) of the UE if available, GPSI if available, External Group Identifier if available), or "PLMN ID(s) of inbound roamers", Non-3GPP Device Identifier Information(s) as defined in the clause 4.15.6.15.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.11.4	Nnef_ServiceParameter_Delete service operation

Service operation name: Nnef_ServiceParameter_Delete

Description: The consumer deletes service specific parameters from the UDR via the NEF.

Inputs, Required: Service Descriptor (e.g. the combination of DNN and S-NSSAI, an AF-Service-Identifier or an External Application Identifier), Transaction Reference ID.

Inputs, Optional: Target UE identifiers (e.g. the address (IP or Ethernet) of the UE if available, GPSI if available, External Group Identifier if available) or "PLMN ID(s) of inbound roamers".

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.11.5	Nnef_ServiceParameter_Get service operation

Service operation name: Nnef_ServiceParameter_Get

Description: The consumer retrieves service specific parameters in the UDR via the NEF.

Inputs, Required: Service Descriptor (e.g. the combination of DNN and S-NSSAI, an AF-Service-Identifier or an External Application Identifier).

Inputs, Optional: Service Parameters and Target UE identifiers (e.g. the address (IP or Ethernet) of the UE if available, GPSI if available, External Group Identifier if available), or "PLMN ID(s) of inbound roamers".

Outputs, Required: Transaction Reference ID, operation execution result indication, requested data.

Outputs, Optional: None.

##### 5.2.6.11.6	Nnef_ServiceParameter_Notify service operation

Service operation name: Nnef_ServiceParameter_Notify

Description: This service operation is used by the NEF to notify a Service Parameter Authorisation Update (e.g. to revoke an authorization) to AF, see clause clause 4.15.6.7a or to forward a notification for a subscribed event by the AF, see clause 4.15.6.7, e.g. the notification of outcome of UE Policies Delivery to AF.

Inputs, Required: Transaction Reference ID, Target UE (i.e. GPSI), Result.

The Transaction Reference ID identifies the AF request for service specific parameter provisioning that the notification (i.e. notification of an authorization update or reporting a subscribed event) is related to. The GPSI is the identifier of the UE for which the notification is related to.

Inputs, Optional: DNN, S-NSSAI, PLMN ID(s) of inbound roamers, External Group Id, Event Information (defined on a per Event ID basis, for UE Policies delivery outcome it may include the result of the UE Policies delivery procedure and for unsuccessful results, an identifier of the reason), authorization update information (e.g. authorization revocation cause).

The Event ID may be the UE Policies delivery outcome defined in clause 4.15.6.7. The GPSI is the identifier of the UE for which the event report is related to.

The NEF provides, in addition to the GPSI, the External-Group-Id if provided as Target UE in the Nnef_ServiceParameter_Subscribe operation.

Outputs, Required: None.

Outputs, Optional: None.

#### 5.2.6.12	Nnef_APISupportCapability service

##### 5.2.6.12.1	General

In order to support interworking with EPC, the NEF is integrated with SCEF as a combined SCEF+NEF node for network exposure as defined in clause 5.17.5 of TS 23.501 [2].

The common services provided by SCEF+NEF may become unavailable because the UE is now being served by a node (e.g. MME) or NF (e.g. AMF) that does not support that service. If the availability or expected level of support of a service API associated with a UE changes, for example due to a mobility between 5GC and EPC, the AF can be made aware of the change via this service.

This service consists of Subscribe, Unsubscribe, Notify service operations. It supports informing AF of the availability or expected level of support of a given service via a response to the One-time report type subscribe request of the AF or via a notification to the Continuous report type subscription of the AF.

##### 5.2.6.12.2	Nnef_APISupportCapability_Subscribe service operation

Service operation name: Nnef_APISupportCapability_Subscribe

Description: The AF subscribes to receive notification about the availability or expected level of support of a service API for a UE or a group of UEs.

Inputs, Required: UE ID or External Group ID, Report Type (One-time report or Continuous report).

Inputs, Optional: Duration of Reporting, callback URI.

Outputs, Required: Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, API indication.

##### 5.2.6.12.3	Nnef_APISupportCapability_Notify service operation

Service operation name: Nnef_APISupportCapability_Notify

Description: The AF is notified about the availability or expected level of support of a service API for a UE or a group of UEs if it has subscribed to receive it.

Inputs, Required: API Indication, UE ID or External Group ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

##### 5.2.6.12.4	Nnef_APISupportCapability_Unsubscribe service operation

Service operation name: Nnef_APISupportCapability_Unsubscribe

Description: The AF unsubscribes to receive notification about the availability or expected level of support of a service API for a UE or a group of UEs.

Inputs, Required: UE ID or External Group ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

#### 5.2.6.13	Nnef_NIDDConfiguration service

##### 5.2.6.13.1	General

See clause 4.25.3.

##### 5.2.6.13.2	Nnef_NIDDConfiguration_Create service operation

Service operation name: Nnef_NIDDConfiguration_Create

Description: This service operation is used by the consumer to request NIDD Configuration between NF consumer and NEF to support NIDD via NEF.

Inputs, Optional: Reliable Data Service Configuration, Requested Action, TLTRI, NIDD Duration.

Inputs, Required: GPSI or External Group Identifier, AF Identifier, T8 Destination Address, MTC Provider Information).

Outputs, Required: TLTRI, Reliable Data Service Indication, Maximum Packet Size, Cause.

Outputs, Optional: NIDD Duration.

##### 5.2.6.13.3	Nnef_NIDDConfiguration_TriggerNotify service operation

Service operation name: Nnef_NIDDConfiguration_TriggerNotify

Description: NEF triggers NIDD Configuration Create Request if there is no NIDD Configuration between NF consumer and NEF.

Inputs, Required: GPSI, AF Identifier, NEF ID.

Outputs, Required: None.

##### 5.2.6.13.4	Nnef_NIDDConfiguration_UpdateNotify service operation

Service operation name: Nnef_NIDDConfiguration_UpdateNotify

Description: This service operation is used by the NEF to notify NF consumer the NIDD Configuration Update due to NIDD Authorization Update.

Inputs, Required: GPSI, TLTRI, Result.

Inputs, Optional: Cause, NIDD Duration.

Outputs, Required: None.

##### 5.2.6.13.5	Nnef_NIDDConfiguration_Delete service operation

Service operation name: Nnef_NIDDConfiguration_Delete

Description: This service operation is used by the NF consumer to request NIDD Configuration delete between NF consumer and NEF to support NIDD via NEF.

Inputs, Required: TLTRI.

Outputs, Required: Cause.

#### 5.2.6.14	Nnef_NIDD service

##### 5.2.6.14.1	General

See clauses 4.25.4, 4.25.5 and 4.25.9.

##### 5.2.6.14.2	Nnef_NIDD_Delivery service operation

Service operation name: Nnef_NIDD_Delivery

Description: This service operation is used by the NF consumer to deliver the unstructured data between NF consumer and NEF to support NIDD via NEF.

Inputs, Required: User Identity or External Group Identifier, unstructured data, TLTRI (Optional), Reliable Data Service Configuration (Optional), MO Exception Data Counter.

Outputs, Required: Cause.

##### 5.2.6.14.3	Nnef_NIDD_DeliveryNotify service operation

Service operation name: Nnef_NIDD_DeliveryNotify

Description: This service operation is used by the NEF to forward the unstructured data to NF consumer to support NIDD via NEF.

Inputs, Required: User Identity, unstructured data, TLTRI (Optional), Reliable Data Service Configuration (Optional).

Outputs, Required: Cause.

##### 5.2.6.14.4	Nnef_NIDD_GroupDeliveryNotify service operation

Service operation name: Nnef_NIDD_GroupDeliveryNotify

Description: This service operation is used by the NEF to inform the AF of the aggregated response of Group MT NIDD operation.

Inputs, Required: The TLTRI that was provided when the Nnef_NIDD_Delivery operation was invoked with an External Group Identifier, External Identifier list with associated with the External Group Identifier and cause values.

Inputs, Optional: Re-transmission Time(s).

Outputs, Required: None.

#### 5.2.6.15	Nnef_SMContext service

##### 5.2.6.15.1	General

The service provides the capability to create, update or release the SMF-NEF Connection. See clause 4.25.2 for the detailed procedure.

##### 5.2.6.15.2	Nnef_SMContext_Create service operation

Service operation name: Nnef_SMContext_Create

Description: This service operation is used by the consumer to request connection establishment between NF consumer and NEF to support NIDD via NEF.

Inputs, Required: User Identity, PDU session ID, NIDD information, S-NSSAI, DNN.

Inputs, Optional: RDS support indication, Serving PLMN Rate Control parameters, Small Data Rate Control parameters and Small Data Control Status.

Outputs, Required: Cause.

Outputs, Optional: Extended Buffering Support, NIDD Parameters, RDS support indication.

##### 5.2.6.15.3	Nnef_SMContext_Delete service operation

Service operation name: Nnef_SMContext_Delete

Description: This service operation is used by the NF consumer to request SMF-NEF Connection release between NF consumer and NEF to support NIDD via NEF.

Inputs, Required: User Identity, PDU Session ID, NEF ID, S-NSSAI, DNN, Release Cause.

Outputs, Required: Cause.

Outputs, Optional: Small Data Rate Control Status, APN Rate Control Status.

##### 5.2.6.15.4	Nnef_SMContext_DeleteNotify service operation

Service operation name: Nnef_SMContext_DeleteNotify

Description: This service operation is used by the NEF to notify NF consumer that the SMF-NEF Connection for NIDD via NEF is no longer valid.

Inputs, Required: User Identity, PDU session ID, S-NSSAI, DNN, Reason of the SMF-NEF connection release

Inputs, Optional: Small Data Rate Control Status, APN Rate Control Status.

Outputs, Required: Cause.

##### 5.2.6.15.5	Nnef_SMContext_Delivery service operation

Service operation name: Nnef_SMContext_Delivery

Description: This service operation is used by the NF consumer to deliver the unstructured data between NF consumer and NEF to support NIDD.

Inputs, Required: User Identity, PDU session ID, unstructured data.

Outputs, Required: Cause.

#### 5.2.6.16	Nnef_AnalyticsExposure service

##### 5.2.6.16.1	General

This service is for allowing NF Service Consumer to ask analytics information as specified in TS 23.288 [50].

##### 5.2.6.16.2	Nnef_AnalyticsExposure_Subscribe service operation

Service operation name: Nnef_AnalyticsExposure_Subscribe

Description: The NF consumer subscribes or modifies an existing subscription on analytics information.

Inputs, Required: (Set of) Analytics ID(s), Analytic Filter Information, Target of Analytic Reporting (UEs (e.g. GPSI), External Group Identifier, any UEs), Analytic Reporting Information, Notification Target Address (+ Notification Correlation ID). These input parameters are detailed in TS 23.288 [50].

Inputs, Optional: Subscription Correlation ID (in the case of modification of the analytics subscription), Expiry time, slice specific information, Geographical area, Analytics feedback information.

NOTE 1:	When the Analytics ID is set to "User Data Congestion", the input parameters are defined in TS 23.288 [50].

NOTE 2:	The Geographical area could be provided as e.g. shapes (e.g. polygons, circles, etc.) or civic addresses (e.g. streets, districts, etc.) as referenced by OMA Presence API.

NOTE 3:	The Analytics feedback information only can be included in modification request for the existing analytics subscription as described in TS 23.288 [50].

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding analytics report is included, if available.

##### 5.2.6.16.3	Nnef_AnalyticsExposure_Unsubscribe service operation

Service operation name: Nnef_AnalyticsExposure_Unsubscribe

Description: The NF consumer unsubscribes to an existing subscription on analytics information.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

##### 5.2.6.16.4	Nnef_AnalyticsExposure_Notify service operation

Service operation name: Nnef_AnalyticsExposure_Notify

Description: NEF reports the analytics to the NF consumer that has previously subscribed.

Inputs, Required: Analytics ID(s), Notification Correlation Information, Analytic information (defined on a per Analytics ID basis). These input parameters are detailed in TS 23.288 [50].

Inputs, Optional: Timestamp of analytics generation, Probability assertion, Termination Request specified in TS 23.288 [50].

Outputs, Required: Operation execution result indication.

##### 5.2.6.16.5	Nnef_AnalyticsExposure_Fetch service operation

Service operation name: Nnef_AnalyticsExposure_Fetch

Description: The NF consumer requests analytics information.

Inputs Required: Analytics ID, Analytic Filter Information, Target of Analytic Reporting (UE (e.g. GPSI), External Group Identifier, any UEs), Analytic Reporting Information. These input parameters are detailed in TS 23.288 [50].

Inputs, Optional: Slice specific information, Geographical area.

NOTE 1:	When the Analytics ID is set to "User Data Congestion", the input parameters are defined in TS 23.288 [50].

NOTE 2:	The Geographical area could be provided as e.g. shapes (e.g. polygons, circles, etc.) or civic addresses (e.g. streets, districts, etc.) as referenced by OMA Presence API.

Outputs Required: Analytic information (defined on a per Analytics ID basis) specified in TS 23.288 [50].

Outputs, Optional: Timestamp of analytics generation, Probability assertion, specified in TS 23.288 [50].

#### 5.2.6.17	Nnef_UCMFProvisioning service

##### 5.2.6.17.1	General

This service is for allowing external party to provision the UCMF with UCMF dictionary entries for Manufacturer-assigned UE Radio Capability IDs.

##### 5.2.6.17.2	Nnef_UCMFProvisioning_Create service operation

Service operation name: Nnef_UCMFProvisioning_Create

Description: The consumer creates a UCMF dictionary entry (or more entries) for a Manufacturer-assigned UE Radio Capability ID via the NEF. For each UE Radio Capability ID the following inputs are provided:

a)	a UE radio access capability set with respective Coding format or the UE radio access capability set in both TS 36.331 [16] and TS 38.331 [12] coding formats and each RATs' UE Radio Capability for Paging; and

b)	the related UE model(s) IMEI/TAC value(s) the UE radio capability ID applies to.

Inputs, Required: (list of) [UE Radio Capability ID, set(s) of UE Radio Access Capability set and UE Radio Capability for Paging and respective Coding format(s), (List of) IMEI/TAC value(s)].

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

The Coding format(s) indicates the format of the respective UE radio access capabilities as defined in TS 36.331 [16] or TS 38.331 [12].

##### 5.2.6.17.3	Nnef_UCMFProvisioning_Delete service operation

Service operation name: Nnef_UCMFProvisioning_Delete

Description: The consumer deletes a UCMF dictionary entry for a Manufacturer-assigned UE Radio Capability ID via the NEF. The consumer provides a (list of) UE radio capability ID value(s) to be deleted or it may provide the IMEI/TAC values for which the associated UE radio capability ID entries shall be no longer used.

Inputs, Required: UE Radio Capability ID(s) of the UCMF dictionary entry(ies) to be deleted or IMEI/TAC(s) that no longer use associated UE radio Capability ID(s).

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.6.17.4	Nnef_UCMFProvisioning_Update service operation

Service operation name: Nnef_UCMFProvisioning_Update

Description: The consumer updates the list of IMEI/TAC values a UCMF dictionary entry(or a list of entries) applies to for a Manufacturer-assigned UE Radio Capability ID via the NEF. For each UE Radio Capability ID provided, (a list of) UE model(s) IMEI/TAC value(s) to be added or removed to the related UCMF entry is provided.

Inputs, Required: Update Type (one of "Add IMEI/TAC Values" or "Remove IMEI/TAC Values") and:

-	If Update Type is "Add IMEI/TAC Values", the (list of) UE Radio Capability ID(s) of the UCMF dictionary entry(ies) to be updated and the related additional (list of) IMEI/TAC(s); or

-	If Update Type is "Remove IMEI/TAC Values", the (list of) UE Radio Capability ID(s) of the UCMF dictionary entry(ies) to be updated and the related (list of) IMEI/TAC(s) to be removed.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

#### 5.2.6.18	Nnef_ECRestriction service

##### 5.2.6.18.1	General

This service is for allowing the AF to query status of Enhanced Coverage Restriction or enable/disable Enhanced Coverage Restriction per individual UE.

##### 5.2.6.18.2	Nnef_ECRestriction_Get service operation

Service operation name: Nnef_ECRestriction_Get

Description: The consumer query the status of Enhanced Coverage Restriction information.

Inputs, Required: GPSI, AF Identifier.

Inputs, Optional:

Outputs, Required: Operation execution result indication.

Outputs, Optional: Enhanced Coverage Restriction information.

##### 5.2.6.18.3	Nnef_ECRestriction_Update service operation

Service operation name: Nnef_ECRestriction_Update

Description: The consumer enables or disables the Enhanced Coverage Restriction.

Inputs, Required: GPSI, AF Identifier, Enhanced Coverage Restriction information.

Inputs, Optional: MTC Provider Information.

Outputs, Required: Operation execution result indication.

Outputs, Optional:

#### 5.2.6.19	Nnef_ApplyPolicy service

##### 5.2.6.19.1	General

The service provides the capability to apply a previously negotiated Background Data Transfer Policy to a UE or a group of UEs. See clause 4.15.6.8 for the detailed procedure.

##### 5.2.6.19.2	Nnef_ApplyPolicy_Create service operation

Service operation name: Nnef_ApplyPolicy Create

Description: The consumer requests to apply a policy to the UE.

Inputs, Required: AF Identifier, External Identifier or External Group ID, Background Data Transfer Reference ID for a previously negotiated policy of a background data transfer.

Inputs, Optional: None.

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.19.3	Nnef_ApplyPolicy_Update service operation

Service operation name: Nnef_ApplyPolicy Update

Description: The consumer requests to update a policy to the UE.

Inputs, Required: Transaction Reference ID, Background Data Transfer Reference ID for a previously negotiated policy of a background data transfer.

Inputs, Optional: None.

Outputs, Required: Result.

Output (optional): None.

##### 5.2.6.19.4	Nnef_ApplyPolicy_Delete service operation

Service operation name: Nnef_ApplyPolicy Delete

Description: The consumer requests to delete a policy to the UE.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Result.

Output (optional): None.

#### 5.2.6.20	Void

#### 5.2.6.21	Nnef_Location service

##### 5.2.6.21.1	General

The service provides the capability to deliver UE location to AF.

##### 5.2.6.21.2	Nnef_Location_LocationUpdateNotify service operation

Service operation name: Nnef_Location_LocationUpdateNotify

Description: Provides UE location information to the consumer NF.

Inputs, Required: Identity of the AF, UE identifier (GPSI), event causing the location estimate (5GC-MO-LR), location estimate, age of location estimate, accuracy indication, LCS QoS class.

Inputs, Optional: Service identity.

Outputs, Required: Success/Failure indication.

Output (optional): Failure Cause (in the case of failure indication provided).

#### 5.2.6.22	Nnef_AMPolicyAuthorization Service

##### 5.2.6.22.1	General

Service description: This service is to authorise an AF request and trigger a respective Npcf_AMPolicyAuthorization request. This service allows the NF consumer to subscribe/unsubscribe the notification of events for reporting change of service coverage defined in clause 6.1.3.18 of TS 23.503 [20]. The description of the Throughput requirements, service coverage requirements, AF requested network slice replacement and policy duration are defined in clause 6.1.2.6.1 of TS 23.503 [20].

##### 5.2.6.22.2	Nnef_AMPolicyAuthorization_Create service operation

Service operation name: Nnef_AMPolicyAuthorization_Create

Description: Authorizes the request and triggers an Npcf_AMPolicyAuthorization_Create, potentially translating GPSI to SUPI.

Inputs, Required: GPSI.

Inputs, Optional: Throughput requirements, service coverage requirements, AF requested slice replacement requirement, policy duration, subscribed event(s).

The subscribed event includes Event ID as specified in Nnef_AMPolicyAuthorization_Notify service operation, Event Reporting Information defined in Table 4.15.1-1 (only the Event Reporting mode and the immediate reporting flag when applicable), Notification Target Address.

Outputs, Required: Success or Failure.

Outputs, Optional: Identification of the created application context, the inputs that can be accepted by the PCF.

##### 5.2.6.22.3	Nnef_AMPolicyAuthorization_Update service operation

Service operation name: Nnef_AMPolicyAuthorization_Update

Description: Provides updated information about an application context to the NEF and triggers an Npcf_AMPolicyAuthorization_Update.

Inputs, Required: Identification of the application context.

Inputs, Optional: Throughput requirements, service coverage requirements, AF requested slice replacement requirement, policy duration.

Outputs, Required: Success or Failure.

Outputs, Optional: The inputs that can be accepted by the PCF.

##### 5.2.6.22.4	Nnef_AMPolicyAuthorization_Delete service operation

Service operation name: Nnef_AMPolicyAuthorization_Delete

Description: Provides means for the NF Consumer to delete an application context by triggering an Npcf_AMPolicyAuthorization_Delete.

Inputs, Required: Identification of the application context.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

##### 5.2.6.22.5	Nnef_AMPolicyAuthorization_Notify service operation

Service operation name: Nnef_AMPolicyAuthorization_Notify

Description: provided by the NEF to notify NF consumers when the NEF receives from the PCF notifications about subscribed events.

Inputs, Required: Subscription Correlation ID, Event ID.

The event that can be subscribed is the event for reporting change of coverage defined in clause 6.1.3.18 of TS 23.503 [20].

Inputs, Optional: Event information as defined in clause 6.1.3.18 of TS 23.503 [20].

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.22.6	Nnef_AMPolicyAuthorization_Subscribe service operation

Service operation name: Nnef_AMPolicyAuthorization_Subscribe

Description: provided by the NEF for NF consumers to explicitly subscribe the notification of events, triggering an Npcf_AMPolicyAuthorization_Subscribe.

Inputs, Required: Event ID as specified in Nnef_AMPolicyAuthorization_Notify service operation, SUPI, Event Reporting Information defined in Table 4.15.1-1 (only the Event Reporting mode and the immediate reporting flag when applicable), Notification Target Address.

Inputs, Optional: Subscription Correlation ID (in the case of modification of the event subscription).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

##### 5.2.6.22.7	Nnef_AMPolicyAuthorization_Unsubscribe service operation

Service operation name: Nnef_AMPolicyAuthorization_Unsubscribe

Description: Enable NF consumers to explicitly unsubscribe the notification of events related to Nnef_AMPolicyAuthorization_Subscribe operation.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

#### 5.2.6.23	Nnef_AMInfluence service

##### 5.2.6.23.1	General

Service description: This service is to authorize the request and store in the UDR application data that can be retrieved by relevant PCFs in order to influence access and mobility management policies of one or multiple UEs. This service allows the NF consumer to subscribe/unsubscribe the notification of events about service coverage (defined in clause 6.1.3.18 of TS 23.503 [20]). The description of the Throughput requirements, service coverage requirements, slice replacement requirement and policy duration are defined in clause 6.1.2.6.1 of TS 23.503 [20].

##### 5.2.6.23.2	Nnef_AMInfluence_Create service operation

Service operation name: Nnef_AMInfluence_Create

Description: Authorize the request and store the AM influence data in the UDR, potentially translating GSPI to SUPI and External Group Identifier to Internal Group Identifier.

Inputs, Required: AF Transaction Id.

The AF Transaction Id refers to the request.

Inputs, Optional: List of (DNN, S-NSSAI)(s), Target (GPSI, or External Group Identifier(s), or any UE, or any inbound roaming UEs identified by their PLMN ID(s)), throughput requirements, service coverage requirements, AF requested network slice replacement, policy duration, List of External Application Identifiers, subscribed event(s).

The subscribed event(s) includes Event ID(s) as specified in Nnef_AMInfluence_Notify service operation, Event Reporting Information defined in Table 4.15.1-1 (only the Event Reporting mode and the immediate reporting flag when applicable), Notification Target Address.

When a list of External Application Identifiers is provided, the throughput requirement, service coverage requirements, AF requested network slice replacement, policy duration and subscribed event(s) are assigned the same value for all External Application Identifiers.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

NOTE:	"any UE" refers to the UEs within the PLMN of the NEF.

##### 5.2.6.23.3	Nnef_AMInfluence_Update service operation

Service operation name: Nnef_AMInfluence_Update

Description: Authorize the request and forward the request to update the AM influence data, potentially translating GSPI to SUPI and External Group Identifier to Internal Group Identifier.

Inputs, Required: AF Transaction Id.

The AF Transaction Id identifies the NF Service Consumer request to be updated.

Inputs, Optional: Same as in Nnef_AMInfluence_Create.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.23.4	Nnef_AMInfluence_Delete service operation

Service operation name: Nnef_AMInfluence_Delete

Description: Authorize the request and forward the request to delete the AM influence data.

Inputs, Required: AF Transaction Id.

The AF Transaction Id identifies the NF Service Consumer request for AM influence data to be deleted.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.23.6	Nnef_AMInfluence_Notify service operation

Service operation name: Nnef_AMInfluence_Notify

Description: Forward the notification of change of service coverage event report to the AF.

Inputs, Required: AF Transaction Id, Event ID.

The AF Transaction Id identifies the AF request for AM influence data that the event report is related to.

The event that can be subscribed is the event for reporting change of coverage defined in clause 6.1.3.18 of TS 23.503 [20]

Inputs, Optional: Event information (as defined in clause 6.1.3.18 of TS 23.503 [20]).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

#### 5.2.6.24	Void

#### 5.2.6.25	Nnef_TimeSynchronization service

##### 5.2.6.25.1	General

Service description: This service provides:

-	authorization of NF Service Consumer requests;

-	time synchronization service exposure as described in clause 5.27.1.8 of TS 23.501 [2] when the NF Service Consumer requests to create or update time synchronization configuration as well as to activate and deactivate the time synchronization service, for which the NEF uses service operations provided by TSCTSF as described in clause 5.2.27.

##### 5.2.6.25.2	Nnef_TimeSynchronization_ConfigCreate service operation

Service operation name: Nnef_TimeSynchronization_ConfigCreate

Description: The consumer requests to create a time synchronization configuration and activate the time synchronization service with the configuration, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.2.2).

Inputs, Required: As specified in clause 5.2.27.2.2.

Inputs, Optional: As specified in clause 5.2.27.2.2.

Outputs, Required: Operation execution result indication and in the case of successful operation, any outputs as specified in clause 5.2.27.2.2.

Outputs, Optional: As specified in clause 5.2.27.2.2.

##### 5.2.6.25.3	Nnef_TimeSynchronization_ConfigUpdate service operation

Service operation name: Nnef_TimeSynchronization_ConfigUpdate

Description: The consumer requests to update the time synchronization configuration, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.2.3).

Inputs, Required: As specified in clause 5.2.27.2.3.

Inputs, Optional: As specified in clause 5.2.27.2.3.

Outputs, Required: Operation execution result indication and in the case of successful operation, any outputs as specified in clause 5.2.27.2.3.

Outputs, Optional: As specified in clause 5.2.27.2.3.

##### 5.2.6.25.4	Nnef_TimeSynchronization_ConfigDelete service operation

Service operation name: Nnef_TimeSynchronization_ConfigDelete

Description: The consumer requests to delete the time synchronization configuration and deactivate the corresponding time synchronization service, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.2.4).

Inputs, Required: As specified in clause 5.2.27.2.4.

Inputs, Optional: As specified in clause 5.2.27.2.4.

Outputs, Required: Operation execution result indication and in the case of successful operation, any outputs as specified in clause 5.2.27.2.4.

Outputs, Optional: As specified in clause 5.2.27.2.4.

##### 5.2.6.25.5	Nnef_TimeSynchronization_ConfigUpdateNotify service operation

Service operation name: Nnef_TimeSynchronization_ConfigUpdateNotify

Description: Forward the notification for the time synchronization configuration or (g)PTP time distribution status change. When the NEF receives a notification of a change corresponding to a time synchronization configuration or (g)PTP time distribution status from the TSCTSF, it forwards the notification by invoking a Nnef_TimeSynchronization_ConfigUpdateNotify service operation to the NF consumer(s) that has subscribed for the event. The event parameters are described in Table 5.2.27.2.5-1.

Known NF Service Consumers: AF.

Inputs, Required: As specified in clause 5.2.27.2.5.

Inputs, Optional: As specified in clause 5.2.27.2.5.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.25.6	Nnef_TimeSynchronization_CapsSubscribe service operation

Service operation name: Nnef_TimeSynchronization_CapsSubscribe

Description: The AF requests the subscription to receive notifications about time synchronization capabilities for a list of UE(s) or a group of UEs or any UEs using DNN/S-NSSAI combination, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.2.6).

Event Filters are used to specify the conditions to match for notifying the event. If there are no conditions to match then the Event Filter is not provided. The Event Filters supported by the service are described in Table 5.2.27.2.6-1.

Inputs, Required: As specified in clause 5.2.27.2.6.

Inputs, Optional: As specified in clause 5.2.27.2.6.

Outputs, Required: Operation execution result indication and in the case of successful operation, any outputs as specified in clause 5.2.27.2.6.

Outputs, Optional: As specified in clause 5.2.27.2.6.

##### 5.2.6.25.7	Nnef_TimeSynchronization_CapsUnsubscribe service operation

Service operation name: Nnef_TimeSynchronization_CapsUnsubscribe

Description: The AF requests to unsubscribe from receiving notifications about time synchronization capabilities for a list of UE(s) or a group of UEs or any UE using a DNN/S-NSSAI combination, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.2.7).

Inputs, Required: As specified in clause 5.2.27.2.7.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication and in the case of successful operation, any outputs as specified in clause 5.2.27.2.7.

Outputs, Optional: As specified in clause 5.2.27.2.7.

##### 5.2.6.25.8	Nnef_TimeSynchronization_CapsNotify service operation

Service operation name: Nnef_TimeSynchronization_CapsNotify

Description: Forward the notification for the time synchronization configuration.

When the NEF receives a notification of a change corresponding to a Subscription from the TSCTSF, it forwards the notification by invoking a Nnef_TimeSynchronization_CapsNotify service operation to the NF consumer(s) that has subscribed for the event. The event parameters are described in Table 5.2.27.2.8-1.

Known NF Service Consumers: AF.

Inputs, Required: As specified in clause 5.2.27.2.8.

Inputs, Optional: As specified in clause 5.2.27.2.8.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.25.9	Void

##### 5.2.6.25.10	Void

##### 5.2.6.25.11	Void

##### 5.2.6.25.12	Void

#### 5.2.6.26	Nnef_EASDeployment service

##### 5.2.6.26.1	General

Service description: The EAS Deployment information management in the NEF provides functions for Node Level EAS Deployment information provision from AF and for subscribing and retrieving EAS Deployment Information in the UDR and providing EAS Deployment Information to the I-SMF for Local Offloading Management or to the SMF. For related procedures see clause 6.2.3.4 of TS 23.548 [74].

##### 5.2.6.26.2	Nnef_EASDeployment_Create service operation

Service operation name: Nnef_EASDeployment_Create

Description: The NF consumer requests to create EAS Deployment Information.

Inputs, Required: EAS Deployment Information.

Inputs, Optional: AF-service-identifier.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.26.3	Nnef_EASDeployment_Update service operation

Service operation name: Nnef_EASDeployment_Update

Description: The NF consumer requests to Update EAS Deployment Information.

Inputs, Required: EAS Deployment Information.

Inputs, Optional: AF-service-identifier.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.26.4	Nnef_EASDeployment_Delete service operation

Service operation name: Nnef_EASDeployment_Delete

Description: The NF consumer requests to delete EAS Deployment Information.

Inputs, Required: Either:

-	AF-identifier;

-	S-NSSAI and DNN;

-	EDI.

Inputs, Optional: Application Identifier, External Group Identifier.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.26.5	Void

##### 5.2.6.26.6	Nnef_EASDeployment_Subscribe service operation

Service operation name: Nnef_EASDeployment_Subscribe

Description: provided by the NEF for NF consumers to explicitly subscribe the notification of changes of EAS Deployment Information.

Inputs, Required: Event ID, Notification Target Address(+Notification Correlation ID).

Inputs, Optional: list of (DNN, S-NSSAI), Application Identifier, Internal Group Identifier.

Outputs, Required: Result Indication. When the subscription is accepted: Subscription Correlation ID (reference of the subscription).

Outputs, Optional: None.

##### 5.2.6.26.7	Nnef_EASDeployment_Unsubscribe service operation

Service operation name: Nnef_EASDeployment_Unsubscribe

Description: provided by the NEF for NF consumers to explicitly unsubscribe the notification of changes of EAS Deployment Information.

Inputs, Required: Subscription Correlation ID (reference of the subscription).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.26.8	Nnef_EASDeployment_Notify service operation

Service operation name: Nnef_EASDeployment_Notify

Description: Provides subscribed event information, e.g. updated EAS Deployment Information of Application Identifier, to the NF Consumer.

Inputs, Required: Event ID, Notification Correlation ID, EAS Deployment Information.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.5.2.6.2

#### 5.2.6.27	Nnef_UEId service

##### 5.2.6.27.1	General

See clause 4.15.10 for AF specific UE ID retrieval, clause 4.15.10A for MSISDN retrieval and clause 4.3.6.5 when V-NEF query UE information for HR-SBO PDU Session.

This service can also be used to create, update, delete and query Application Layer ID and GPSI mapping for Ranging/SL Positioning UE as defined in TS 23.586 [88].

##### 5.2.6.27.2	Nnef_UEId_Get service operation

Service operation name: Nnef_UEId_Get

Description: Get the UE identifier.

Inputs, Required: GPSI or UE address (i.e. IPv4/IPv6 address or MAC address).

Inputs, Optional: DNN, S-NSSAI, Port number (e.g. TCP or UDP port), IP domain, Application port ID, MTC Provider Information, AF Identifier.

Outputs, Required: Result, GPSI either as an AF specific UE Identifier represented in the form of an External Identifier, or in the form of a MSISDN or SUPI.

Outputs, Optional: HPLMN DNN, HPLMN S-NSSAI.

NOTE 1:	SUPI can only be exposed to roaming partners, i.e. V-NEF, to support HR-SBO Sessions as described in clause 4.3.6.5.

NOTE 2:	The AF specific UE Identifier in GPSI form of MSISDN can only be exposed to an AF when allowed and authorized by the operator as described in clause 4.15.10A.

Outputs, Optional: None.

##### 5.2.6.27.3	Nnef_UEId_UeIdMappingGet service operation

Service operation name: Nnef_UEId_UeIdMappingGet

Description: Get the Ranging/SL Positioning UE identifier.

Inputs, Required: GPSI or Application Layer ID.

Outputs, Required: Result, Application Layer ID or GPSI.

Outputs, Optional: None.

##### 5.2.6.27.4	Nnef_UEId_UeIdMappingCreate service operation

Service operation name: Nnef_UEId_UeIdMappingCreate

Description: Create the UE identifier mapping information (e.g. the mapping GPSI and Application Layer ID for Ranging/SL Positioning).

Inputs, Required: GPSI and Application Layer ID mapping information.

Inputs, Optional: None.

Outputs, Required: Result, Transaction Reference ID.

Outputs, Optional: None.

##### 5.2.6.27.5	Nnef_UEId_UeIdMappingUpdate service operation

Service operation name: Nnef_UEId_UeIdMappingUpdate

Description: Update the UE identifier mapping information (e.g. the mapping GPSI and Application Layer ID for Ranging/SL Positioning).

Inputs, Required: Transaction Reference ID, GPSI and Application Layer ID mapping information.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.27.6	Nnef_UEId_UeIdMappingDelete service operation

Service operation name: Nnef_UEId_UeIdMappingDelete

Description: Delete the UE identifier mapping information (e.g. the mapping GPSI and Application Layer ID for Ranging/SL Positioning).

Inputs, Required: Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.27.7	Nnef_UEId_VerificationGet service operation

Service operation name: Nnef_UEId_VerificationGet

Description: Verify if the GPSI in the form of MSISDN retrieved from UDM matches the MSISDN the AF provided in the request.

Inputs, Required: GPSI (in MSISDN form) and UE address (i.e. IPv4/IPv6 address or MAC address).

Inputs, Optional: DNN, S-NSSAI, Port number (e.g. TCP or UDP port), IP domain, Application port ID, MTC Provider Information, AF Identifier.

Outputs, Required: MSISDN verification result.

Outputs, Optional: None.

#### 5.2.6.28	Nnef_ASTI service

##### 5.2.6.28.1	General

Service description: This service provides:

-	authorization of NF Service Consumer requests;

-	time synchronization service exposure as described in clause 5.27.1.8 of TS 23.501 [2] when a NF Service Consumer requests to control the 5G access stratum time distribution configuration, for which the NEF uses service operations provided by TSCTSF as described in clause 5.2.27.

##### 5.2.6.28.2	Nnef_ASTI_Create service operation

Service operation name: Nnef_ASTI_Create

Description: The consumer requests to activate the 5G access stratum time distribution, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.4.2).

Inputs, Required: As specified in clause 5.2.27.4.2.

Inputs, Optional: As specified in clause 5.2.27.4.2.

Outputs, Required: Operation execution result indication and in the case of successful operation, any outputs as specified in clause 5.2.27.4.2.

Outputs, Optional: As specified in clause 5.2.27.4.2.

##### 5.2.6.28.3	Nnef_ASTI_Update service operation

Service operation name: Nnef_ASTI_Update

Description: The consumer requests to update the 5G access stratum time distribution configuration, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.4.3).

Inputs, Required: As specified in clause 5.2.27.4.3.

Inputs, Optional: As specified in clause 5.2.27.4.3.

Outputs, Required: Operation execution result indication and in the case of successful operation, any outputs as specified in clause 5.2.27.4.3.

Outputs, Optional: As specified in clause 5.2.27.4.3.

##### 5.2.6.28.4	Nnef_ASTI_Delete service operation

Service operation name: Nnef_ASTI_Delete

Description: The consumer requests to delete the 5G access stratum time distribution configuration and deactivate the corresponding 5G access stratum time distribution service, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.4.4).

Inputs, Required: As specified in clause 5.2.27.4.4.

Inputs, Optional: As specified in clause 5.2.27.4.4.

Outputs, Required: Operation execution result indication and in the case of successful operation, any outputs as specified in clause 5.2.27.4.4.

Outputs, Optional: As specified in clause 5.2.27.4.4.

##### 5.2.6.28.5	Nnef_ASTI_Get service operation

Service operation name: Nnef_ASTI_Get

Description: The consumer makes a query about the status of the access stratum time distribution, for which the NEF authorizes the request and invokes the corresponding service operation with TSCTSF (clause 5.2.27.4.5).

Inputs, Required: As specified in clause 5.2.27.4.5.

Inputs, Optional: As specified in clause 5.2.27.4.5.

Outputs, Required: Operation execution result indication and in the case of successful operation, any outputs as specified in clause 5.2.27.4.5.

Outputs, Optional: As specified in clause 5.2.27.4.5.

##### 5.2.6.28.6	Nnef_ASTI_UpdateNotify service operation

Service operation name: Nnef_ASTI_UpdateNotify

Description: Forward the notification for the 5G access stratum time distribution status change. When the NEF receives a notification of a change from the TSCTSF, it forwards the notification by invoking a Nnef_ASTI_UpdateNotify service operation to the NF consumer(s) that has subscribed for the event.

Inputs, Required: As specified in clause 5.2.27.4.6.

Inputs, Optional: As specified in clause 5.2.27.4.6.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.28.7	Void

##### 5.2.6.28.8	Void

#### 5.2.6.29	Nnef_SMService service

##### 5.2.6.29.1	General

Service description: This service can be used for SBI-based MO SM transmit through NEF for MSISDN-less MO SMS.

##### 5.2.6.29.2	Nnef_SMService_MoForwardSm service operation

Service operation name: Nnef_SMService_MoForwardSm.

Description: Transmit MO SMS message from consumer NF to NEF.

Inputs, Required: SMS payload, Application port ID, SUPI, destination SME address (long/short code of the AF).

Inputs, Optional: None.

Outputs, Required: SMS message transmission result.

Outputs, Optional: None.

#### 5.2.6.30	Nnef_PDTQPolicyNegotiation service service

##### 5.2.6.30.1	General

Service description: This service provides functionality for negotiation and management of planned data transfer with QoS requirements policies (PDTQ policies), which includes the following functionalities:

-	Determine PDTQ policies based on the request from AF; and

-	Update PDTQ policies based on the selection provided by AF.

##### 5.2.6.30.2	Nnef_PDTQPolicyNegotiation_Create service operation

Service operation name: Nnef_PDTQPolicyNegotiation_Create

Description: The consumer requests to create a PDTQ policy.

Inputs, Required: ASP Identifier, Number of UEs, list of Desired time windows, QoS reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: Network Area Information, Request for notification, Alternative Service Requirements.

Outputs, Required: PDTQ Reference ID, one or more PDTQ policies.

Output, Optional: None.

##### 5.2.6.30.3	Nnef_PDTQPolicyNegotiation_Update service operation

Service operation name: Nnef_PDTQPolicyNegotiation Update

Description: The consumer requests the selected PDTQ policy to be set.

Inputs, Required: PDTQ Reference ID.

Inputs, Optional: Stop notification, one PDTQ policy.

NOTE:	PDTQ policy is only indicated if the AF selects one of the PDTQ policies provided by the PCF.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.6.30.4	Nnef_PDTQPolicyNegotiation_Notify service operation

Service operation name: Nnef_PDTQPolicyNegotiation_Notify

Description: NEF sends the PDTQ policy warning notification to the NF consumer.

Inputs, Required: PDTQ reference ID, list of candidate PDTQ policies.

Inputs, Optional: None.

Outputs, Required: None.

#### 5.2.6.31	Void

#### 5.2.6.32	Nnef_MemberUESelectionAssistance service

##### 5.2.6.32.1	General

Service description: The service can be used by the consumer to subscribe or unsubscribe to the Member UE selection assistance information, or to receive notifications on the Member UE selection assistance information, from the NEF. See Table 4.15.13.2-1 for the Member UE filtering criteria.

##### 5.2.6.32.2	Nnef_MemberUESelectionAssistance_Subscribe service operation

Service operation name: Nnef_MemberUESelectionAssistance_Subscribe

Description: The NF consumer subscribes to receive the Member UE selection assistance information, or the subscription is updated if the same subscription is already existing in NEF.

Inputs, Required: At least one Member UE filtering criterion shown in Table 4.15.13.2-1.

Inputs, Conditional Required:

If no Subscription Correlation ID is provided in the subscription, AF Identifier, Notification Target Address (+ Notification Correlation ID), a list of target member UEs in the form of a list of GPSIs or a list of UE IP addresses) is required.

Inputs, Optional: Application ID, Subscription Correlation ID (in the case of update of the existing subscription), Expiry time, time window(s) for selecting the candidate UEs, specific parameters depending on the Member UE filtering criteria as shown in Table 4.15.13.2-1, Periodicity (the periodicity of member update), maximum number of candidate UEs.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

##### 5.2.6.32.3	Nnef_MemberUESelectionAssistance_Unsubscribe service operation

Service operation name: Nnef_MemberUESelectionAssistance_Unsubscribe

Description: The NF consumer deletes a subscription for Member UE selection assistance information that is already created in NEF.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

##### 5.2.6.32.4	Nnef_MemberUESelectionAssistance_Notify service operation

Service operation name: Nnef_MemberUESelectionAssistance_Notify

Description: NEF reports the Member UE selection assistance information to the consumer that has previously subscribed.

Inputs, Required: Notification Correlation Information.

Inputs, Conditional Required:

At least one of the following inputs is required:

-	One or more list(s) of candidate UEs in the form of a list of GPSIs or a list of UE IP addresses.

Inputs, Optional: Recommended time window for performing the application operation per list of candidate UEs as described in clause 4.15.13.1, specific values of the parameters that NEF gathered for the Member UE filtering criteria per candidate UE, a number for each filtering criterion that indicates the amount of UEs in the initial list which do not meet the criterion.

NOTE:	This number can be an indication for AF to revise one or multiple of the corresponding filtering criteria.

Outputs, Required: Operation execution result indication.

#### 5.2.6.33	Nnef_AF_request_for_QoS service

##### 5.2.6.33.1	General

See clause 4.15.6.14.

This service is also used to support subscription and notification of QoS Monitoring, as described in clause 5.33.3.2 of TS 23.501 [2].

##### 5.2.6.33.2	Nnef_AF_request_for_QoS_Create service operation

Service operation name: Nnef_AF_request_for_QoS Create

Description: The consumer requests the network to provide a specific QoS for a traffic flow for a UE or a group of UEs.

Inputs, Required: AF Identifier, Target UE identifier (GPSI or External Group Identifier), Flow description(s) or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available, traffic characteristics as described in clause 6.1.3.23 or 6.1.3.23a of TS 23.503 [20].

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.33.3	Nnef_AF_request_for_QoS_Notify service operation

Service operation name: Nnef_AF_request_for_QoS Notify

Description: NEF reports the QoS Flow level event(s) to the consumer.

Inputs, Required: Reports of the events as defined in clause 6.1.3.18 of TS 23.503 [20].

Inputs, Optional: When the event report is for QoS Monitoring, includes Packet delay for UL, DL, or round trip of the single UP path or two UP paths in the case of redundant transmission, as defined in clause 5.33.3.2 of TS 23.501 [2].

Outputs, Required: None.

Output (optional): None.

##### 5.2.6.33.4	Nnef_AF_request_for_QoS_Revoke service operation

Service operation name: Nnef_AF_request_for_QoS Revoke

Description: The consumer requests the network to revoke the Service Requirement(s) and/or additional Alternative Service Requirement(s) for a UE or a group of UEs.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.33.5	Nnef_AF_request_for_QoS_Update service operation

Service operation name: Nnef_AF_request_for_QoS Update

Description: The consumer requests the network to update the Service Requirement(s) and/or additional Alternative Service Requirement(s) for a UE or a group of UEs.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: Flow description information (as described in clause 6.1.3.6 of TS 23.503 [20]), QoS reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], traffic characteristics as described in clause 6.1.3.23 or 6.1.3.23a of TS 23.503 [20].

Outputs, Required: Result.

Output (optional): None.

#### 5.2.6.34	Nnef_DNAIMapping service

##### 5.2.6.34.1	General

The service is allowing AF and NWDAF to obtain DNAI.

##### 5.2.6.34.2	Nnef_DNAIMapping_Subscribe service operation

Service operation name: Nnef_DNAIMapping_Subscribe

Description: This service operation is used by the consumer to subscribe for DNAI information by providing specific information of AF and NWDAF.

Inputs, Required: EAS address information i.e. IP address/IP address range or FQDN.

Inputs, Optional: DNN, S-NSSAI, AF identifier, Event Reporting Information defined in Table 4.15.1-1.

NOTE:	If consumer wants, one-time reporting can be done by setting the Event Reporting Information as follows: event reporting mode to maximum number of reports, maximum number of reports=1 and Immediate reporting flag set.

Outputs, Required: When the subscription is accepted and not one-time reporting: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: DNAI, if available.

##### 5.2.6.34.3	Nnef_DNAIMapping_Unsubscribe service operation

Service operation name: Nnef_DNAIMapping_Unsubscribe

Description: The NF consumer terminates a subscription.

Inputs, Required: if not one-time reporting, Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication result.

Outputs, Optional: None.

##### 5.2.6.34.4	Nnef_DNAIMapping_UpdateNotify service operation

Service operation name: Nnef_DNAIMapping_UpdateNotify

Description: This service operation is used by the NEF to notify NF consumer about the update of DNAI - EAS address(es) mapping information.

Inputs, Required: One or more pairs of DNAI(s) and EAS address(es).

Inputs, Optional: Cause.

Outputs, Required: None.

Outputs, Optional: None.

#### 5.2.6.35	Nnef TrafficInfluenceData service

##### 5.2.6.35.1	General

This service is used by SMF in VPLMN to subscribe AF traffic influence request information from the AF via the NEF in VPLMN.

##### 5.2.6.35.2	Nnef_TrafficInfluenceData_Subscribe service operation

Service operation name: Nnef_TrafficInfluenceData_Subscribe

Description: The NF consumer subscribes for the notifications of AF traffic influence request information.

Inputs, Required: Data Set Identifier (i.e. Application data), Data Subset Identifier (i.e. AF traffic influence request information), Data Key(s), Notification Target Address, Event Reporting Information defined in Table 4.15.1-1.

Inputs, Optional: None.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

##### 5.2.6.35.3	Nnef_TrafficInfluenceData_Unsubscribe service operation

Service operation name: Nnef_TrafficInfluenceData_Unsubscribe

Description: Provided by the NEF for NF consumers to explicitly unsubscribe the notification of AF traffic influence request information.

Inputs, Required: Subscription Correlation ID (reference of the subscription).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.35.4	Nnef_TrafficInfluenceData_Notify service operation

Service operation name: Nnef_TrafficInfluenceData_Notify

Description: NEF notifies NF consumer(s) about AF traffic influence request information, when receives notification of AF traffic influence request information from UDR.

Inputs, Required: Notification Correlation ID, Data Set Identifier as defined in clause 5.2.12.2.1, Target of Event Reporting as defined in clause 5.2.12.2, AF traffic influence request information.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

#### 5.2.6.36	Nnef_UEAddress service

##### 5.2.6.36.1	General

See clauses 6.2.8.2.4.3 and 6.2.8.2.4.5 of TS 23.288 [50].

##### 5.2.6.36.2	Nnef_UEAddress_Get service operation

Service operation name: Nnef_UEAddress_Get

Description: Get the UE IP address (i.e. IPv4 address or IPv6 prefix).

Inputs, Required: GPSI, AF Identifier.

Inputs, Optional: None.

Outputs, Required: Result, UE address (i.e. IPv4 address or IPv6 prefix).

Outputs, Optional: None.

##### 5.2.6.36.3	Nnef_UEAddress_Subscribe service operation

Service operation name: Nnef_UEAddress_Subscribe

Description: The NF consumer subscribes for the immediate notifications of the NATed UE public IP address and Port number corresponding to UE private IP address. See clause 6.2.8.2.4.5 of TS 23.288 [50].

Inputs, Required: UE (private) IP address, AF Identifier, IP address of remote end, Notification Correlation ID.

Inputs, Optional: IP domain, Port number of remote end.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

##### 5.2.6.36.4	Nnef_UEAddress_UnSubscribe service operation

Service operation name: Nnef_UEAddress_UnSubscribe

Description: Provided by the NEF for NF consumers to explicitly unsubscribe the notification of NATed UE public IP address and Port number.

Inputs, Required: Subscription Correlation ID (reference of the subscription).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.36.5	Nnef_UEAddress_Notify service operation

Service operation name: Nnef_UEAddress_Notify

Description: NEF notifies NF consumer(s) about NATed UE public IP address and Port number, when receives notification from UPF. See clause 6.2.8.2.4.5 of TS 23.288 [50].

Inputs, Required: Notification Correlation ID, NATed UE public IP address and Port number.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

#### 5.2.6.37	Nnef_ECSAddress service

##### 5.2.6.37.1	General

Service description: The ECS Address service in the V-NEF provides functions for ECS Address Configuration Information provision for a group of UE or any UE from AF and for subscribing and retrieving ECS Address Configuration Information and providing ECS Address Configuration Information to the V-SMF to support. For related procedures see clauses 6.5.2.6.2 and 6.5.2.6.3 of TS 23.548 [74].

This procedure is defined only for the support of HR-SBO.

##### 5.2.6.37.2	Nnef_ECSAddress_Create service operation

Service operation name: Nnef_ECSAddress_Create

Description: The NF consumer requests to create ECS Address Configuration Information.

Inputs, Required: ECS Address Configuration Information.

Inputs, Optional: AF-service-identifier, AF Transaction Id, DNN, S-NSSAI.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.37.3	Nnef_ECSAddress_Update service operation

Service operation name: Nnef_ECSAddress_Update

Description: The NF consumer requests to update ECS Address Configuration Information.

Inputs, Required: ECS Address Configuration Information.

Inputs, Optional: AF-service-identifier, AF Transaction Id, DNN, S-NSSAI.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.37.4	Nnef_ECSAddress_Delete service operation

Service operation name: Nnef_ECSAddress_Delete

Description: The NF consumer requests to delete ECS Address Configuration Information.

Inputs, Required: Either:

-	AF-identifier;

-	S-NSSAI and DNN;

-	ECS Address Configuration Information;

-	AF Transaction Id.

The AF Transaction Id identifies the NF Service Consumer request for ECS Address Configuration Information to be deleted.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.37.5	Nnef_ECSAddress_Subscribe service operation

Service operation name: Nnef_ECSAddress_Subscribe

Description: Provided by the NEF for NF consumers to explicitly subscribe the notification of changes of ECS Address Configuration Information.

Inputs, Required: Event ID, Notification Target Address (+Notification Correlation ID).

Inputs, Optional: list of (DNN, S-NSSAI).

Outputs, Required: Result Indication. When the subscription is accepted: Subscription Correlation ID (reference of the subscription).

Outputs, Optional: None.

##### 5.2.6.37.6	Nnef_ECSAddress_Unsubscribe service operation

Service operation name: Nnef_ECSAddress_Unsubscribe

Description: Provided by the NEF for NF consumers to explicitly unsubscribe the notification of changes of ECS Address Configuration Information.

Inputs, Required: Subscription Correlation ID (reference of the subscription).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.37.7	Nnef_ECSAddress_Notify service operation

Service operation name: Nnef_ECSAddress_Notify

Description: Provides subscribed event information, e.g. updated ECS Address Configuration Information to the NF Consumer.

Inputs, Required: Event ID, Notification Correlation ID, ECS Address Configuration Information.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

#### 5.2.6.38	Void

#### 5.2.6.39	Nnef_ImsEventExposure service

##### 5.2.6.39.1	General

Service description: This service enables the NF to subscribe for a specific IMS event and be notified about subscribed IMS event.

##### 5.2.6.39.2	Nnef_ImsEventExposure (ImsEE) service

5.2.6.39.2.1	Nnef_ImsEE_Subscribe service operation

Service operation name: Nnef_ImsEE_Subscribe

Description: The NF consumer subscribes for a specific IMS event.

Inputs, Required: Event-ID(s) representing the event of interest, Event Reporting Information as defined in Table 4.15.1-1.

Inputs, Optional: List one or more IMS Public Subscriber identity, Event Filters, Notification Target address, Notification Correlation Information, Expiry time, Subscription Correlation ID (in the case of modification of the event subscription).

IMS Public subscriber identity entry is only present for subscriber specific IMS events.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can expire based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available (see clause 4.15.1).

5.2.6.39.2.2	Nnef_ImsEE_Unsubscribe service operation

Service operation name: Nnef_ImsEE_Unsubscribe

Description: The NF consumer unsubscribe the notification of Event.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

5.2.6.39.2.3	Nnef_ImsEE_Notify service operation

Service operation name: Nnef_ImsEE_Notify

Description: This service operation is used by the NEF to report to the NF consumer received information by the NEF in a Notify Request from the IMS AS that detected the event previously subscribed.

Inputs, Required: Event ID, time stamp.

Inputs, Optional: Notification Correlation Information, Event information (defined on a per Event ID basis).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

#### 5.2.6.40	Nnef_ImsSessionManagement service

##### 5.2.6.40.1	General

The service provides the capability to create and release an IMS session with standalone data channel media and modify data channel media in a specific IMS session.

NOTE:	In this Release only session management of IMS data channel is specified.

##### 5.2.6.40.2	Nnef_ImsSessionManagement_Create service operation

Service operation name: Nnef_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with standalone data channel media.

Inputs, Required: Calling ID, Called ID, Media information set.

Calling ID: It is the public identity of calling IMS Subscriber.

Called ID: It is the public identity of called IMS Subscriber.

Media information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Media Type: It indicates the media type used in the created session, and can be Data Channel.

-	Media Parameter: It indicates the related media parameters. For different media, it contains different parameters:

-	When the Media Type is Data Channel, it indicates the Data Channel Type (ADC or BDC).

-	When ADC is selected, it indicates the ADC type (P2A, P2P, P2A2P), App Binding Information, and if P2A ADC is selected, the party which this request is targeted,

-	If PA2 or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When BDC is selected, it indicates the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [55].

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.6.40.3	Nnef_ImsSessionManagement_Update service operation

Service operation name: Nnef_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the data channel media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the operated media is Data Channel.

-	Media Parameter: It indicates the related media parameters. For different media, it contains different parameters:

-	When the Media Type is Data Channel, it indicates the Data Channel Type (ADC or BDC).

-	When ADC is selected, it indicates the ADC type (P2A, P2P, P2A2P), App Binding Information, and if P2A ADC is selected, the party which this request is targeted

-	If P2A or P2A2P is selected, the MDC2 media endpoint address of the application layer is indicated.

-	When BDC is selected, it indicates the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: Calling ID, Called ID, Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [55].

Calling ID: It is the public identity of calling IMS Subscriber.

Called ID: It is the public identity of called IMS Subscriber.

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.6.40.4	Nnef_ImsSessionManagement_Delete service operation

Service operation name: Nnef_ImsSessionManagement_Delete

Description: This service operation enables the consumer NF to release the specific IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.6.40.5	Nnef_ImsSessionManagement_Notify service operation

Service operation name: Nnef_ImsSessionManagement_Notify

Description: Provides the subscribed event information to the NF Consumer.

Inputs, Required: Session ID, Event ID as described in Table AA.2.4.4.5-1 of TS 23.228 [55], Notification Correlation ID.

Inputs, Optional: Media resource information

Media resource information set includes entries corresponding to each media. Each entry in the set includes the Media correlation ID and DC Media Specification. The DC Media Specification shall exist only with media type "DC" and contains additional media resource information, e.g. remote media address for MDC2 interface.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### 5.2.6.41	Nnef_ImsParameterProvision service (ImsPP)

##### 5.2.6.41.1	General

This service is for allowing third party to provision information which can be used for an IMS user in the IMS (e.g. RCD server address, RCD URL or RCD information for a single IMPU or wildcard IMPU as described in clause AF.2.2.6 of TS 23.228 [55]).

##### 5.2.6.41.2	Nnef_ImsPP_Update service operation

Service operation name: Nnef_ImsPP_Update

Description: This service operation is used by the AF to update the IMS user related information (e.g. RCD server address, RCD URL or RCD information) for the target IMPU.

Inputs, Required: AF Identifier, Transaction Reference ID(s).

Inputs, Optional: Public User Identity, updated RCD properties as defined in clause AF.2.2.1 of TS 23.228 [55]).

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.41.3	Nnef_ImsPP_Create service operation

Service operation name: Nnef_ImsPP_Create

Description: This service operation is used by the AF to create the IMS user related information (e.g. RCD server address, RCD URL or RCD information) for the target IMPU.

Inputs, Required: AF Identifier, Transaction Reference ID(s).

Inputs, Optional: Public User Identity, RCD properties as defined in clause AF.2.2.1 of TS 23.228 [55]).

Outputs, Required: Result Indication.

Outputs, Optional: None.

##### 5.2.6.41.4	Nnef_ImsPP_Delete service operation

Service operation name: Nnef_ImsPP_Delete

Description: This service operation is used by the AF to delete the provisioned IMS user related information (e.g. RCD server address, RCD URL or RCD information) for the target IMPU.

Inputs, Required: AF Identifier, Transaction Reference ID(s).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

5.2.6A	Void

# *** End of Change ***