3GPP SA WG2 Meeting #166AH	S2-2500875

20-24 Jan 2025, E-meeting	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6022 | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NSAC for EHPLMN case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, 5GS_Ph1, 5GProtoc17 |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In non roaming case the I-SMF doesn’t interact with the NSACF. This is true for the case when the UE selects HPLMN. When the UE selects EHPLMN and if the operator configures different S-NSSAI for EHPLMN and HPLMN, the AMF provides the S-NSSAI of the EHPLMN and the mapped S-NSSAI of the HPLMN to the I-SMF, as described in clasue 5.15.5.3. In this case the I-SMF needs to interact with the NSACF for the maximum number of PDU session number for the S-NSSAI of the Serving PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification when the I-SMF can also interact with the NSACF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NSACF may count wrong number of PDU sessions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.15.11.2.1	Non- Hierarchical NSAC architecture

The NSACF keeps track of the current number of PDU Sessions per network slice so that it can ensure it does not exceed the maximum number of PDU session allowed to be served by the network slice. When an event related to a UE causes the current number of PDU sessions established within the network slice is to increase, the NSACF checks whether the maximum number of PDU sessions per network slice for that network slice has already been reached and if it has, the NSACF applies admission control policies.

The anchor SMF triggers a request to NSACF for maximum number of PDU sessions per network slice control during PDU session establishment/release procedures in clauses 4.3.2 and 4.3.4 of TS 23.502 [3].

The SMF provides the Access Type to the NSACF when triggering a request to increase or decrease the number of PDU Sessions. The NSACF takes Access Type into account for increasing and decreasing the current number of PDU Sessions depending on the applicability of the Access Type for the NSAC for maximum number of PDU Sessions for the S-NSSAI.

NOTE 1:	For MA PDU Session, the SMF provides the Access Type to NSACF when the user plane connection is about to be established or released in the corresponding access network. With this, the SMF provides one or two Access Types for the MA PDU Session in the same request message to the NSACF. The NSACF can reject a single or both Access Types depending on the applicability of the Access Type for the NSAC.

In non-roaming case, if the I-SMF receives only the S-NSSAI of the Serving PLMN, I-SMF does not interact with NSCAF. If the I-SMF receives both the S-NSSAI of the Serving PLMN and the mapped S-NSSAI of the HPLMN as described in clause 5.15.5.3, the I-SMF triggers the a NSAC request for maximum number of PDU Sessions for the S-NSSAI of the Serving PLMN, and the SMF trigger the NSAC for maximum number of PDU Sessions for the mapped S-NSSAI of the HPLMN.

End of CHANGES