SA WG2 Meeting #S2-166-Ad-Hoc-e	S2-2500885

20 - 24 January, 2025, Electronic meeting	(revision of S2-250xxxx)

Source:	ZTE

Title:	Discussion and proposal on the application level energy consumption information exposure

Document for:	Discussion

Agenda Item:	19.4.2

Work Item / Release:	EnergySys / Rel-19

Abstract of the contribution: This paper discusses the solution for per application energy consumption information collection and calculation and give the proposal.

# 1. Discussion

There is an EN in the clause 5.51.2.4 on per application energy consumption information exposure.

Editor's note:	Whether and how the per application energy consumption information can be exposed are FFS.

There is this EN because there is no solution for per application energy consumption information collection and calculation in the spec.

There are two potential solutions to resolve the issue.

Solution 1: On top of per UE per Qos flow energy consumption information collection and calculation

The basic idea of this solution is that the energy consumption of an application is the sum of the energy consumption of all UEs using the application.

For the energy consumption generated by each UE using the application, it can use the existing per UE per Qos flow energy consumption information collection and calculation defined in the 23.501 clause 5.51.2.

The detail steps are:

- The EIF receives the subscribe for the energy consumption of an application (identified by IP 5 tuples list)

- The EIF subscribe with all the SMF for energy information collection with: target = any UE, DNN/S-NSSAI and Flow description(s).

- For all the serving PDU session using the DNN/S-NSSAI, the SMF perform the procedure in the clause 4.29.2 (Energy Consumption information collection) of 23.502.

- The EIF aggregates the report from SMF for all the UE using the application, queries the OAM and performs calculations.

The cons is, it creates the huge amount of signalling.

Solution 2: UPF data collection and slice-based calculation.

The data volume collection is re-using the option 1 in the clause 4.15.4.5.3 (Information flow for UPF event exposure service for any UE)

#### 4.15.4.5	Exposure of Events from UPF for UPF Data Collection

##### 4.15.4.5.1	General

…………..

The UPF event exposure events are described in clause 5.2.26.2. In this Release of the specification, the following events are used for UPF Data collection:

-	QoS Monitoring. This event provides QoS Flow performance information.

-	User DataUsage Measures. This event provides information of user data usage of the User PDU Session.

##### 4.15.4.5.3	Information flow for UPF event exposure service for any UE

After the EIF collect total data volume of the application. It retrieve the OAM for the slice energy consumption information (data volume and energy consumption, see 28.554 clause 6.7.3.3)

Then the EIF can calculate the energy consumption for the application using a similar method to the UE level energy consumption calculation.

EApplication=EsliceDVApplicationDVslice

Considering the solution complexity, this paper proposes to use the solution 2.

# 2. Proposal

It is proposed to adopt the solution 2 in the CR S2-2500886 and S2-2500887.