3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500900

# Online, Jan 20th – 24th, 2025		(revision of S2-2411920)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5848 | rev | 1 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provision of the MMSID for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. As agreements in the Reply from RAN2 LS R2-2409272 and SA LS SP-241921, the multi-modality information (e.g., MMSID), can be used for joint admission control and QoS flow to DRB mapping. This paper intends to support the provisioning of the MMISD to the NG-RAN. 2.However, the MMSID allocated by the AF is not unique for different application. For the uniquenes, it is proposed to leave this to AF implementation to assign unique MMSID value. 3.Besides, as indicated in the Reply LS S4-242223, only the application can decide which particular data flows can be rejected in case network cannot fulfil all data flows for the multi-modal service. Therefore, it is proposed to indicate the essential data flows by providing an essential flag in the AF request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provision of the MMSID for essential QoS flows related to a multi-modal service to NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The joint admission control and QoS flow to DRB mapping considering the multi-modality information is missing due to lack of the awareness of the MMSID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 1436 TS 23.502 CR 5302 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.37.2	Policy control enhancements to support multi-modal services

A multi-modal service is a communication service that consists of several data flows that relate to each other and that are subject to application coordination. The data flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources(e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs).

For the single UE case, it is expected that those data flows are closely related and require strong application coordination for the proper execution of the multi-modal application and therefore, all those data flows are transmitted in a single PDU session.

The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each data flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements:

-	The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application.

-	The AF may provide QoS monitoring requirements for data flows associated to a multi-modal service to the PCF . The PCF generates the authorized QoS Monitoring policy for each data flow.

NOTE 1:	The uniqueness of Multi-modal Service ID values among different AFs can be achieved based on the local configuration in the AFs, e.g. different ranges of Multi-modal Service ID values for different AFs.

NOTE 2:	In order to start the QoS monitoring for the data flows associated to a multi-modal service within a certain period of time, the PCF needs to receive the QoS monitoring requirements for those data flows from AF within a single request or, in case of multiple requests, within a short period of time.

The Nnef_AFsessionWithQoS service also allows the AF to provide an essential flag for one or more data flows within the multi-modal service:

-	The essential flag is used to indicate the data flows are essential among the data flows in the multi-modal service. In case the network cannot fulfil the requirements of all data flows, these essential data flows should be at least guaranteed.

The PCF may include Multi-modal Service ID into the PCC rules corresponding to the essential data flows related to a multi-modal service. Based on the PCC rule, the SMF provides the Multi-modal Service ID to the NG-RAN for joint admission control and QoS flow to DRB mapping.

NOTE 3:	The details on joint admission control considering the Multi-modal Service ID are left to RAN3 progress.

NOTE 4:	The details on QoS Flow to DRB mapping considering the Multi-modal Service ID depend on RAN implementation.

In addition to the features that are provided for the case that the data flows are associated with a single UE, the following features are provided for the case where the data flows are associated with more than one UE:

-	The same DNN/S-NSSAI combination for the multi-modal service should be selected by each of the involved UEs. The URSP Rule evaluation framework is used to ensure that the same DNN/S-NSSAI is selected.

-	The AF should use the same Multi-modal Service ID in the interactions with the PCF(s) for all the involved UEs that relate to a multi-modal service. The PCF may take this information into account (e.g. to apply a specific QoS policy) when processing each AF request independently. The data flows contribute to the service experience, but are still valid stand-alone, as they are transmitted over separate PDU Sessions to/from the involved UEs.

-	If multiple PCFs are involved, the PCFs take policy decisions according to the input provided by the AF. There is no support for policy coordination among the multiple PCFs in this Release of the specification. Policy decisions are taken by each PCF separately on a per PDU Session basis.

# * * * * End of changes * * * *