3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500903

# Online, 20th – 24th Jan, 2025	                          	                       (revision of S2-24XXXXX)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6034 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for support of mixed traffic handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The agreed CR (S2-2412984) in last SA2#166 meeting specified the support of mixed traffic handling. New term [(S)RTP Multiplexed Media Information] is introduced to identify the multiplexed media flows. However, there are still several issues to be resolved. - Based on the LS from SA4 in S2-2412586, the RTP-PT, RTP-SSRC and MID can be used for the multiplexed traffic differentiation. It is necessary to clarify relationship between the RTP-PT and RTCP packet type. - When the UE has no support of the (S)RTP Multiplexed Media Information, how the AF behaves also needs to be clarified. - Clarify the assumption on the UPF capability for supporting (S)RTP Multiplexed Media Information |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify relationship between the RTP-PT and RTCP packet type. - Clarify the AF behaviours in case the UE has no support of the (S)RTP Multiplexed Media Information. - Add clarification on UPF capability for supporting (S)RTP Multiplexed Media Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description for the multiplexed media flows identification may be misleading and not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.6.2, 5.37.11, 6.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.7.6.2	IP Packet Filter Set

For IP PDU Session Type, the Packet Filter Set shall support Packet Filters based on at least any combination of:

-	Source/destination IP address or IPv6 prefix.

-	Source / destination port number.

-	Protocol ID of the protocol above IP/Next header type.

-	Type of Service (TOS) (IPv4) / Traffic class (IPv6) and Mask.

-	Flow Label (IPv6).

-	Security parameter index.

-	Packet Filter direction.

NOTE 1:	A value left unspecified in a Packet Filter matches any value of the corresponding information in a packet.

NOTE 2:	An IP address or Prefix can be combined with a prefix mask.

NOTE 3:	Port numbers can be specified as port ranges.

NOTE 4:	Type of Service (IPv4)/Traffic class (IPv6) can be used to define packet filters for DSCP and ECN as described in RFC 3168 [193].

-	(S)RTP Multiplexed Media Identification Information including a combination of at least one of the following:

-	Synchronization Source (SSRC), as defined by IETF RFC 3550 [185];

-	Payload Type (PT), as defined by IETF RFC 3550 [185];

-	Media Identification (MID);

-	RTCP packet type.

NOTE 5:	The RTCP packet type and RTP Payload Type (PT) shares the same 7-bit field of the second Byte of the UDP payload.

# * * * * Next change * * * *

### 5.37.11	Traffic identification and differentiated QoS for multiplexed media flows

XR and interactive media services can send data traffic of different media components with different QoS requirements. Several media flows could be multiplexed on the same end-to-end transport layer connection. In order to uniquely identify each media flow, the IP Packet Filter can further include multiplexed media specific (S)RTP Multiplexed Media Identification Information to differentiate the media flow among multiple media flows that share the same transport layer related elements in the packet filter (see clause 5.7.6.2).

The AF may provide individual QoS requirements for each individual RTP media flow within the multiplexed media flows and for the (S)RTCP traffic that controls the (S)RTP transmission, by including the (S)RTP Multiplexed Media Identification Information associated with the individual QoS requirements within the flow description in the "AF session with the required QoS" procedure specified in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3]. The policy control related to multiplexed media flows identification is specified in clause 6.1.3.22 of TS 23.503 [45].

If a UE indicates support of (S)RTP Multiplexed Media Identification Information in the IP packet filter, it shall indicate this to the SMF at PDU Session establishment and the SMF provides this UE capability information to the PCF. The PCF considers such UE capability information when providing the PCC rules to the SMF. In case of no support of (S)RTP Multiplexed Media Information in the IP Packet Filter Set by the UE, the PCF may notify AF to re-provision the QoS request. Alternatively, the AF may provide both the QoS requirements for the single UDP/IP traffic flow and the QoS requirements for the media flows, and the PCF determines the PCC rules accordingly based on the support of (S)RTP Multiplexed Media Information in the IP Packet Filter Set by the UE as described in clause 6.1.3.27.X of TS 23.503 [45].

If (S)RTP Multiplexed Media Identification Information is received in the PCC rule, the SMF provides (S)RTP Multiplexed Media Identification Information to the UPF in the corresponding PDR(s) and also to the UE in the QoS Rule if the UE has indicated that it supports (S)RTP Multiplexed Media Identification Information in IP packet filter. If the UE has not indicated so, the SMF shall not provide the (S)RTP Multiplexed Media Identification information in the QoS Rule to the UE. Reflective QoS shall not be applied to SDFs which are identified with (S)RTP Multiplexed Media Identification Information defined in clause 5.7.6.2.

NOTE:	It is assumed that the PSA UPF supports the IP Packet Filter with (S)RTP Multiplexed Media Identification Information IP Packet Filter Set extensions for multiplexed media.

# * * * * Next change * * * *

#### 6.3.3.3	Selection of an UPF for a particular PDU Session

The following parameter(s) and information may be considered by the SMF for UPF selection and re-selection:

-	UPF's dynamic load.

-	Analytics (i.e. statistics or predictions) for UPF load, Service Experience analytics and/or DN Performance analytics per UP path (including UPF and/or DNAI and/or AS instance) and UE related analytics (UE mobility, UE communication and expected UE behavioural parameters) as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

-	UPF's relative static capacity among UPFs supporting the same DNN.

-	UPF location available at the SMF.

-	UE location information.

-	Capability of the UPF and the functionality required for the particular UE session: An appropriate UPF can be selected by matching the functionality and features required for an UE.

-	Data Network Name (DNN).

-	PDU Session Type (i.e. IPv4, IPv6, IPv4v6, Ethernet Type or Unstructured Type) and if applicable, the static IP address/prefix.

-	SSC mode selected for the PDU Session.

-	UE subscription profile in UDM.

-	DNAI as included in the PCC Rules and described in clause 5.6.7.

-	Local operator policies.

-	S-NSSAI.

-	Access technology being used by the UE.

-	Information related to user plane topology and user plane terminations, that may be deduced from:

-	5G-AN-provided identities (e.g. CellID, TAI), available UPF(s) and DNAI(s);

-	Identifiers (i.e. a FQDN and/or IP address(es)) of N3 terminations provided by a W-AGF or a TNGF or a TWIF;

NOTE 1:	A W-AGF or a TNGF may provide Identifiers of its N3 terminations when forwarding over N2 uplink NAS signalling to the 5GC. The AMF may relay this information to the SMF, as part of session management signalling for a new PDU Session.

-	Information regarding the user plane interfaces of UPF(s). This information may be acquired by the SMF using N4;

-	Information regarding the N3 User Plane termination(s) of the AN serving the UE. This may be deduced from 5G-AN-provided identities (e.g. CellID, TAI);

-	Information regarding the N9 User Plane termination(s) of UPF(s) if needed;

-	Information regarding the User plane termination(s) corresponding to DNAI(s).

-	RSN, support for redundant GTP-U path or support for redundant transport path in the transport layer (as in clause 5.33.2) when redundant UP handling is applicable.

-	Information regarding the ATSSS Steering Capability of the UE session (e.g. any combination of ATSSS-LL capability, MPTCP capability, MPQUIC capability) and information on the UPF support of RTT measurements without PMF.

-	Support for UPF allocation of IP address/prefix.

-	Support of the IPUPS functionality, specified in clause 5.8.2.14.

-	Support for High latency communication (see clause 5.31.8).

-	Support for functionality associated with high data rate low latency services, eXtended Reality (XR) and interactive media services, specified in clause 5.37 (for example, ECN marking for L4S, specified in clause 5.37.3, PDU Set Marking, specified in clause 5.37.5, UE power saving management, specified in clause 5.37.8, time to next data burst marking, specified in clause 5.37.10.2, PDU Set Importance based transport level packet marking, specified in clause 5.8.2.7, Traffic identification and differentiated QoS for multiplexed media flows as specified in clause 5.37.11).

-	Support for the On-path N6 signalling methods, e.g. MoQ relay functionality and/or CONNECT-UDP HTTP client functionality and/or UDP option functionality (as described in clause 5.37.9)

-	User Plane Latency Requirements within AF request (see clause 5.6.7.1 and clause 6.3.6 of TS 23.548 [130]).

-	List of supported Event ID(s) for exposure of UPF-related information via service based interface (see clause 7.2.29 and clause 5.2.26.2 of TS 23.502 [3]).

-	Information regarding required and/or preferred UPF functionalities (see clause 5.2.3.3.1 of TS 23.502 [3]). If received from UDM, the SMF selects a PSA UPF supporting the required UPF functionalities and the best set of preferred functionalities based on their priorities if received.

-	Support of NAT information exposure functionality.

-	Support of IP or MAC filter-based packet detection functionality.

-	Supported operator configurable UPF capabilities as described in clause 5.8.2.21.

-	N6 delay between the candidate UPF(s) and measurement endpoint in the DN as specified in clause 5.8.2.23.

NOTE 2:	How the SMF determines information about the user plane network topology from information listed above and what information is considered by the SMF, is based on operator configuration.

NOTE 3:	In this release the SMF uses no additional parameters for UPF selection for a PDU Session serving TSC or Deterministic Networking. If a PDU Session needs to connect to a specific UPF hosting a specific TSN 5GS bridge or 5GS router, this can be achieved e.g. by using a dedicated DNN/S-NSSAI combination.

If there is an existing PDU Session and the SMF receives another PDU Session request to the same DNN and S-NSSAI and if the SMF determines that interworking with EPC is supported for this PDU Session (as specified in clause 4.11.5 of TS 23.502 [3]), the SMF should select the same UPF if it supports all capabilities required for the new PDU Session. Otherwise, if the SMF determines that interworking with EPC is not supported for the new PDU Session or the UPF of the existing PDU Session does not support all capabilities required for the new PDU Session, a different UPF may be selected according to operator policy.

For the same DNN and S-NSSAI if different UPFs are selected at 5GC, when the UE is moved to EPC network, there is no requirement to enforce APN-AMBR. Whether and how to apply APN-AMBR for the PDN Connection associated with this DNN/APN is implementation dependent, e.g. possibly only AMBR enforcement per PDU Session applies.

# * * * * End of changes * * * *