3GPP TSG-WG SA2 Meeting #166Ad Hoc-e	S2-2500913

# Online, 20th Jan – 24th Jan, 2025		(revision of S2-2410333, 8834, 8474)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5588 | rev | 3 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MDBV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MDBV (for delay-critical GBR resource type) was introduced with the assumption that the periodicity of data burst arrivals is considerably larger than the PDB. With XRM service, the typical periodicity became much shorter (i.e. in the range of 8ms to 33ms for video services with 120 to 30 frames per second). It is therefore likely, that the periodicity is in the same range as the PDB. As an example, consider a service with a PDB of 10 ms and a periodicity of 8 ms. In this case with a PDB larger than the periodicity of data burst arrivals, there will be typically more than a single data burst to be transferred by the RAN within the 5G-AN PDB. A RAN implementation following the standardized requirements will however have resources available for only data burst. It was discussed in XRM that the AF can take 5G-AN PDB into account to generate Burst Size but the CN-PDB is not known to the AF and it would be too complicated to transfer the CN-PDB to the AF (due to its dependency on the 5QI and the deployment specifics). Furthermore, for some AFs it may also not be possible to change the logic of how to derive the burst size (e.g. in the TSN case, how to generate burst size by TSN AF is standardized outside of 3GPP). It is therefore proposed to describe this situation in a note in order to highlight the consequences. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a note describing the problem of supporting a QoS Flow with a periodicity of data burst arrivals that is smaller than the AN-PDB. In addition, some statements are reworded to better describe the usage of the MDBV by application and RAN as well as to explain the term data burst in this clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scenario is not described in which the RAN fulfills the standardized requirements for PDB and MDBV but data burst arrive too quickly and thus, the accepted QoS requirements of the application cannot be fulfilled (even though the application traffic is in conformance with the GFBR). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.7.3.7	Maximum Data Burst Volume

Each GBR QoS Flow with Delay-critical resource type shall be associated with a Maximum Data Burst Volume (MDBV) which denotes the size of the largest expected data burst, i.e. the amount of data for the largest expected set of multiple PDUs that are generated and sent by the application in a short period of time.

Every standardized 5QI (of Delay-critical GBR resource type) is associated with a default value for the MDBV (specified in QoS characteristics Table 5.7.4.1). The MDBV may also be signalled together with a standardized 5QI to the (R)AN and if it is received, it shall be used instead of the default value.

The MDBV may also be signalled together with a pre-configured 5QI to the (R)AN and if it is received, it shall be used instead of the pre-configured value.

The (R)AN applies the MDBV to derive the largest amount of data that the (R)AN is required to serve within a period of 5G-AN PDB (see clause 5.7.3.4).

NOTE:	If Perodicity is signalled to (R)AN (i.e. in TSCAI as described in clause 5.27) and the Periodicity is smaller than 5G-AN PDB, more than one data burst can arrive within a period of 5G-AN PDB while the (R)AN may not be able to transfer more than one data burst for this QoS Flow. In order to avoid this, the (R)AN could derive the largest amount of data that the (R)AN is required to serve within a period of 5G-AN PDB under consideration of the maximum number of data bursts that could arrive within a period of 5G-AN PDB according to the indicated periodicity.

# * * * * End of changes * * * *