3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500921

# 20 – 24, January, 2025, E-meeting

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0181 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unsubscribe service operation with UAS NF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.256 it is mentioed that how NF consumer like AMF and SMF need to unsubscribe with UAS NF. In clause 5.2.2.2, it reads: If the UE is deregistered as per clause 4.2.2.3 of TS 23.502 [3], then the AMF shall unsubscribe to UAS NF and then UAS NF/NEF may clear the UUAA-MM context and update USS. And in clause 5.2.3.2: If the PDU session is released as per clause 4.3.4 of TS 23.502 [3] then the SMF shall unsubscribe to UAS NF/NEF and then UAS NF/NEF may clear the UUAA-SM context and update USS. CT4 has sent one LS (C4-245436) by stating that the Unsubscribe service operation for Nnef_Authentication service is not defined in neither TS 23.256 nor TS 29.256, and the UUAA context in the UAS NF/NEF may not be properly cleared. Hence unsubscribe service operation is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Unsubscribe operation is defined with UAS NF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of UE deregistration or PDU session release for UAV service, the UUAA-MM or UUAA-SM content at UAS NF will not be cleared. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.1, 4.4.1.1.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

##### 4.4.1.1.1	General

In addition to those defined in TS 23.501 [2] clause 7.2.8 and TS 23.502 [3] clause 5.2.6, the following table illustrates additional NEF services to support UAS.

Table 4.4.2.1.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_Authentication | AuthenticateAuthorize | Request/Response | AMF, SMF |
|  | Unsubscribe | Subscribe/Notify | AMF, SMF |
|  | Notification | Subscribe/Notify | AMF, SMF |

##### 4.4.1.1.2.3	Nnef_Authentication_Unsubscribe service operation

Service operation name: Nnef_Authentication_Unsubscribe

Description: The NF consumer unsubscribes to the existing implicit subscription and based on this unsubscription request, UAS NF clears the UUAA-MM or UUAA-SM context..

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

4.4.1.1.2.4	Nnef_Authentication_Notification service operation

Service operation name: Nnef_Authentication_Notification

Description: Re-authenticate, update authorization data or revoke the UUAA authorization of a UAV.

NOTE:	This notification corresponds to an implicit subscription by Nnef_Authentication_AuthenticateAuthorize service operation.

Input, Required: Notification Correlation Information, Service Level Device Identity, 3GPP UAV ID, Notify reason (revoke, re-authentication, or authorization data update).

Input, Conditional Required: Authorization Data container (if the Notify reason is authorization data update).

Input, Optional: None.

Output, Required: Acknowledge indication.

Output, Optional: None.

* * * End of Changes * * * *