3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500926

Online, 20th Jan – 25th Jan, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.xyz: Ambient IoT Device Identifier

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: The Ambient IoT Device Identifier is introduced to the specification.

1. Introduction/Discussion

Based on the conclusion in TR 23.700-13, the Ambient IoT Device Identifier is defined. The components in the Ambient IoT Device Identifier is renamed in order to make it clear the usage of each component of Ambient IoT Device Identifier.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts) * * * *

5.7	Identifier

5.7.1	Ambient IoT Device Permanent Identifier

A globally unique Ambient IoT Device Permanent Identifier shall be stored in each Ambient IoT Device.

The Ambient IoT Device Permanent Identifier includes following component:

-	The ID type, including

-	information indicating whether the network identifier is included or not.

-	information indicating whether a 3rd party identifier is included or not.

-	information indicating whether the ID includes EPC or the other format.

-	The domain information, including one or both of the following:

-	A network identifier as specified in TS 23.003 [X]

-	A 3rd party identifier used to identify a 3rd party

-	The information (e.g. EPC or others) used to distinguish different Ambient IoT Devices within the scope identified by the network identifier and/or 3rd party identifier.

NOTE 1:	The format of the information used to distinguish different Ambient IoT Devices for the operator assigned and 3rd party assigned ID is based on implementation.

NOTE 2:	The coding for the above information is left to stage 3.

# * * * * End of changes * * * *