3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500929

Online, 20th – 25th Jan, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Procedures for AIOTF and AIoT Reader Communication

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: The call flow steps for AIOTF and AIoT Reader Communication in the Indirect architecture option will be specified in this contribution.

1. Introduction/Discussion

The indirect topology 1 architecture option concluded in TR 23.700-13 uses an AMF to route messages between and AIOTF and AIoT RAN node. The AIoT RAN nodes does not distinguish whether it is using the direct or indirect architecture option, so the additional steps need to be described for when the AMF is used.

This approach to define the additional steps in a dedicated clause will reduce the definition overhead in other procedure flows or duplication of flows to account for the different routing options between direct and indirect option and allow for potential future expansion to include other reader types or routing.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts) ****

6.3	Procedures for AIOTF and AIoT Reader Communication

6.3.1	General

Ambient IoT Supports different paths for communication between AIoT RAN and AIOTF, as defined in clause 4.2.2.1. The procedures define in clauses 6.2 uses these procedures for communication between the AIOTF and the AIoT Reader.

The handling of the different communication paths is described in the flowing clauses.

6.3.2	Procedures for Direct interface between AIOTF and A-IoT RAN

When the AIOTF and AIoT RAN use the Nx interface then the AIOTF uses NGAP to communicate with AIoT RAN as defined in TS 38.xxx [x].

6.3.3	Procedures for Indirect interface between AIOTF and A-IoT RAN

An AIOTF and AIoT RAN can use an indirect interface via an AMF, as described in clause 4.2.2.4, with the AIOTF using services provided by an AMF to route information to and from AIoT RAN. The additional steps used for Indirect interface between AIOTF and A-IoT RAN are shown in Figure 6.3.3-1.

Figure 6.3.3-1: AIOTF and A-IoT RAN indirect communication via an AMF

1.	When an AIOTF sends information to AIOT RAN via AMF, it includes information and parameters in an AIoT Container for the required operation. The information included in the AIoT Container is the same as the one sent from AIOTF to AIoT RAN when an AIOTF and AIoT RAN communicate directly. The AIOTF sends Namf_XXXX message to the AMF which has an N2 interface towards AIOT RAN, including AIOT RAN ID to identify the target AIoT RAN node and the AIoT Container.

2.	The AMF sends an NGAP message to the target AIoT RAN, and includes the AIOTF ID and the AIoT Container from the AIOTF.

3.	When receiving an AIOT NGAP message, the AIoT RAN shall store the mapping for the requested operation, AIOTF ID and the AMF which sent the NGAP message.

4.	When AIoT RAN sends responses an to AIOTF via an AMF the AIOT RAN includes an AIoT Container with request specific responses, and the AIOTF ID received in the request in step 2. AIOT RAN determines the AIOTF ID and AMF based on the mapping stored in step 3.

5.	AMF sends the Namf_XXXX message to the AIOTF identified by the AIOTF ID received in step 4, including the AIOT RAN ID and the AIoT Container received from the AIoT RAN.

NOTE:	The details of message type and IE used between AIOTF and AIOT RAN and between AIOT RAN and AMF depends on RAN WG3 decision.

# * * * * End of changes * * * *