3GPP TSG-WG 3GPPSA2#166-Ad Hoc-e	S2-2500931

Online, 20th JAN – 24th JAN, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Network Functions Services

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: add the services for AMF, NEF, AIOTF, ADM (new data storage NF) in respect to Ambient IoT.

1. Discussion

Follow the conclusion in clause 8.3 of TR 23700-13, this proposal introduces the support of Ambient IoT services for AMF, NEF, AIOTF, ADM (new data storage NF).

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts) * * * *

7.1	General

In the context of Ambient IoT specification, an NF service is offering a capability to authorised upstream and downstream NF consumers, thus to enable the network to offer the end-to-end Ambient IoT services from system perspective.

The following clauses provide for each involved NF the NF services it exposes through its service-based interfaces.

7.2	AIOTF service

The following table shows the Naiotf_AIoT Service and Service Operations.

Table 7.2-1: NF services provided by AIOTF

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF |
|  | Read | Request/Response | NEF |
|  | Write | Request/Response | NEF |
|  | PermanentDisable | Request/Response | NEF |
|  | Notify | Subscribe/Notify | NEF |

7.2.1	Naiotf_AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory

Description: the NF consumer requests to perform an inventory device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.2	Naiotf_AIoT_Read service operation

Service operation name: Naiotf_AIoT_Read

Description: the NF consumer requests to perform a read device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Read command specific parameters (including response application data size).

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.3	Naiotf_AIoT_Write service operation

Service operation name: Naiotf_AIoT_Write

Description: the NF consumer requests to perform a write device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Write command specific parameters:

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.4	Naiotf_AIoT_PermanentDisable service operation

Service operation name: Naiotf_AIoT_PermanentDisable

Description: the NF consumer requests to perform a permanent disable device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Permanent disable command specific parameters:

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.5	Naiotf_AIoT_Notify service operation

Service operation name: Naiotf_AmbientIoT_Notify

Description: This service operation allows the consumer to receive the status or results of the requested service operation. If the NF consumer invokes the Naiotf_AIoT_Inventory, or Naiotf_AIoT_Read or Naiotf_AIoT_Write or Naiotf_AIoT_PermanentDisable service operation, the NF consumer implicitly subscribes to receive the status or results of the requested service operation

Inputs, Required: Correlation ID, Device Operation Execution Result indication (Success or Failure), List of Ambient IoT Device ID, Failure Cause in case of Failure. In case of Naiotf_AIoT_Read, data to read for each target Ambient IoT Device.

Outputs, Required: Operation execution result indication.

7.3	NEF services

The following table shows the Nnef_AIoT Service and Service Operations, in addition to the NEF services offered as defined in TS 23.502 clause 5.2.6.

Table 7.3-1: NF services provided by NEF

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Read | Request/Response | AF |
|  | Write | Request/Response | AF |
|  | PermanentDisable | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

7.3.1	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_AIoT_Inventory

Description: the NF consumer requests to perform an inventory device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Transaction ID.

7.3.2	Nnef_AIoT_Read service operation

Service operation name: Nnef_AIoT_Read

Description: the NF consumer requests to perform a read device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Read command specific parameters (including response application data size).

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Transaction ID.

7.3.3	Nnef_AIoT_Write service operation

Service operation name: Nnef_AIoT_Write

Description: the NF consumer requests to perform a write device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Write command specific parameters:

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Transaction ID.

7.3.4	Nnef_AIoT_PermanentDisable service operation

Service operation name: Nnef_AIoT_PermanentDisable

Description: the NF consumer requests to perform a permanent disable device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Permanent disable command specific parameters:

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Transaction ID.

7.3.5	Nnef_AIoT_Notify service operation

Service operation name: Nnef_AmbientIoT_Notify

Description: This service operation allows the consumer to receive the status or results of the requested service operation. If the NF consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Read or Nnef_AIoT_Write or Nnef_AIoT_PermanentDisable service operation, the NF consumer implicitly subscribes to receive the status or results of the requested service operation.

Inputs, Required: Transaction ID, Device Operation Execution Result indication (Success or Failure), List of Ambient IoT Device ID, Failure Cause in case of Failure. In case of Naiotf_AIoT_Read, data to read for each target Ambient IoT Device.

Outputs, Required: Operation execution result indication.

7.4	AMF services

The following table shows the Namf_AIoT Service and Service Operations. This Namf_AIoT AMF service is used when AIOT RAN and the AIoTF communicate indirectly via an AMF.

Table 7.4-1: NF services provided by AMF

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageDelivery | Request/Response | AIOTF |
|  | Notify | Subscribe/Notify | AIOTF |

7.4.1	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: the NF consumer requests to send AIoT data towards AIOT RAN or AIoT devices.

Inputs, Required:

1) AIoT NGAP Message Type ("Inventory" or "Command").

2) AIoT data to deliver:

-	NGAP INVENTORY REQUEST Message Container.

-	or, NGAP COMMAND REQUEST Message Container.

3) AIOTF Identifier, this is to allow the AMF to route back the requesting AIOTF with the response(s) to the delivered request.

4) AIOT RAN ID/Reader ID (s).

Outputs, Required: NGAP INVENTORY RESPONSE Container or NGAP COMMAND RESPONSE Container, Result indication (Success or Failure), Failure Cause in case of Failure.

7.4.2	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: the NF consumer requests to receive AIoT data from AIOT RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from AIOT RAN or AIoT devices.

Inputs, Required:

1) AIoT data to receive:

-	NGAP INVENTORY Report Message Container.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

7.5	ADM services

The following table shows the Nadm_DM Service and Service Operations.

Table 7.5-1: NF services provided by AIOTF

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_DM | Query | Request/Response | AIOTF, NEF |
|  | Update | Request/Response | AIOTF |

7.5.1	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query.

Description: NF service consumer requests the AIoT data from the ADM.

Inputs, Required: AIoT Device ID, or AF ID.

Outputs, Required: Requested subscription data.

7.5.1	Nadm_DM_Update service operation

Service operation name: Nadm_DM_Update.

Description: NF service consumer intends to update AIoT data in the ADM.

Inputs, Required: AIoT Device ID, updated AIoT data

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

# * * * * End of changes * * * *