3GPP TSG-WG 3GPPSA2#166-Ad Hoc-e	S2-2500933

Online, 20th JAN – 24th JAN, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Editor’s Notes resolutions in the TR (Cat-1 ENs)

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Conclusion update to resolve the related ENs identified as Cat-1 editor’s notes.

1. Discussion

1.1	Overview

We examine the ENs that remain in the TR conclusion and provide updates to address part of them. Those target ENs have no dependencies on other WGs or should be led by SA2 WG.

1.2	Key Issue 1 Editor’s Notes

1.2.1	Reader Selection

Editor’s note:	Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 needs coordination between SA WG2 and RAN WG(s).

It is to be determined by RAN WG3 whether the BS reader will be exposed to AIOTF. If RAN WG3 determines that only AIOT RAN is visible to an AIOTF, the BS reader is selected by AIOT RAN which is not in SA2 scope and SA2 expects to provide information to allow AIOT RAN to determine which BS Reader(s) to select. If RAN WG3 determines that AIOT RAN will also expose BS reader to AIOTF, the AIOTF will select the BS reader and shall determine the associated AIOT RAN when it sends the DL traffic.

SA2 will align the required information, either BS Reader list or other information require by AIOT RAN select BS Readers, as appropriate to AIOT RAN during the normative phase.

Proposal-1: Keep both and align with RAN3 during normative phase.

1.2.2	Aggregation

Editor's note:	How the aggregation can be done is FFS.

Aggregation can not only be done by AIOTF and the RAN TR 38.769 has concluded that "Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report."

The AIOTF can also instruct the Reader to perform the aggregation before transferring the responses back to the AIoTF, e.g., during the inventory procedure. This is to mitigate signal storms in the 5GS. The aggregation is performed for the same transaction and based on CN assistance information e.g., period time, number of devices etc.

Proposal-2: Clarify how the aggregation is done and the AIoT RAN node, as RAN concluded, can also perform aggregation as AIOTF instructed.

1.2.3	Additional AIoT Device NAS messages

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g., Read and Write) Request and Response over AIoT Device NAS layer.

Based on TR 38.769, during the inventory procedure, the CN will send the inventory request to the Reader which will trigger the reader to send an A-IoT paging message which includes an indication of which devices that need to respond. During the A-IoT random access procedure, the AIoT Device will send D2R message to the reader which includes the first UL AIoT Device NAS layer containing AIoT Device ID.

For command operations, the Command Request and Command Response will be included in the AIoT Device NAS layer.

Therefore, there is no more messages foreseen at this stage. The EN can be deleted directly at this stage.

Proposal-3: No more AIoT Device NAS messages are required in this release.

1.2.4	AIOT Device Related Information

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:	Where to store the AIOT device related information is FFS.

The network may manage the AIOT device related information (e.g., device context information), including the AIOT device permanent ID, and optionally the last known reader information.

The last known reader information is mainly to optimize the reader selection for the subsequent AF requests targeting to a specific device and is used by the AIOTF.

Additionally, the AIOTF can obtain the subscription-like data for the device, this can be cached as part of the AIOT device related information.

Proposal-4: Clarify that the AIOTF manages the device related information locally, and the device context information can optionally include the device’s subscription-like data. The device context information is used to optimize the handling of the AF request, like reader selection or AF request authorization.

1.3	Key Issue 2 Editor’s Notes

1.3.1	Length of Device ID

Editor's note:	The length of Identifier is fixed or dynamical is FFS.

The length of Identifier will have impact on both system flexibility and complexity.

If the length of the Ambient IoT Device ID is fixed, the system complexity would be minimized since the radio resource can be allocated properly and the key for fetching subscription-like data can be easily derived. However, it may make the length quite long in order to consider the maximum capacity of the whole system, which will have impact on the efficiency of inventory as longer data transmission is always required including in the paging message.

If the length of Identifier is dynamical, the AIoT Device needs to understand the length of its Device ID in order to report fully report it. Moreover, since the length of the Device ID is related with radio resource allocation, if the length of Device ID is dynamic, the radio resource may be difficult to determine.

Therefore, there should be a trade-off between fix length and dynamic length.

One possible option is to define a few levels (e.g., two or three) of the length of Device ID and during the procedure of a service request from a third party, core network can further subdivide the AIoT Devices with same length of Device ID into the same service operation task to enable proper radio resource allocation.

The detailed solution can be further discussed and decided during normative phase taking into account the trade-offs and stage 3 encoding. The EN can be removed and a NOTE can be added to clarify that this issue can be resolved during normative phase based on SA WG2 guidance and coordination with CT WGs.

Proposal-5: remove the EN and clarify to align with CT WGs during normative phase in a NOTE.

1.3.2	Subscription-like Data Storage

Editor’s note:	Where to store the AIoT device subscription is FFS.

Disable service operation is supported as one of the Ambient IoT services. When an AF requests to disable an Ambient IoT Device, the disable operation may fail due to many reasons, e.g., out of coverage, link failure, low energy etc. In this case, the network should not only inform the AF of the failure of the disable operation, but also record the status for the devices to be disabled. Because at the time of disabling operation or after disabling failure, the network may receive other AF requests (like inventory, read, write) towards the devices to be disabled, and the network should prevent such requests since such devices are supposed to be disabled already. Thus, the device disabled flag is proposed as part of the device subscription-like data.

A network may have multiple AIOTF instances. To support AIOTF selection/discovery, it is beneficial to record the last known AIOTF that serves the an AIoT Device in the subscription-like data. For example, when the AF request has no target area information, it will be efficient for the NEF to check the last known AIOTF from target device’s subscription-like data to locate the last known AIOTF rather than try all possible AIOTF instances in the network.

This becomes especially important for subsequent AF requests with commands targeting to a specific device, after an inventory, as the same AIOTF that reported the device in the inventory procedure will have a AIoT Device related information, such as which reader to use and retrieved device subscription-like information/context (as described but the AIoT Device Related Information).

As the network may have multiple AIOTF instances, the AIoT device subscription should be stored at a network entity other than the AIOTF, otherwise it unclear which AIOTF stores which AIoT Device subscription-like information.

This network entity can additionally support ID security like mechanisms depending upon SA3.

Proposal-6: Clarify that the device subscription-like data includes the device disabled flag, last known AIOTF. A new network entity, proposed as ADM (Ambient IoT Data Management) is used to store device subscription-like data. Whether this entity support ID security needs coordination with SA3. Additionally, ADM is proposed to also store the subscription data for AF.

1.4	Key Issue 3 Editor’s Notes

1.4.1	Temporarily Disable and Enable

Editor's note:	It is FFS whether and how to support enabling temporarily disabled AIoT devices.

Temporarily disabled and enabled service operation are not discussed during TR phase. If an AIoT Device is temporarily disabled, how to enhance paging message to request the AIoT Device to respond to the paging message for the following command is not discussed. If temporary disabled and enable service operations are further supported, there may be further impacts on other WGs such as SA WG3, RAN WG2 and RAN WG3. Therefore, considering there this has not been addressed by solutions during the study, it is recommended and suggested that temporary disable and enable is not supported in this release. Hence, the EN is deleted and add a NOTE to further clarify that.

Proposal-7: Temporary disable and enable is not supported in this Release

1.4.2	Command-only Support

Editor’s note:	If the “command-only” case applies, pending SA WG3, if a command and paging can be performed in a single operation, then an AIoT specific NAS message may be included in the request.

From AF perspective, it is not visible whether the command is sent to the AIOT Device via Inventory and Command procedure or Command-only procedure.

Currently, the procedure of Inventory and Command is the basic procedure based on TR 38.769. It is unnecessary to additionally define another procedure to achieve command service operation.

In addition, the security mechanism applied in Command-only procedure may be different from the one applied in Inventory and Command procedure.

Introducing two different procedures may add complexity. Therefore, it is recommended and suggested that Command-only is not supported in this release.

Proposal-8: Command-only is not supported in this release

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

8.1	Conclusion on Key Issue #1

8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIOTF performs the following functionality.

a.	The AIOTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF can select BS readers or AIOT RAN nodes. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

The information to provide to AIOT RAN for reader selection, (i.e., a BS Reader list / other information to allow AIOT RAN to select the BS Readers), will be determined during the normative phase.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

c.	The AIOTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIOTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF. The aggregation can be performed based on an aggregation time period or number of devices for a given AF request. Additionally, the AIOTF may instruct the A-IoT RAN to aggregate the service operation results from AIoT devices, e.g., Multiple individual A-IoT Device IDs (one ID per device) can be provided to the AIOTF via a single Inventory Report as specified in TR38.769 [x] clause 6.5.1.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer only supports Inventory Response and Command (e.g. Read and Write) Request and Response in this release.

5.	The AIOTF may have the AIOT device context information, this device context information is locally generated and used by an AIOTF and is deleted when e.g., a timer expires, it includes the AIOT device permanent ID, and optionally the last known reader information and retrieved device subscription data.

8.1.2	Architecture to Support Topology 1

8.1.2.1	General

The principles and aspects in this clause are agreed to support Topology 1:

-	The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1:

-	Send requests that trigger BS Reader(s) (via AIoT RAN) to perform AIoT operations, either directly or via an AMF.

NOTE 1:	It is assumed AIoT RAN supports Ambient IoT specific functionalities. There is no assumption about whether the AIoT RAN (i.e. BS Reader) also has any gNB functionality for NR-Uu or not.

NOTE 2:	NGAP is used when the AIOTF directly communicates with the AIOT RAN (i.e. over the Nx reference point) or indirect communication with AIOT RAN via an AMF (i.e. over the N2 reference point).

NOTE 3:	It is assumed Ambient IoT services can be deployed independently from existing deployments.

NOTE 4:	It is not expected a deployment will use both direct communication between AIOT RAN and an AIOTF and indirect communication between AIOT RAN and an AIOTF via an AMF. The deployment choice can be based on, for example, using a direct communication path for a local deployments.

-	The signalling transport for NGAP at the A-IoT RAN node is SCTP/IP.

8.1.2.2	AIOT RAN and the AIOTF communicate directly

When a AIOT RAN and the AIOTF communicate directly:

-	The AIOTF communicates with a AIOT RAN via a direct interface reference point Nx.

-	Figure 8.1.2.2-1 below shows the aspects related to Topology 1 (direct path) architecture in reference point representation architecture with other NFs removed.

Figure 8.1.2.2-1: Non-Roaming 5G System Architecture (Direct Path)

-	Figure 8.1.2.2-2 below shows the aspects related to Topology 1 (direct path) architecture in reference point representation with other NFs removed.

Figure 8.1.2.2-2: Non-Roaming 5G System Architecture in reference point representation (Direct Path)

NOTE 1:	NGAP used over Nx reference point will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

NOTE 2:	The protocol stack used between the AIoTF and the AIOT RAN will be concluded by RAN WG3.

-	Figure 8.1.2.2-3 below shows the aspects related to Topology 1 (direct path) protocol stack between the AIOT RAN and AIoTF.

Figure 8.1.2.2-3: Example Protocol Stack between AIOTF and AIoT Device for Topology 1 (Direct Path)

NOTE 3:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be determined by RAN WG3.

8.1.2.3	AIOT RAN and the AIOTF communicate indirectly via an AMF

When AIOT RAN and the AIoTF communicate indirectly via an AMF:

-	The AIoTF connects with AIOT RAN via an AMF. NGAP over the N2 reference point between the AIOT RAN and AMF supports Ambient IoT services including delivery of inventory/command messages.

NOTE 1:	If network isolation is required an AMF instance is deployed for supporting the AIOTF communication with AIOT RAN.

-	NGAP between AIOT RAN and the AMF is enhanced to support Ambient IoT Services.

NOTE 2:	The details of the enhancements will be concluded by RAN WG3.

-	The AMF shall be enhanced to support:

-	On the Nz reference point, the AMF supports Services which are used by an AIoTF for Ambient IoT Operations e.g. to send AIoT requests towards AIOT RAN and to receive AIoT responses from the AIOT RAN.

-	On the N2 reference point, the AMF supports sending AIOT information to the AIOT RAN (e.g. operation requests) and receiving responses from the AIOT RAN.

-	The AMF routes the AIoT messages between the AIOT RAN (over N2 reference point) and the AIOTF (over Nz reference point).

NOTE 3:	Whether to enhance an existing service or define a new service will be determined in the normative phase.

-	Figure 8.1.2.3-1 below shows the aspects related to Topology 1 (indirect path via AMF) architecture in reference point representation with other NFs removed.

Figure 8.1.2.3-1: Non-Roaming 5G System Architecture (indirect Path via AMF)

-	Figure 8.1.2.3-2 below shows the aspects related to Topology 1 (indirect  Path via AMF) architecture in reference point representation with other NFs removed.

Figure 8.1.2.3-2: Non-Roaming 5G System Architecture in reference point representation (indirect Path via AMF)

-	Figure 8.1.2.3-3 below shows the aspects related to Topology 1 (indirect path via AMF) protocol stack between AIOT RAN and AIoTF.

-	AIoT Data represents information exchanged between the AIoT Device and AF (application specific content) and AIoT Reader Control represents the requests and responses between the AIOTF and AIOT RAN.

Figure 8.1.2.3-3: Example Protocol Between AIOTF and AIoT Device for Topology 1 (indirect Path via AMF)

NOTE 4:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be determined by RAN WG3.

Editor's note:	How addressing works for UL traffic (i.e. how the BS Reader identifies the appropriate AMF to which to forward UL messages) in the indirect path via AMF is FFS.

8.1.3	Architecture to Support Topology 2

8.1.3.1	General

The following principles apply:

-	Architecture: Two architecture options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

-	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

Editor’s note:	Additional subscription information, e.g. validity information, for the UE Reader is FFS.

-	UE Reader subscription information is available to AMF and AIoTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Editor’s note:	Whether and how to enable authorization to the UE is FFS.

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g. taking into account assistance information received from the AIoTF) from the gNB.

NOTE 1:	Assistance information is further clarified in other clauses.

NOTE 2:	The details of UE Reader radio resource management will be defined by RAN WG2.

8.1.3.2	User-plane option

The following additional principles apply:

-	As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

Figure 8.1.3.2-1: User-plane architecture for Topology 2

Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2

NOTE 1:	Which transport protocol to use for AIoT-AP can be decided by CT WG1.

NOTE 2:	Security for AIoT-AP is assumed to be defined by SA WG3.

-	To operate as a UE Reader, the UE establishes a PDU Session to a specific DNN/S-NSSAI and establishes an association with the AIoTF identified by an FQDN using the AIoT AP protocol. In this release, the UE is preconfigured with the specific DNN/S-NSSAI and the AIoTF FQDN.

Editor’s note:	Whether and which additional methods to dynamically provision the UE with this information can be specified and in which release is FFS.

-	If the AIoTF determines that the UE is not authorized to operate as a UE Reader according to the UE’s subscription, then the AIoTF rejects the UE’s association request.

-	If the AIoTF detects that the Reader function in the UE does not respond to a request by the AIoTF (e.g. an Inventory Request or Command Request), then the AIoTF considers the UE Reader unreachable and locally deletes the association for the UE Reader.

8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIoTF. The related protocol stack is shown in Figure 8.1.3.3-1.

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details how the RRC option works are FFS.

Figure 8.1.3.3-1: Protocol Stack for the RRC option

8.1.3.4	Interim Conclusion for authorization

The following interim conclusion are agreed for the UE Reader authorization in Topology 2:

-	The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader.

-	The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader.

Editor's note:	How to authorize the UE Reader will be determined later.

8.2	Conclusion on Key Issue #2

8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier and Identification Management.

The following principles are agreed for Identifier and Identification Management:

-	An Ambient IoT Device is configured with a permanent Ambient IoT Device Identifier which can be assigned by an operator or by a third party. The Identifier is used to identify Ambient IoT Device and locate the corresponding authentication server.

NOTE 1:	How to configure Ambient IoT Device with the permanent Ambient IoT Device Identifier is out of 3GPP scope.

The permanent Ambient IoT Device Identifier includes the following information:

1)	Part1information:

-	The ID type, including

-	information indicating whether the network identifier is included or not.

-	information indicating whether information used to identify a 3rd party is included or not.

-	the Part2 type indicating EPC or the other format.

-	A network identifier (i.e. MCC+MNC and/or NID), when the ID includes the network identifier.

-	Information used to identify a 3rd party when the ID includes the information used to identify a 3rd party.

2)	Part2 information:

-	The information (e.g. EPC or others) used to distinguish different Ambient IoT Devices within the scope identified by the Part1 information.

Editor's note:	Part2 information for the operator assigned and 3rd party assigned ID needs further study.

NOTE 3:	The coding for the above information is left to stage 3.

Figure 8.2.1-1: The example of Operator allocated ID

For operator allocated Part 1 information, the network identifier is mandatory and can be used to index the authentication server or not. If it is not used to index the authentication server, the ID may further include the information used to identify a 3rd party. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the authentication server.

Figure 8.2.1-2: The example third party allocated ID

For third party allocated Part 1 information, the network identifier is not needed. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the authentication server.

NOTE: 	The length of AIoT Device ID will be determined during normative phase with coordination with CT WGs.

With the above information, the AIoT device ID is globally unique.

Editor's note:	Whether the temporary ID in the AIoT NAS layer is required for the privacy protection is FFS and is pending SA WG3 decision.

8.2.2	Subscription Management

The AIoT device may or may not have operator’s subscription data in the network. If the AIoT device doesn’t have the operator’s subscription data in the network, the corresponding related data (e.g. Device ID or credentials) is stored in the AAA server external to the network.

Subscription data for an Ambient IoT Device is required in case the Ambient IoT Device is managed by the network:

-	It is used by the AIOTF to check whether the device is subscribed.

NOTE 1:	Checking whether the device is subscribed needs coordination with SA WG3 study outcomes on validation of the ambient IoT Device identifier.

-	It is dedicated subscription data for Ambient IoT Devices, i.e. different with UE subscription data. It is stored in a new network entity (Ambient Data Management, ADM) that supports management of AIoT device subscription data, this new ADM network entity is independent from the UDM managing the UE subscription data.

NOTE 2:	The ADM network entity will support ID security mechanisms, depending upon SA3.

-	It contains at least a permanent Ambient IoT Device ID, disabled flag, last known AIOTF information etc.

Editor’s note:	SA WG2 will align security related materials in subscription data with SA WG3 decision later.

The Ambient IoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate where the Subscription data or related data of an Ambient IoT Device is.

Subscription data for 3rd party AF is required:

-	It is used by the AIOTF together with SLA and operator policy at NEF to perform authorization of the AF request targeting for Ambient IoT service.

-	it is stored at ADM within 5G network.

Editor’s note:	Items within the subscription data for 3rd party AF need to be figured out later.

UE Subscription data is enhanced to include an indication that whether the UE is allowed to operate as a UE reader. This is concluded only for topology 2 which is not pursued in Rel-19 time-frame.

8.3	Conclusion on Key Issue #3

8.3.1	General

The clause concludes the following aspects:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE:	The above aspect includes studying whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

Editor's note:	Additional conclusions are FFS.

8.3.2	AIoT services supported by the 5GC

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

NOTE:	The security aspect of Permanent Disable is to be concluded by SA WG3. Temporary Disable and Enable is not supported in this Release.

8.3.3	NEF exposure

NEF supports to expose Ambient IoT service towards the AF.

-	New NEF service (e.g. Nnef_AIoT) to support the AIoT services including Service Operations of Inventory, Read, Write, PermanentDisable and Notify will be defined.

Editor's note:	NEF event exposure aspect is FFS.

8.3.4	Principles on the procedures to support AIoT services

To support the services provided by 5GC and the NEF exposure of those AIoT services, the following procedures are supported:

-	Inventory Procedure.

-	Command Procedure, to e.g. transfer AF AIoT Data to/from AIoT Device(s) as AIoT specific NAS messages.

There are requests that are used from the AIOTF towards the Reader and responses from the Reader to the AIOTF. The routing of the request and response messages and their encoding depends on the topology and transport to the Reader (see KI#1).

All the procedures follow have the following steps:

1.	The AF makes a service request to the NEF, including parameters to identify the target AIoT Device(s), target Readers and service operation specific parameters.

2.	The NEF determines an AIOTF for the requested operation, and invokes a new service operation on the AIOTF.

3.	For the requested operation the AIOTF:

1).	Performs initial reader selection by either:

-	The AF providing information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader to use for the request.

NOTE 1:	Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation.

- 	If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

- 	If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

If no readers can be selected then the request is rejected.

2).	Determines A-IoT Device Identification information  based on the information from the AF, to be included in the paging message on the AIoT radio interface to find the AIoT Devices. AIoT Devices compare the A-IoT Device Identification information  with their own AIoT Device Identifier (part of or full AIoT Device Identifier) to determine whether respond to the paging message.

Editor's note:	Whether and how the A-IoT Device Identification information will be security protected will be concluded by SA WG3.

3).	Determines Reader Assistance information required for the operation used to the Reader, taking into account assistance information from the AF.

4).	Constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request. The Inventory request is routed to the Readers determined by the initial reader selection.

NOTE x:	“command-only” case is not supported.

See clause 8.1 for how to provide the request to a Reader.

5).	The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages. The AIOTF can determine which request the results are for using the correlation identifier.

6).	The AIOTF may, depending on the information within the AIoT Device identifier, obtain subscription-like information from either:

-	the serving network performing the operation (either as identified by a AIoT Device Identifier or operator policy to check it been provided with information for a specific AIoT Device), or

-	another network as identified by a AIoT Device Identifier, or

-	A third party as identified by a AIoT Device Identifier.

7).	Checks if the AIoT Device is subscribed.

NOTE 2:	Whether and how AIOT Device Identifiers are verified depends on SA WG3.

8).	If the operation is a command operation, the AIOT generates a request, including an AIoT specific NAS message for the command, along with any additional information required by the Reader to execute the command, a TASK ID etc. The request is then routed to the Reader. The Reader executes the command, passing the AIoTF specific NAS message to the AIoT Device and collecting any AIoT specific NAS responses. The AIoT specific NAS responses are then routed back to the AIOTF.

NOTE 3:	Whether and how security protection is applied to the AIoT specific NAS message send to the AIoT Device and the response AIoT specific NAS message from the AIoT Device depends on SA WG3.

9).	Provide the results of the operation to the NEF. Results from multiple AIoT Devices may be included/aggregated in the service response(s).

4.	Provide the results of the operation from the NEF to the AF.

8.3.5	Information provided by the AF

The AF provides one or some of the following information:

-	Information about AIoT services:

-	Service type indicating e.g. inventory or command (e.g. read or write).

-	Information to be used for AIoT reader selection:

-	At least one of the UE reader ID or the target area information.

-	Optionally, Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

-	Optionally, Information to be used for resource allocation that is detailed in TR 38.769 [8]:

-	Approximate number of AIoT devices.

-	Approximate D2R message size.

NOTE:	Information to be used for resource allocation will be aligned with RAN WGs during the normative phase.

# * * * * End of changes * * * *