3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500939

Online, 20th Jan – 24th Jan, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Ambient IoT Architecture model and concepts

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: Ambient IoT architecture model and concept’s introduction and description for the 5GS system support of Ambient IoT.

1. Introduction/Discussion

The architecture model and concepts clause of the Ambient IoT TS includes an overall general concept description, architecture, split into 2 parts, general architecture and AIoT RAN specific aspects, the reference points and service-based interfaces to support it.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (All new texts) * * * *

4	Architecture model and concepts

4.1	General concept

Ambient IoT is a service that can be provided by the 5GS based to support Ambient power-enabled IoT devices that are powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g., using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other suitable power source.

The 5GS System architecture for Ambient IoT include the following functions and procedures for:

-	Ambient IoT Device identification;

-	Ambient IoT Device inventory;

-	Providing to, and obtaining from, an Ambient IoT Device application data;

-	Utilising Ambient IoT Reader functionality in RAN.

4.2	Architecture

4.2.1	General

The 5GS System architecture for Ambient IoT has a general architecture to support Ambient IoT, which includes the core network functions, AIoT Reader and Ambient IoT Device. Building upon this, the 5GS system supports different AIoT Reader architectures, which allow different deployment options and different AIoT Readers to be used by the AIOTF, for the following cases:

-	AIoT RAN hosted AIoT Readers, which includes a direct path between AIOT RAN and the AIOTF and an indirect path between AIOT RAN and the AIOTF via an AMF.

Figure 4.2.1-2 depicts the non-roaming Ambient IoT architecture showing the AIoT Reader independent aspects, with the link between the AIOTF and AIoT Reader which depends Reader architecture described in other clauses represented by a dashed line.

Figure 4.2.1-1: Non-roaming Ambient IoT System Architecture (AIoT Reader Independent)

Figure 4.2.1-2 depicts the non-roaming Ambient IoT architecture, using the reference point representation, showing AIoT Reader independent reference points, with the link between the AIOTF and AIoT Reader which depends Reader architecture described in other clauses represented by a dashed line.

Figure 4.2.1-2: Non-roaming Ambient IoT System Architecture (AIoT Reader Independent) in reference point representation

NOTE 1:	For the sake of clarity of the point-to-point diagrams, the AF and NRF have not been depicted. However, all depicted Network Functions can interact with the UDR, NEF and NRF as necessary.

NOTE 2:	The ADM uses subscription data and authentication data that may be stored in UDR (refer to clause 4.5.8).

NOTE 3:	For clarity, the UDR and its connections with other NFs, e.g. ADM, are not depicted in the point-to-point and service-based architecture diagrams. For more information on the ADM data storage architectures refer to clause 4.5.8.

The architecture for Network Exposure Function, using reference point representation, defined in TS 23.501 [x] clause 4.2.3 is applicable for Ambient IoT, with a southbound interface from NEF to AIOTF.

4.2.2	Architecture for A-IoT RAN Readers

4.2.2.1	General

A-IoT RAN supports AIoT Reader functionality and the 5GS system architecture for Ambient IoT supports the following paths to access an AIoT RAN:

-	Direct Path: AIOTF communicates with a AIOT RAN via a direct interface, as shown in clause 4.2.2.2.

-	Indirect Path via AMF: AIOT RAN and the AIOTF communicate indirectly via an AMF, as shown in clause 4.2.2.3.

4.2.2.2	Direct interface between AIOTF and A-IOT RAN

Figure 4.2.2.2-1 depicts the Ambient IoT architecture showing how the AIOTF access A-IoT RAN by a direct interface.

Figure 4.2.2.1-1: A-IoT RAN – AIOT Direct Interface Architecture

Figure 4.2.2.2-2 depicts the Ambient IoT architecture, using the reference point representation, showing how the AIOTF access A-IoT RAN by a direct interface.

Figure 4.2.2.2-2: A-IoT RAN – AIOT Direct Interface Architecture in reference point representation

4.2.2.3	Indirect interface via AMF between AIOTF and A-IOT RAN

Figure 4.2.2.3-1 depicts the Ambient IoT architecture, using the reference point representation showing how the AIOTF access A-IoT RAN by an indirect interface via an AMF.

Figure 4.2.2.1-1: A-IoT RAN – AIOT Indirect Interface Architecture

Figure 4.2.2.2-2 depicts the Ambient IoT architecture, using the reference point representation showing how the AIOTF access A-IoT RAN by an indirect interface via an AMF.

Figure 4.2.2.2-2: A-IoT RAN – AIOT Indirect Interface Architecture in reference point representation

4.3	Reference points

The Ambient IoT Architecture contains the following reference points:

Ny:	Reference point between the AIoT Device and the AIOTF.

Nx:	Reference point between the A-IoT RAN and the AIOTF.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

Na:	Reference point between the AIOTF and the AMF.

Nb:	Reference point between the AIOTF and the NEF.

Nc:	Reference point between the AIOTF and the NRF.

Nd:	Reference point between the AIOTF and the ADM.

Ne:	Reference point between the ADM and the UDR.

Nf:	Reference point between the ADM and NEF.

In addition to the relevant reference points defined in TS 23.501 [x], in the case of Ambient IoT, these referenece points are as follows:

N2:	Reference point between the A-IoT RAN and the AMF.

N33:	Reference point between NEF and AF.

4.4	Service-based interfaces

Naiotf:	Service-based interface exhibited by the AIOTF.

Nadm:	Service-based interface exhibited by the ADM.

In addition to the relevant services defined in TS 23.501 [x] for the following service-based interfaces, in the case of Ambient IoT, the services can be provided by corresponding NF are as follows:

Namf:	Service-based interface exhibited by AMF.

Nnef:	Service-based interface exhibited by NEF.

Naf:	Service-based interface exhibited by AF.

Nnrf:	Service-based interface exhibited by NRF.

4.5	Functional Entities

4.5.1	Ambient IoT Device

An Ambient IoT Device is a type of equipment that has an identity and can store a range of information. The identity is read from the AIoT Device by an AIoT Reader and the information can be read and written via an AIoT Reader using the A-IoT radio interface.

The Ambient IoT Device supports:

-	Inventory procedures, including reporting its identity,

-	Command procedures, to read and write application data to and from the device, and disable functionality.

The Ambient IoT Device supports a protocol between the Ambient IoT Device and the AIOTF to facilitate and support the procedures.

4.5.2	AIoT Readers

4.5.2.1	General

An AIoT Reader is functionality that provides communication services towards Ambient IoT Devices, in response to requests from the network, over an A-IoT Radio.

The following types of AIoT Reader are supported:

-	A-IoT RAN Reader, as described in clause 4.5.2.2.

4.5.2.2	A-IoT RAN Reader

A-IoT RAN supports NGAP to either AIOT over Nx or to an AMF over N2. The A-IoT RAN may support 1 or more AIoT Readers.

4.5.3	AIOTF

The AIOTF supports Ambient IoT operations and provides the following functionality:

-	Receives and authorises Ambient IoT service requests from an AF and triggers AIoT Readers to perform operations towards Ambient IoT Devices.

-	Select which AIoT Readers will be used to fulfil an AF request.

-	Provide assistant information to AIoT Readers to enable to them to fulfil requests.

-	Aggregate results from multiple AIoT Readers and providing results to the AF.

-	Read and update information in the Ambient IoT Device profile data in the ADM.

-	supports the protocol between the Ambient IoT Device and the AIOTF to facilitate and support the procedures and Ambient IoT services.

4.5.4	NEF

In addition to the functions defined in TS 23.501 [x], the NEF supports the following:

-	To enable AFs to provide service specific information to the 3GPP network, the NEF supports additional services for Ambient IoT as specified in clause 7.4.

-	Select which AIOTF is used for an AFs request, based on include the information in the AF request and information for Ambient IoT Devices stored in the ADM accessed using the Nadm interface.

4.5.5	AF

AFs and NFs may access Ambient IoT services from an AIOTF in the same trust domain (e.g. in the same PLMN) using the Naiotf interface. External AFs may access Ambient IoT services from an NEF using Nnef interface.

4.5.6	NRF

In addition to the functions defined in TS 23.501 [x], the NRF performs the following functions:

-	AIOTF discovery.

4.5.7	AMF

In addition to the functions defined in TS 23.501 [x], the AMF supports the following:

-	The AMF contains functionality responsible for routing messages to and from A-IoT RAN and AIOTF. The AMF is accessible to the AIOTF via the Namf interface and to the A-IoT RAN via the N2 reference point.

If the AMF is deployed to only support Ambient IoT, the other functionally defined in TS 23.501 [x] is optional to support.

4.5.8	ADM

The Ambient Data Management NF, ADM. contains Ambient IoT Device information and authorisation data for external AFs to validate the external AFs request for Ambient IoT services. The ADM is accessible from an AIOTF or NEF via the Nadm interface.

The ADM may also contain an indication which AIOTF was the last to interact with an Ambient IoT Device.

As depicted in Figure 5.5.8-1, the 5GS System architecture for Ambient IoT allows the ADM to store data in the UDR, in addition to NFs as described in TS 23.501 [x] clause 4.2.5, and services the following additional function:

-	UDR accessed by the ADM belongs to the same PLMN where the AMD is located.

Figure 4.5.8-1: ADM Data Storage Architecture

NOTE 1: A separate instance of the UDR can be deployed, storing Ambient IoT data and providing service to the ADM, as described in TS 23.501 [x] clause 4.2.5.

4.6	Protocol Stacks

4.6.1	General

This clause specifies the overall stacks between entities used for Ambient IoT. The protocol stacks include the following cases:

-	Protocol stack between Ambient IoT Device and the AIOTF and between the Ambient IoT Device and the AF,

-	Protocol stack between AIoT RAN and AIOTF

4.6.2	Protocol stack between Ambient IoT Device - AIOTF - AF

Figure 4.6.2.2-1 represents the protocol stack from the Ambient IoT Device, via an AIoT Reader towards the AIOTF and AF.

The protocol stacks and routing of information between an AIOT Reader and the AIOTF protocol is defined in other clauses depending upon the architecture used, e.g. directly between AIoT RAN containing an AIoT Reader and the AIOTF, indirectly between AIoT RAN containing an AIoT Reader and the AIOTF via an AMF, etc, and AIoT Reader Stack represents all those options in the figure.

Legend:

-	AIoT radio layers: Ambient IoT radio layers define by RAN in TS 38.xxxx [x].

-	AIoT Reader Stack: represents the protocols, layers, and NFs used to transport information between an AIOTF and AIoT Reader, as defined in this specification, e.g. direct interface between AIOTF and A-IOT RAN, etc.

-	AIoT NAS: represents the protocol with requests and responses between the AIOTF and Ambient IoT Device.

-	AIoT Data: represents the application layer data and information transported between an AF and an Ambient IoT Device.

Figure 4.6.2.2-1: Protocol stack between AIoT Device and AIOTF

4.6.3	Protocol stacks between AIoT Reader - AIOTF

4.6.3.1	Overview

The protocol stacks between an AIoT Reader and AIOTF includes the following case:

-	Protocol Stacks between AIoT RAN and AIOTF, including a direct path between AIoT RAN and AIOTF and an indirect path between AIoT RAN and AIOTF via an AMF.

-	Protocol Stacks between UE Reader and AIOTF.

4.6.3.2	Protocol Stacks between AIoT RAN – AIOTF

The protocol stacks include the following two cases:

-	Direct Path: AIOTF communicates with a AIOT RAN via a direct interface reference point Nx, as shown in Figure 4.6.2.2-1.

-	Indirect Path via AMF: AIOT RAN and the AIOTF communicate indirectly via an AMF, as shown in Figure 4.6.2.2-2.

Whether the AIOT RAN communicates with an AIOTF via the direct path or indirect path is not visible to the Ambient IoT Device.

Legend:

-	NG Application Protocol (NG-AP): Application Layer Protocol between the AIOT RAN and the 5GC. The NG-AP supports Ambient IoT Services and is defined in TS 38.xxxx [x].

-	Stream Control Transmission Protocol (SCTP): This protocol guarantees delivery of signalling messages between the AIOT RAN and the 5GC. SCTP is defined in RFC 4960 [44].

-	AIoT Reader Control: represents the requests and responses between the AIOTF and AIOT RAN.

Figure 4.6.3.2-1: Protocol stack between AIOTF and AIoT Device (Direct Path)

Editor’s Note: Whether AIoT Reader Control is transported over NGAP or is part of the NGAP protocol will be determined by RAN WG3.

Legend:

-	Service-based Interface (SBI) Protocol Stack: The protocol stack for service-based interfaces (consisting of IP/TCP/HTTP2/JSON) as defined in TS 29.500 [9].

-	NG Application Protocol (NG-AP): Application Layer Protocol between the AIOT RAN and the 5GC. The NG-AP supports Ambient IoT Services and is defined in TS 38.xxxx [x].

-	Stream Control Transmission Protocol (SCTP): This protocol guarantees delivery of signalling messages between the AIOT RAN and the 5GC. SCTP is defined in RFC 4960 [44].

-	AIoT Reader Control: represents the requests and responses between the AIOTF and AIOT RAN.

Figure 4.6.3.2-2: Protocol Stack Between AIOTF and Ambient IoT Device (Indirect Path)

Editor’s Note: Whether AIoT Reader Control is transported over NGAP or is part of the NGAP protocol will be determined by RAN WG3.

# * * * * End of changes * * * *