3GPP TSG-SA WG2#166bis	S2-2500942

Albania, 20-24 Januaruy, 2025	(was S2-240xxxx)

Source:	NEC

Title:	Discussion paper attach procedure with default PDN connection

Document for:	Discussion/Approval

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3_ARCH/Rel.19

Abstract of the contribution: Discusses and propose a way forward for attach procedure with default PDN connection.

# 1.	Background

During the study phase of KI#2 Store and Forward Satellite Architecture, following things have been concluded it was concluded to have following two architecture.

1) Whole EPC onboard:

## .3.2	Architecture and Principles of Operation

An example architecture of Model B is shown in Figure Y.3.2-1. Each satellite contains the functionally of an eNB plus a full EPC that can include an MME, SGW, PGW, HSS, E-SMLC, SMSC etc. Each satellite further includes an endpoint proxy function that emulates the behaviour of a real endpoint (e.g. an AF) from the perspective of a UE. There is also store and forward functionality on the ground that can be part of, or connected to, an NTN Gateway and that contains the proxy functionality.

NOTE:   This architecture does not support roaming.

Figure Y.3.2-1: Example Architecture of Model B

2) Split MME architecture

1.	eNB is onboard the satellite.

2.	MME is split into two functions:

a.	MME-onboard:	the MME part which is onboard the satellite. MME-onboard is in charge of (1) handling the S1 interface with the onboard eNB and (2) handling the NAS protocol signalling from/to UEs via the onboard eNB.

b.	MME-ground:	the MME part which is on the ground network. MME-ground is in charge of handling the rest of interfaces towards other CN functions (e.g. S6a towards HSS, SGd towards SMS-GMSC/IWMSC /SMS Router, T6a towards SCEF, T6ai towards IWF-SCEF, S11 towards SGW).

Figure Y.2-1: "Split-MME" architecture for supporting S&F satellite operation for SMS and CP CIoT services

Conclusion:

Relevant part from Agreed CR S2-2412903.

When an MME is operating in S&F Mode:

- If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, if the UE supports Store and Forward Satellite operation, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation.

- If a UE indicates Store and Forward capability, the MME may provide to the UE a S&F Wait Timer when accepting or rejecting a NAS procedure.

# 2. 	Discussion

3GPP SA2#166 meeting has captured the normative CRs for Store and Forward Satellite feature based on the conclusion for the KI#2 . The CRs captures both functional and procedure part related to two architecture models Split MME onboard and whole EPC onboard of the Store and Forward feature. According to the conclusion the model 1 a part of the MME will be on board a satellite as a payload and the other part of the s

Following are the facts and conclusions:

Fact 1: The MME in the satellite rejects the NAS procedure if the NAS procedure can’t be completed e.g. due to non-availability of user subscription or authentication vectors or other UE parameters. When the UE receives the S&F wait time, the UE will start the S&F wait timer and restart the NAS procedure again when the service link is available.

Based on a conclusion it is captured in the normative CR that the when the MME can’t complete a NAS procedure the MME will reject the NAS procedure and will provide a Store and Forward wait timer to the UE which has indicated support of S&F functionality. When the feeder link is available and the MME in the satellite can talk to the network in the ground the MME will fetches the related information from the ground network (e.g. subscription information, authentication vectors etc.)

When the UE receives the S&F wait timer it will restart the NAS procedure again after expiry of the timer. When the service link is available it will initiate the NAS procedure and the MME in the satellite completes the NAS procedure.

Fact 2: The Attach procedure, tracking area update procedure and service request procedure capturing above conclusion is captured in the normative text in TS 23.401.

Fact 3: EPS session management procedure including attach with PDN connection, standalone PDN connection and other UE or network-initiated session management procedure are not captured yet in the split MME Architecture.

Fact 4: UE is agnostic of two architectures define to support S&F feature i.e. the UE can’t know if it is attaching a network based on split MME.

Problem statement 1: Without normative work to support ESM procedure in split MME architecture including the attach with PDN connection and standalone PDN connection establishment, a user data (IP or non-IP) cannot be sent to the AF or SCEF.

1. Attach procedure for UE and network supporting S&F operation mode.

#### 2. Attach procedure with default PDN Connection establishment (Subclause 5.3.2.1	E-UTRAN Initial Attach of TS 23.401)

Figure 2:  Figure 5.3.2.1-1: Attach procedure of TS 23.401

Problem Statement 1: S&F operation is not supported for the UE supporting Attach with default PDN connection.

As per the current conclusion and ongoing normative work based on Figure 1 above, when the feeder link is connected, the MME-GND fetches the subscription data and authentication vectors and gives the subscription and authentication vectors to the MME-SAT.

when the MME-SAT gets UE subscription and Authentication vectors, authentications and security mode procedure takes place between the UE and the MME-SAT when the UE initiates attach procedure when the S&F wait timer expires and the UE accesses the network second time. After successful authentication procedure the UE sends attach accept message.

The above procedure doesn’t cover the steps 12.-16 of Figure 2 which establishes the default PDN connection establishment procedure. Therefore, the UE is attached with S&F mode without default PDN connection establishment. The UE supporting attach with PDN connection establishment will not be able to support S&F service.

Problem Statement 2:

Current procedure doesn’t specify how a standalone UE initiated SM procedure will be completed by the UE and the EPC in case of S&F architecture.

# 2.	Proposal

There can be following two solutions:

Solution 1: PDN connection establishment after successful authentication procedure.

To solve problem 1, it is proposed that when the UE with S&F operation enabled is registering to the network supporting S&F operation mode, after the successful authentication and SMC procedure, the network rejects the attach procedure with S&F wait time and optionally satellite monitoring list.

Afterwards when the feeder link is connected between the MME-GND and MME-SAT, the MME-GND performs PDN connection establishment procedure (Steps 12-16 of Figure 2).

After establishing the service link, when the UE initiates attach procedure with MME-SAT, the MME-SAT sends attach accept message after successful establishment of the PDN connection.

The scenario flow is proposed for combined attach with default PDN connection establishment.

Solution 2: Establish default PDN connection during the provisional location update procedure.

To solve problem 1, it is proposed that the EPC will establish the default PDN connection after the MME-ground has fetched the authentication vector and subscription from the HSS. When the default PDN connection establishment is successful stores the result in the MME-satellite. When S&F wait timer expires and the service link connection is established, the UE initiates the attach procedure. The UE and the MME-satellite finishes the default PDN connection along with the EMM procedure.

The detailed scenarios are described below.

# 3.	Way Forward

Following two contributions are submitted in the meeting.

- S2-2500944: 3870 Support of attach with PDN connection in S&F mode of operation-option 1 capturing solution 1

- S2-2500945 3871 Support of attach with PDN connection in S&F mode of operation-option 2 capturing solution 2.

It is proposed to discuss solutions 1 and 2 and choose most appropriate solution among the two.