3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500952

# 20 – 24, January, 2025, E-meeting

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6041 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing EN for AF requested slice selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ SliceSel |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.501 CR there is one EN which is added to discuss the behaviour for non supporting UE regarding the handling of Replaced S-NSSAI. Editor's note: It is FFS whether the Replaced S-NSSAI should be removed from the Configured NSSAI. In Rel 18 features for temporary slice and NS-AoS as defined in clause 5.15.16 and 5.15.18.3, already addressed the scenarios for UE which does not support the feature. Similarly it is proposed that for AF requested slice replacement feature, if UE does not support the feature then AMF shall remove the Repalced S-NSSAI from Configured NSSAI and when slice replacement request is received to be stopped, then AMF shall add the Replaced S-NSSAI back to the Configured NSSAI Also the Slice rejected due to RA is access dependent but slice replacement featureis access indepdent. Hence more signalling and inefficient mechanism in comparison to removing from Configured NSSAI mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In stead of adding the Replaced S-NSSAI to the Rejected NSSAI with cause code set to not supported in RA, AMF shall remove the Replaced S-NSSAI from Configured NSSAI during slcie replacement and add back when requiest is received to stop the slice replacement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN will remain in the specification without solving the problem |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.5.2.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

##### 5.15.5.2.2a	AF Requested modification of the Set of Network Slice(s) for a UE

In non-roaming cases, an authorized AF or an AF subject to authorization via the NEF, may request to replace a certain S-NSSAI (Replaced S-NSSAI) for a UE with another S-NSSAI (Alternative S-NSSAI) which is part of the UE subscription. The request in either case is sent to the PCF for the UE handling the impacted subscriber. Similarly, an authorized AF or an AF subject to authorization via the NEF, may request to terminate the replacement then request to use the Replaced S-NSSAI instead of the Alternative S-NSSAI for a UE again. The AF may subscribe to Notification on the outcome of the Network Slice Replacement.

Only a single AF can request the modification of the Set of Network Slice(s) for one replaced S-NSSAI.

NOTE 1:	How the AF obtains the S-NSSAI to be replaced and the Alternative S-NSSAI is out of scope of this specification.

The PCF for the UE based on operator policies considers the AF request. Consequently the PCF for the UE triggers the Network Slice Replacement at the AMF as defined in TS 23.503 [45].The PCF sends Slice replacement management in the access and mobility management policies to the AMF per implicitly subscription, as described in TS 23.503 [45]. The Slice replacement management includes the Replaced S-NSSAI and the corresponding Alternative S-NSSAI as provided in the AF request, PCRT to request Notification on outcome of Network Slice Replacement request, together with a Network Slice Replacement Type. The Network Slice Replacement Type indicates that the Network Slice Replacement is requested by AF. The AMF handles the procedure as following:

Editor's note:	The value of Network Slice Replacement Type is FFS.

Editor's note:	Which PCRF to be used to request Notification on outcome of Network Slice Replacement request is FFS.

-	The AMF shall not handle the Network Slice Replacement procedure as described in clause 5.15.19.

-	If the Alternative S-NSSAI is not within the UE subscription, or is not supported in the UE Registration Area, the AMF shall notify the PCF that the Network Slice Replacement is not triggered with a cause value. The PCF further notify the AF.

-	If the Alternative S-NSSAI is within the UE subscription and is supported in the UE Registration Area, the AMF includes the Alternative S-NSSAI in the Allowed NSSAI (if not present).

-	The AMF removes the Replaced S-NSSAI from the Allowed NSSAI and Configured NSSAI  by a UE Configuration Update procedure.

-	The AMF releases the PDU sessions that were using the Replaced S-NSSAI.

-	The AMF stores in the UE context the access and mobility management policy information as received from PCF and transfer to new AMF during inter AMF mobility.

NOTE 2:	Operator policies in the PCF ensures that the UE is configured with a URSP rule that contains the Alternative S-NSSAI to establish a new PDU Session.

The trusted AF or a NEF may send request to PCF for the UE to stop the Network Slice Replacement. In this case the PCF for the UE sends access and mobility management policies to the AMF, as described in TS 23.503 [45]. The AMF add  the Replaced S-NSSAI to the Configured NSSAI by a UE Configuration Update procedure. The UE may request to register the Replaced S-NSSAIs again next time the UE sends a Requested NSSAI.

* * * End of Changes * * * *