3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500982

# Electronic Meeting, 20-24 January 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3875 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Description of procedures for split-MME architecture in informative annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sateliot, Novamint, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This paper is addressing the following point within the Exception Sheet: (2c) – Document in Informative annex for split-MME deployment that; support of Attach with PDN connection is handled based on implementation between the MME on board and ground CN entities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new subclause is added to the information annex to describe the support of NAS procedures that needs to be executed over several satellite coverage windows. The subclause includes two illustrative flowcharts depicting two possible implementations of the attach with PDN connection procedure. To properly accommodate the new clause (O.2.2), the previous content in O.2 is placed under a separate clause (O.2.1). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will remain unclear/ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | O.2, O.2.1, (new) O.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# FIRST CHANGE

# O.2	Model A: Split MME architecture

## O.2.1	Architecture and Principles of Operation

In this architecture option (see Figure O.2.1-1):

1.	eNB is onboard the satellite.

2.	MME is split into two functions:

a.	MME-onboard: the MME part which is onboard the satellite. MME-onboard is in charge of (1) handling the S1 interface with the onboard eNB and (2) handling the NAS protocol signalling from/to UEs via the onboard eNB.

b.	MME-ground: the MME part which is on the ground network. MME-ground is in charge of handling the rest of interfaces towards other CN functions (e.g. S6a towards HSS, SGd towards SMS-GMSC/IWMSC /SMS Router, T6a towards SCEF, T6ai towards IWF-SCEF, S11 towards SGW).

Figure O.2.1-1: "Split-MME" architecture for supporting S&F satellite operation for SMS and CP CIoT services

The split-MME architecture has below principles:

1.	An MME-ground is associated with at least one MME-onboard. An MME-onboard is associated with a Satellite ID identifier. The MME-ground together with the associated MME-onboard(s) behave jointly as a single MME entity.

2.	How MME-onboard(s) interacts with MME-ground and how synchronization of the UE context between them is done is outside the scope of 3GPP.

NOTE:	Based on implementation, the UE context can be uploaded on a subset of MME-onboard(s) or all MME-onboard(s) associated with MME-ground.

3.	When a UE initiates a MO procedure that needs an interaction with a core network node on the ground, the MME-onboard stores the MO procedure transaction if the feeder link is not available and synchronizes with the MME-ground when the feeder link becomes available. The MME-ground executes the procedure with the ground network nodes and syncs back the UE context with the MME-onboard when the feeder link becomes available.

4.	The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the MME-ground when the feeder link becomes available. The MT data is stored in the MME-ground when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MME-ground determines the satellite through which to send MT data and the MT data is sent to the respective MME-onboard and stored in the MME-onboard when the feeder link is available, and transferred to the UE when service link becomes available.

5.	For MO SMS, if the feeder link is not available upon reception of the MO SMS the MME-onboard can store the MO-SMS and can immediately send the delivery report (i.e. RP-ACK) to the UE i.e. as if the MO-SMS has already been successfully delivered to the Service Centre (SC). Once the feeder link is established the MO-SMS is forwarded to the SC. Subsequently the SC sends the RP-ACK to the UE. The MME-ground can also discard the RP-ACK received from the SC.

6.	To support UE location verification on satellite, the E-SMLC, if needed, can be deployed on satellite to perform the verification of UE location functionality.

7.	For the monitoring event which allows the SCS/AS to be notified of Store and Forward Satellite operation (see clauses 5.6.1.11 and 5.6.3.10 of TS 23.682 [74]) the SCS/AS communicates with HSS and MME-ground to configure and/or delete Monitoring Event.

8.	For data transfer, only Control Plane CIoT EPS Optimization applies.

## O.2.2	NAS procedure execution over multiple satellite coverage availability windows

With the split-MME architecture, given all CN functions excepting the MME-onboard are deployed on ground (see Figure O.2.1-1), a NAS procedure initiated by a UE may not be completed during a coverage availability window in a satellite operating in S&F Mode but need several coverage availability windows.

As specified in clause 4.13.9, if the MME-onboard rejects a NAS procedure because it cannot be completed during the current satellite coverage window, for a UE that supports S&F capability, the MME may include a reject cause indicating that the NAS rejection is due to S&F operation. The UE, after receiving a reject cause due to S&F operation, may re-attempt the procedure in the selected PLMN in a next coverage availability window in the same or in another satellite. During the interval between the time that the NAS procedure was rejected and the time that the UE re-attempts it, the CN is expected to take the steps needed to progress towards a successful completion of the procedure when the re-attempt occurs. Following these principles, one or several re-attempts may be needed for completion of a given NAS procedure, with the number of re-attempts being dependent on implementation and/or network policies.

The above is applicable to both EMM and ESM procedures. As an example, Figure O.2.2-x and Figure O.2.2-y shows the execution of an attach with PDN connectivity request procedure using, respectively, two satellite coverage availability windows (i.e. one re-attempt) and three satellite coverage availability windows (i.e. two re-attempts). The illustration assumes that a single satellite operating in S&F mode is providing the service. The main difference between the two cases is whether UE authentication is performed before or after the establishment of the PDN connectivity on the ground.

Figure O.2.2-x: Illustration of a procedure for Attach with PDN connectivity completed over two satellite coverage visibility windows

Figure O.2.2-y: Illustration of a procedure for Attach with PDN connectivity completed over three satellite coverage visibility windows

# End of CHANGEs