3GPP SA WG2 Meeting #166-AdHoc-e	S2-2501022

# Electronic Meeting, 20-24 January 2025								                 (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6053 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | further details for QoS information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.501, the term “Non-3GPP Device Identifier Information” is defined to include Non-3GPP Device Identifier and QoS information, but the QoS inforamtion without clear description what parameters it may include. However the details description is captured in clause 4.15.6.15 of TS 23.502 as: 2) Service Parameters. Contains the non-3GPP Device Identifier Information which has the following parameters: - Non-3GPP Device Identifier is a generic string, which uniquely identifies a non-3GPP device behind a specific UE subscription. - QoS Reference or Individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20]. - Optionally, DNN and S-NSSAI. This may be provided by the AF or may be determined by the NEF based on the AF-Service-Identifier if not provided by the AF. - Optionally, Flow description information as specified in clause 6.1.3.6 of TS 23.503 [20] So propose to provide further clarification for QoS inforamtion, based the details in TS 502. Non-3GPP Device Identifier Information is stored in the UDR and includes a Non-3GPP Device Identifier and QoS Information. The QoS information may include following parameters: - QoS Reference or Individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [45]. - Optionally, DNN and S-NSSAI. This may be provided by the AF or may be determined by the NEF based on the AF-Service-Identifier if not provided by the AF. - Optionally, Flow description information as specified in clause 6.1.3.6 of TS 23.503 [45] |  |  |  |  |  |  |  |  |
| Summary of change: |  | To give details description for QoS inforamtion captured in TS 23.502 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | the QoS inforamtion is ambiguous without clear description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.52.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

5.52.2	Traffic identification

When a non-3GPP device is connecting to the UE, the UE may bind the Non-3GPP Device Identifier to a non-3GPP device, for the traffic of non-3GPP devices that require differentiated QoS. This binding enables the 5G System to distinguish between the traffic generated by different non-3GPP devices connected through the same UE.

NOTE 1:	How the UE identifies the non-3GPP device and binds the Non-3GPP Device Identifier to a non-3GPP device is implementation specific.

NOTE 2:	At any point in time the Non-3GPP Device Identifier can be bound to only one non-3GPP device.

Non-3GPP Device Identifier Information is stored in the UDR and includes a Non-3GPP Device Identifier and QoS Information. The QoS information may include following parameters:

- QoS Reference or Individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [45].

- Optionally, DNN and S-NSSAI. This may be provided by the AF or may be determined by the NEF based on the AF-Service-Identifier if not provided by the AF.

- Optionally, Flow description information as specified in clause 6.1.3.6 of TS 23.503 [45]

* * * End of Change * * *