3GPP SA WG2 Meeting #166-AdHoc-e                                                        	S2-2501025

Electronic Meeting, 20-24 January 2025

Source:	Samsung

Title:	Discussion on List of satelliteID(s).

Document for:	Discussion

Agenda item:	5GSAT_Ph3-ARC

# 1. Introduction

23.401 have below EN and there is an exception received on determining how to handle the List of satellite IDs:

Editor's note:	How to handle the specification of the "List of satelliteID(s)" and "S&F Monitoring List" as per conclusions in TR 23.700-29 [96] is FFS.

This paper discusses and proposes the potential way forwards.

# 2. Discussion

Issue mainly from split MME perspective:

a)	UE is registered in S&F mode.

b)	UE context is uploaded on SAT-1, SAT-3 and not uploaded on SAT-2 and SAT-4 by the ground-MME.

c)	Thus, UE is indicated list of satellite IDs as SAT-1, SAT-3.

d)	Now if UE access on the cells broadcasting satellite ID other than SAT-1, SAT-3 using service request procedure then network on board the satellite does not have the UE context and network is not aware if the UE supports S&F feature. Following legacy principles the network will indicate reject cause# 9	(UE identity cannot be derived by the network) to the UE. This results in the loss of UE context at the UE side as UE enters EMM-DEREGISTERED state and UE may trigger Attach procedure again on the SAT-2.

## 2.1 Way forward-1:

Observation-1: If UE access from a cell not in the list of satellite IDs indicated to the UE, then UE context will be lost and UE will end up triggering attach procedure again though the UE context is already available on another set of satellites. This will trigger unnecessary signalling on network to re-execute the attach procedure of already registered UE and services to the UE will be delayed.

Observation-2: Ideally UE should be restricted to access only from the cells which are part of list of satellite IDs indicated to it.

Observation-3: Some companies raise concerns on observation-2 that UE may be stuck in an area where it may never find the cell part of list of satellite IDs.

Observation-4: Concerns raised by some companies as observation-3 is not correct because the list of satellite IDs should be applied in registration area(i.e. TAI list) thus if UE moves out of registration area it will automatically trigger TAU procedure ignoring the list of satellite IDs stored in the UE.

Observation-4: Another Concerns raised by some companies in observation-2 from the perspective of monitoring list where the expected behaviour of the list is only to indicate to UE where the mobile terminated data can be expected for the UE so that UE can perform power saving by not monitoring other satellites. Thus by implementing observation-2 UEs which can access for MO data are also not allowed and this can delay in sending MO data.

Observation-5: This concern from observation-4 that UE will be delayed to send MO data for S&F is not a strong argument because:

a)	We all know that using S&F there will be delay so this additional delay to send MO data in allowed list of satellites does not seems to cost much.

b)	It is just a wishful imagination that operators will implement a complicated list of satellite IDs feature only to save power in the UE – without any benefit to operators.

Proposal-1: Following the observation-1 to observation-5, we propose to have mandatory behaviour of the UE to access only the cells of satellites which are indicated to the UE.

## 2.2 Way forward-2:

Another potential way forward is:

a)	UE is registered in S&F mode.

b)	UE context is uploaded on SAT-1, SAT-3 and not uploaded on SAT-2 and SAT-4 by the ground-MME.

c)	Thus, UE is indicated list of satellite IDs as SAT-1, SAT-3.

d)	Now if UE access on the cells broadcasting satellite ID other than SAT-1, SAT-3 using service request procedure then network on board the satellite rejects with S&F reject cause to the UE. When UE receives the S&F reject cause, then UE remain in the state it was before triggering the NAS procedure. This will not impact the UE context thus it will not trigger any attach procedure with the network and UE will attempt on the next satellite again (if it is in the registered state).

Observation-6: In way forward-2, the concern raised by one company was that when UE triggers service request procedure, network is not aware if the UE supports S&F feature. Thus network cannot give S&F reject cause to the UE and it will have to use #9 reject cause which will result in UE context being lost.

Proposal-2: To solve the concern raised in observation-6, i.e. network not knowing about the UE capability on S&F, we propose that UE when access a cell not part of list of satellite IDs provided to it, instead of sending service request message it can send TAU message and indicate its S&F capability.

# 3. Conclusion & proposal

The two proposals discussed in this paper are proposed as part of S2-2501026(Proposal-1) and S2-2501028(Proposal-2).

# 4. Reference

NA.