3GPP SA WG2 Meeting #166-AdHoc-e	S2-2501026

# Electronic Meeting, 20-24 January 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3876 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling list of satellite IDs (Opt-1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in DP S2-2501025, this paper proposes the proposal-1. i.e. UE will access only the cells which are broadcasting the satellite IDs part of list of satellite IDs indicated to the UE. The UE will not access other cells in the registration area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the List of satellite IDs feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The List of satellite IDs feature not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

### 4.13.9	Support of Store and Forward Satellite Operation

#### 4.13.9.1	General

The Store and Forward Satellite Operation applies in E-UTRAN with satellite access and is suitable for delay-tolerant communication services (e.g. CIoT/MTC, SMS, etc.). If the satellite does not support Store and Forward Satellite operation and there is no feeder link connection to the ground network, then the satellite cannot provide any service to any UEs. When both the service link and feeder link are available, all services can be provided. To support Store and Forward Satellite operation, some network functionality needs to be deployed on the satellite payload.

Example deployments are described in Annex O.

Figure 4.13.9.1-1: Store and Forward (S&F) Satellite operation

NOTE 1:	In the Figure 4.13.9.1-1, the NTN GW belongs to the satellite deployment and is not subject to 3GPP specifications.

In Store and Forward Satellite operation, the end-to-end exchange of signalling/data traffic is handled in a sequence of steps reflecting the intermittent availability of the service link when the satellite can exchange data with the UE and of the intermittent availability of the ground link when the satellite can exchange data with the core network. This is depicted in Figure 4.13.9.1.1.

For example, a simple sequence of events for the transmission of data from the UE to a server on the ground may involve two steps: firstly, when service link is available (e.g. location L1 in Figure 4.13.9.1.1) the UE sends the data to the satellite. Subsequently (e.g. location L2 in Figure 4.13.9.1.1), the satellite carrying the payload connects to the ground network and delivers the UE data to the network.

Downlink data can be stored onto the same or a different satellite and provided to the UE later using the first step and the process is repeated.

When a satellite supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in S&F Mode and if a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

NOTE 2:	Some deployments cannot support all system features, for example they cannot support user plane establishment and user plane data transfer, when operating in S&F Mode in the split MME deployment described in Annex O.

The MME may adapt periodic update timer, mobile reachable timer and Implicit Detach timer to the fact that the UE is served in S&F mode.

When an MME is operating in S&F Mode:

-	If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, if the UE supports Store and Forward Satellite operation, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation.

-	If a UE indicates Store and Forward capability, the MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List, or both when accepting or rejecting a NAS procedure. The S&F Monitoring List is a list of satellites (identified by satellite ID(s)) for the same PLMN with which the UE can attempt to retry the NAS procedure, in the future. The Satellite IDs are based on the SIB information broadcast by eNB.

NOTE 3:	How the MME determines the S&F Wait Timer is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc.

When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause.

If the S&F Monitoring List was provided to the UE,

1)	the UE may (re)attempt a NAS procedure on any of the satellites in the S&F Monitoring List;

2)	the UE shall not attempt a NAS procedure on the satellites not part of the S&F Monitoring List;

3)	the S&F Monitoring List are valid only in the registration area (i.e. TAI list) and the UE attempts TAU procedure when it moves out of TAI list irrespective of the satellite ID broadcasted(if any) by the network.

NOTE 4:	If the UE receives a reject cause indicating the NAS rejection is due to S&F operation, the UE continue to remain in the EMM state it was before triggering the NAS procedure.

The MME may provide S&F Monitoring List to the UE as part of detach procedure.

If the UE did not indicate Store and Forward capability in NAS signalling, if the network determines to reject the UE, the network shall reject the UE request with a cause non-specific to S&F Mode.

The MME may indicate to UE the Uplink S&F estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

NOTE 5:	The Uplink S&F estimated Delivery time is the estimated/expected time required to deliver the data to the ground network element from the time the data is sent by the UE.

The MME may trigger a Update Location procedure with the HSS along with the authentication procedure to fetch subscription information from the HSS. The MME may indicate the timestamp information to HSS during the Update Location procedure. This timestamp information shall be used by the HSS to ensure that newer location for that UE is not cancelled. If MME makes an Update Location Request before the completion of the authentication procedure, it shall include an indication that this Location Update is provisional, i.e. the HSS shall not consider the UE as registered until it receives the final Update Location Request (without indication that the Location Update is provisional). The timestamp information is the time when the on-board of satellite MME part has received the NAS procedure from the UE. The HSS compares the timestamp received in the Update Location Request with any stored timestamp of a previous Update Location Request and determines whether to accept or reject the request. If the received Update Location Request does not include a timestamp, the HSS assumes the present time as the timestamp of the received Update Location Request. The HSS shall reject the Update Location Request, if the timestamp associated with this request is older than the stored timestamp. If the HSS accepts the Update Location Request, the HSS shall store the timestamp associated with the latest Update Location Request. If the HSS does not support the timestamp, the MME shall reject a UE Attach request for S&F Mode.

NOTE 6:	The timestamp information is used by the HSS to ensure that interactions with MMEs for a UE are handled in the correct order.

NOTE 7:	The absence of timestamp in the Update Location Request can occur e.g. when the UE is connecting from a network that does not operate in S&F mode.

The EPS may expose whether a UE is in S&F Mode and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE. This is further described in clause 5.6.3.10 of TS 23.682 [74].

*************************** End of Changes ****************************