3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2501052

20 - 24 January, 2025, Online

# Title:	[DRAFT] Reply LS on handling of Nsmf events targeting anyUe for I-SMF

# Response to:	(S2-2500036/C3-246487) LS on handling of Nsmf events targeting anyUE for I-SMF

# Release:	Rel-18

# Work Item:	UPEAS

Source:	Samsung (TO BE SA2)

To:	CT3

Cc:	CT4

Contact Person:

Name:	Naman Gupta

E-mail Address:	nmn.gupta@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank CT3 regarding LS on handling of Nsmf events targeting any UE for I-SMF

CT3 asked:

"

An SMF receiving Nsmf_EventExposure Subscription request to subscribe to UPF events  for "Any UE", does not consider PDU sessions for which it is acting as I-SMF when it selects the PDU session(s) and the UPFs it has to send the request to

"

CT3 understands that this avoids a duplication of subscriptions and wants to ask the following question:

Question1: Is this issue specific to UPF events or can it apply to any event type that is supported by the I-SMF?

SA2 Reply:

In case the particular event is to be observed by SMF (e.g. PDU Session Release, UP Path change, QFI allocation etc.) as described in TS 29.502, the reporting has to be done by SMF itself and similar behaviour as UPF events apply, i.e. an SMF does not consider the PDU Session(s) for which it is acting as I-SMF.

For events like Downlink Data delivery Status (e.g. in cases buffering is done at I-SMF) that can be observed and reported by I-SMF, for such events SMF may also need to convey updated N4 information to the corresponding I-UPF. Thus for such events also, SA2 has agreed to have the similar behaviour as of UPF events, i.e. SMF can subscribe to I-SMF via Nsmf_EventExposureSubscribe operation; and when the target is “Any UE” an SMF does not consider the PDU Session(s) for which it is acting as I-SMF.

Thus the following statement applies: An SMF receiving Nsmf_EventExposure Subscription request to subscribe to UPF/SMF events  for "Any UE", does not consider PDU sessions for which it is acting as I-SMF when it selects the PDU session(s) and optionally the UPFs it has to send the request to.

2. Actions:

To SA1 group.

ACTION: 	SA2 kindly asks CT3 to take above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting#167      	February 17 – 21, 2025                         Athens, Greece

TSG-SA2 Meeting#168                      April 07 -11, 2025                                  Goteburg, Sweden