3GPP TSG-SA WG2 Meeting #166-e	S2-2501092

# Elbonia, 20-24 January, 2025								                      (revision of S2-2500785)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1564 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call setup update for UE-Satellite-UE communication in IMS with IMS-AGW on board |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, NTT DOCOMO, China Telecom, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The initial call setup procedure of UE-satellite-UE communication agreed in SA2-165 meeting has been captured in TS 23.228. Editor's notes in the call setup procedure are not solved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Solve issues of the Editor’s notes based on the convergent content in the S2-2412892 discussed in SA2-166 F2F meeting. - UE subscription is not considered in this release for UE-sat-UE. - The procedure has no impacts on H.248. - Early media traffic is discussed separately. - P-CSCF subscribes to Notification of User Plane Management Events for satellite change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The call step procedure is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.5, AE.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

AE.5	Procedures for optimizing media routing

AE.5.1	At call setup

When the originating P-CSCF receives the initial INVITE request from the originating UE using regenerative NR satellite access, it selects the IMS-AGW on ground and triggers media negotiation by sending SDP offer to the terminating side indicating that satellite access is used on the originating side. If the terminating P-CSCF activates UE-satellite-UE communication, the originating P-CSCF re-selects an IMS-AGW on satellite. See Figure AE.5.1-1 that describes the procedure. In the procedure, UE A and P-CSCF A represent originating side function or entity, UE B and P-CSCF B represent terminating side function or entity.

NOTE 1:	To support the UE-satellite-UE communication, P-CSCFs (AF) need to support N5 interface for AF influence traffic routing and user plane event subscription/notification.

Figure AE.5.1-1: Session establishment procedure with activation of optimizing media routing

1.	UE A sends a SIP INVITE request, containing an initial SDP offer, to P-CSCF A. The access information in the SIP INVITE request indicates that the UE is accessing from NR satellites access as defined in TS 24.229 [10a].

2.	After receiving the SIP INVITE request indicating NR satellite access, if UE A is not roaming and the P-CSCF A supports UE-Satellite-UE communication, the P-CSCF requests Access Network Information for UE A from the PCF via Npcf_PolicyAuthorisation_Subscribe service operation. The PCF obtains the Access Network Information from the SMF via Npcf_SMPolicyControl_UpdateNotify service operation. This is subsequently conveyed to the P-CSCF via Npcf_PolicyAuthorization_Notify service operation.

3.	The P-CSCF A allocates an IMS-AGW on the ground as default. P-CSCF A interacts with IMS-AGW on the ground to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 4-20 are performed if the UE-Satellite-UE communication is possible based on the decision in step 2 (i.e. UE A is using NR regenerative payload satellite access and is not roaming). Otherwise, the P-CSCF selects an IMS-AGW on the ground and SIP message routing procedures with IMS-AGW on ground are performed (details have been omitted from the diagram).

4.	P-CSCF A updates the SIP INVITE request inserting the identification of the satellite serving UE A and updates SDP offer with the IMS-AGW transport addresses allocated in step 3. P-CSCF A forwards the updated SIP INVITE request to the terminating side via IMS core (details have been omitted from the diagram).

5.	After receiving the SIP INVITE request which contains a satellite identification indicating that UE A is using regenerative satellite access, if UE B is not roaming and the P-CSCF B supports UE-Satellite-UE communication, the P-CSCF B retrieves Access Network Information for UE B from PCF via Npcf_PolicyAuthorization_Subscribe service operation.. The PCF obtains Access Network Information from the SMF via Npcf_SMPolicyControl_UpdateNotify service operation. This is subsequently conveyed to the P-CSCF via Npcf_PolicyAuthorization_Notify service operation.

6.	Based on the Access Network Information received at step 5, if UE B is using regenerative payload satellite access and if the two satellites identified in step 4 and 5 are the same or connected with ISL(s), P-CSCF B determines to activate the UE-Satellite-UE communication in IMS.

If UE-Satellite-UE communications in IMS is not activated, P-CSCF B selects an IMS-AGW on ground for UE B; otherwise, P-CSCF B selects an IMS-AGW on a satellite for UE B.

NOTE 2:	How the terminating P-CSCF uses the satellite identifications to derive whether the two satellites are connected with ISL(s) is and are equipped with UPF and IMS-AGW is based on implementation or  operators' policy e.g. by querying transport network system or based on node level information reported by the IMS-AGW deployed on the satellites which reports whether the two satellite are connected or not.

6a-6b.	Based on the decision in step 6, P-CSCF B interacts with IMS-AGW on-board satellite or IMS-AGW on ground to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 7-20 are performed if the UE-Satellite-UE communication is activated. Otherwise, SIP message routing procedures with IMS-AGW on the ground should be performed after P-CSCF B selecting a ground IMS-AGW for usage (details have been omitted from the diagram).

7-9.	P-CSCF B forwards the updated SIP INVITE request to UE B. UE B returns a 18X response with SDP Answer as normal routing SIP message routing procedures.

9a-9b.	After receiving the SDP Answer, P-CSCF B interacts with the IMS-AGW on-board the satellite to allocate transport resources based on the mechanism defined in TS 23.334 [74].

10.	P-CSCF B instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Create service operation.

To indicate the SMF to insert a UL CL/BP and L-PSA UPF for IMS PDU session to enable UE-Satellite-UE communication, the P-CSCF B should send following parameters via the request to PCF based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which contains the IP address of the IMS-AGW on-board the satellite;

-	A DNAI associated with the IMS-AGW on-board the satellite and corresponding N6 traffic routing information.

10a. To be notified of satellite change and perform IMS-AGW relocation at change of satellite as described in clause AE.5.2, P-CSCF B subscribes Notification of User Plane Management Events (i.e., UP path change) via Npcf_PolicyAuthorisation_Update service operation to the PCF which on turn subscribes the events from SMF via Npcf_SMPolicyControl_UpdateNotify service operation and receives the events from the SMF via Nsmf_EventExposure_Notify service operation. The P-CSCF B inserts the following parameters in its subscription request:

-	Early and late notifications about UP path management events;

-	AF transaction identifier.

NOTE 3: If Rx is used by P-CSCF, implicit subscription is assumed as part of PCC rules setting.

11.	P-CSCF B updates the 18X response to carry the identifier of the satellite serving UE B to indicate that UE-Satellite-UE communication is activated. Then, P-CSCF B returns the updated 18X response to originating side via IMS core (details have been omitted from the diagram).

12.	Based on the identification of the satellite serving UE B in step 11, P-CSCF A determines to perform UE-satellite-UE communication and select an IMS-AGW on-board satellite.

12a-12c.	P-CSCF A interacts with IMS-AGW on-board satellite to allocate transport resources for both IMS access and IMS Core network sides. P-CSCF A also releases the IMS-AGW on ground.

13.	P-CSCF A instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Create service operation.

To indicate to the SMF to insert a UL CL/BP and L-PSA UPF for the IMS PDU session to enable UE-Satellite-UE communication in IMS, P-CSCF B should send following parameters via the request to PCF based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which contains the IP address of the IMS-AGW on-board satellite;

-	A DNAI associated with the IMS-AGW on-board satellite and corresponding N6 traffic routing information.

13a. To be notified of satellite change and perform IMS-AGW relocation at change of satellite as described in clause AE.5.2, P-CSCF A subscribes to Notification of User Plane Management Events (i.e., UP path change) via Npcf_PolicyAuthorisation_Update service operation to the PCF which on turn subscribes the events from SMF via Npcf_SMPolicyControl_UpdateNotify service operation and receives the events from the SMF via Nsmf_EventExposure_Notify service operation. The P-CSCF A inserts the following parameters in its subscription request:

-	Early and late notifications about UP path management events;

-	AF transaction identifier.

NOTE 4: If Rx is used by P-CSCF, implicit subscription is assumed as part of PCC rules setting.

14.	P-CSCF A updates the SDP answer in the 18X response with the IMS-AGW transport addresses allocated in step 12, and forwards the updated 18X response to UE A.

15.	UE A acknowledges the 18X Response and sends the Response Confirmation to P-CSCF A.

16-17.	P-CSCF A forwards the Response Confirmation to P-CSCF B. The Response Confirmation includes SDP offer modified/generated by P-CSCF A which contains the IMS-AGW transport address obtained in step 12. Upon receiving the SDP offer, P-CSCF B updates the allocated IMS-AGW on the satellite for UE B with the transport address in the SDP offer.

18.	P-CSCF B sends the Response Confirmation to UE B.

19.	Procedure continues to setup the call as defined in clauses 5.6 and 5.7.

## ---End of the Change---