3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2501226

# Online, , 20th Jan 2025 - 24th Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5960 | rev | 1 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE triggering request for change of QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The non-3GPP device identifiers were introduced for achieving differentiated QoS handling for traffic associated with non-3GPP device connecting to a UE. The UE sends the Non-3GPP Device Identifier and may also send the addressing information to the SMF in session management signalling. It was discussed in the previous meetings that there is no description of how the UE and/or AF triggers the UE to request for change of QoS Flows. The expected behaviour is that user is provisioning non-3GPP device identifiers and associated QoS requirements to the 5GC and also provisions the same non-3GPP device identifiers in the UE through application layer mechanism. The AF has to provision those non-3GPP device identifiers to the UE which have been accepted by 5GC (and entered into the UDR). There is no further need for any triggering mechanism through 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NOTE1 in the clause 5.22.1 is updated to state that how non-3GPP device identifiers are provisioned in the 5GC and in the UE is implementation specific. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would not be clear how the mechanism for requesting differentiated QoS for non-3GPP device identifiers is triggered by the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.52.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>>>> FIRST CHANGE <<<<

### 5.52.2	Traffic identification

When a non-3GPP device is connecting to the UE, the UE may bind the Non-3GPP Device Identifier to a non-3GPP device, for the traffic of non-3GPP devices that require differentiated QoS. This binding enables the 5G System to distinguish between the traffic generated by different non-3GPP devices connected through the same UE.

NOTE 1:	How the UE identifies the non-3GPP device and binds the Non-3GPP Device Identifier to a non-3GPP device is implementation specific. How the Non-3GPP Device Identifier(s) that are associated with the UE’s SUPI in the 5GC are known to the UE is implementation specific.

NOTE 2:	At any point in time the Non-3GPP Device Identifier can be bound to only one non-3GPP device.

Non-3GPP Device Identifier Information is stored in the UDR and includes a Non-3GPP Device Identifier and QoS Information.

>>>>>> END OF CHANGE <<<<