3GPP TSG-WG SA2 Meeting #166-AH-e	S2-2501251

E-meeting, 20th – 24th January 2025	(revision of S2-2500798r05)

Source:	HONOR, Nokia

Title:	KI#1 Conclusion update to address the ENs about assistance information (Cat 2)

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Discuss and update the interim conclusions for KI#1 Cat2 ENs.

- Discussion

In SA2#166-AHe Ambient IoT pre-meeting call, open issues in TR 23.700-13 are categorised in:

- Resolve in the study phase, to reach conclusion principles --- category 1 issues

- Convert the EN to note or remove the EN --- category 2 issues

- No more work in this release --- category 3 issues

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

According to the TR 23.700-13 8.1.1, it’s mentioned that:

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

The assistance information provided by CN is mainly used for the AIOT reader to schedule the response message in D2R direction.

From RAN2 WG perspective, the reader is useful to aware that whether there is an expected response in D2R direction or not. RAN2 WG provides the view in R2-2411232, which shows "If the expected size of the response message is not available at the reader, the reader should at least know whether a message in D2R direction is generated in response to the R2D message.".

Therefore, if the D2R response is not expected in the specific AIoT service request, the CN is expected to give an indication on that (e.g. either explicitly or implicitly) to RAN/UE reader. If an indication is explicit, for example, the CN may tell the reader that there is no D2R response expected. If an indication is implicit, for example, the CN may tell the reader that D2R message size equals zero.

Observation 1: If the AF gives the additional information about whether the D2R response is expected (either explicitly or implicitly), the AIOTF should regard that additional information as the assistance information and provide it to RAN/UE Reader.

Proposal 1: The AIOTF may indicate that whether a message in D2R direction is generated in response to the R2D message based on the AF request.

During RAN2 WG discussion (e.g. agreements in RAN2#128), if further information on AIoT service type like command type info is available and is visible to the reader, the reader may infer the expected D2R message size from that. Therefore, if the AIOTF provides the approximate D2R message size based on AF request, no further information regarding the AIOT service type is required.

Observation 2: If the AIOTF provides the approximate D2R message size based on AF request, no further information regarding the AIOT service type is required.

Proposal 2: Add the note to clarify the further information on AIoT service type and delete EN.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

8.1	Interim Conclusion on Key Issue #1

8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the BS readers or AIOT RAN nodes. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

Editor’s note:	Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 needs coordination between SA WG2 and RAN WG(s).

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to AIoT RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

NOTE 4:	Further assistance information can be added during the normative phase and in cooperation with other WGs if necessary.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

5.	The network may manage the AIOT device related information (e.g. device context information), includes the AIOT device permanent ID, and optionally the last known reader information.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:	Where to store the AIOT device related information is FFS.

# * * * * End of changes * * * *