3GPP TSG-WG SA2 Meeting #166AHE	S2-2501323

# E-meeting, January 20 – 24, 2025		(revision of S2-2500755r01)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5998 | rev | 1 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MWAB-UE Mobility Restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to LS from RAN3 (S2-2500052/R3-247909), RAN3 has agreed the following solution to prevent multi-hop backhauling upon WAB-MT HO: For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling. Also, MWAB mobility is added to the spec during SA2#164 meeting. When an MWAB moves from a gNB to another gNB, the target gNB needs to be another normal gNB. A Normal gNB does not include MWAB-gNB, IAB or MBSR. If the target gNB is MWAB-gNB, IAB or MBSR, then 2 or more NR Uu hops are involved in the MWAB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new Mobility Restrictions to support for Mobility of MWAB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobility regarding MWAB is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.49.5.3	Impact of MWAB mobility

The mobility of MWAB includes the MWAB-UE and the MWAB-gNB aspect.

-	The mobility of the MWAB-UE reuses the exiting UE mobility procedures specified for normal UE, subject to constraints specified in TS 38.401 [42].

-	The MWAB-gNB can obtain a new configuration from OAM server of MWAB Broadcasted PLMN during mobility.

-	The MWAB-UE may request new or modify existing BH PDU Sessions for the corresponding N2, N3 and Xn interfaces, upon request from the MWAB-gNB as it moves in the network and the MWAB-gNB updates the TNL association as specified in clause 5.21.1 and TS 38.401 [42] to use the new TNL information associated with the new BH PDU Session(s) as needed.

-	SSC mode 3 may be applied to the BH PDU Sessions to minimize traffic interruption when UPF re-allocation for these BH PDU Sessions is required during the MWAB mobility.

-	When it is required to change AMF for the UEs served by the MWAB-gNB upon mobility of a MWAB into a new region, the TAC(s) and Cell ID(s) the MWAB-gNB announces also change.

# * * * * End of changes * * * *