# 3GPP TSG-SA WG2 Meeting #166-Ad Hoc-e	S2-2501324

eMeeting, 20 - 24 January 2025	(Revision of S2-2500363r05)

Title:	Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI)

Response to:	S2-2500052/R3-247909

Release:	Rel-19

# Work Item:	     FS_VMR_Ph2, VMR_Ph2

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR 5933 (S2-2501321) and CR 5968 (S2-2501322)

1. Overall Description:

SA2 thanks RAN3 for the LS (S2-2500052/R3-247909) on FS_VMR_Ph2 solution impacts to RAN with focus on Access Control and Additional ULI.

The LS highlights the issue as stated below:

1. Overall Description:

RAN3 thanks SA2 for the LS on FS_VMR_Ph2 (S2-2407345/R3-244019) and would like to provide the feedback as following:

Issue-1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

Reply to Issue-1a:

RAN3 has agreed the following solution to prevent multi-hop backhauling upon WAB-MT HO:

For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

The discussion on RAN based approach for preventing multi-hop backhauling at the WAB-MT’s initial access is still ongoing in RAN3.

Issue-2:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

Reply to Issue-2:

RAN3 discussed this issue and the following WA was achieved:

Additional ULI for WAB consists of TAI and NR CGI, which are determined by the WAB-node, reflecting the WAB-node’s physical location. This solution is compliant with Option 1 and Option 3. It is up to SA2 to support one of Option 1 and Option 3 or both.

RAN3 is still discussing whether there are issues with applying this solution to WAB-MT access via NTN.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into account.

SA2 discussed the RAN3 reply to issue-1a and has agreed 23.501 CR 5933.

SA2 discussed the RAN3 Reply to issue-2 and has agreed 23.501 CR 5968.

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks RAN3 to take above information into consideration and update the corresponding specifications.

3. Date of Next SA2 Meeting:

SA2#167   				17 – 21 February, 2025			        Athens, GR

SA2#168   				 07 – 11 April, 2025			        	     Goteborg, SE