TSG SA WG2 Meeting #166Ad Hoc	S2-2501353

E-meeting, January 20-24, 2025										                  (is revision of S2-2412944)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5651 | rev | 7 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.501 Data boosting triggered by AS/AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Tencent, Tencent Cloud, CATT, China Telecom, Meta USA, CMCC, Xiaomi, Samsung, Huawei, HiSilicon, Ericsson, Nokia, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-1-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is agreed in 23.700-70 KI#5 conclusion to support data boosting (expedited transfer) triggered by AS/AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following enhancements are introduced: - Add description of AS/AF triggered expedited transfer in 5.37.10.x - Add “Expedited Transfer Indication” in PDR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The data boosting (expedited transfer) scheme is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.10.x (new), 5.8.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.37.10.x	Expedited Data Transfer with reflective QoS

Expedited Data Transfer triggered by AS/AF is used to expedite the data transfer of larger payload for IP flow(s) of XR application. Expedited Data Transfer may be used for non-GBR SDF only.

To enable Expedited Data Transfer, the AF provides two media flows associated with different QoS requirements, one QoS requirement is associated with Expedited Transfer Indication set to TRUE and the other one is set to FALSE.

Expedited Data Transfer requires the PSA UPF detects the “Expedited Transfer Indication” (e.g. contained in N6 metadata or in RTP Header Extensions) in the downlink media flow. To enable expedited transfer for encrypted traffic, the AS also provides information to enable transferring media related information over N6 (e.g. in clause 5.37.9 or in RTP Header Extensions). To enable expedited transfer for non-encrypted traffic, the AF may provide protocol description information.

For the UE that supports Reflective QoS, the PCF generates two PCC rules for the two SDFs with Expedited Transfer Indication and with different 5QIs based on the AF request. In the PCC rules, for the SDF with Expedited Transfer Indication set to TRUE, the Reflective QoS control is enabled, and for SDF with Expedited Transfer Indication set to FALSE, it is up to operator configuration to enable the Reflective QoS control or not.

If the PCF does not enable Reflective QoS control in the PCC rule with Expedited Transfer Indication set to FALSE, then the PCF shall set the precedence of the PCC rule to a value higher than the reserved range for PCC rules subject to Reflective QoS as specified in 3GPP TS 24.501 clause 6.2.5.1.1.3. The SMF generates two PDRs including the Expedited Transfer Indication in the PDI. Expedited Transfer Indication is set to TRUE in the one PDR and to FALSE in the other.  If UPF receives the PDRs with the Expedited Transfer Indication it shall also detect “Expedited Transfer Indication” in the downlink media flow (as described in clause 5.37.9x for encrypted traffic, or in RTP Header Extension/transport protocol header as defined in TS 26.522) to process the DL packets based on one of these two PDRs. If UE does not support Reflective QoS, the AF request is rejected with an indication that the Expedited Data Transfer is not supported.

NOTE: In this release, Expedited Data Transfer is only supported when UL and DL PDU set QoS parameters and PDU QoS requirements including PDB and PER are the same.

# * * * Next Change * * * *

#### 5.8.5.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.5.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR. NOTE 5. |  |
| Rule ID |  | Unique identifier to identify this rule. |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied. |  |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| Detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3). | packet filter set, application identifier, Ethernet PDU Session |
| Information. NOTE 4. | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet. | Information and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID. | interface for incoming packets |
|  | Packet Filter Set | Details see clause 5.7.6. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application identifier |  | from core side (i.e. down-link), |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI. | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. | DN), or from "5G VN internal" (i.e. local switch). |
|  | Framed Route Information | Refers to Framed Routes defined in clause 5.6.14. | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
|  | FQDN Filter for DNS Query | Contains one or more FQDN, FQDN range and/or any FQDN. | The FQDN or FQDN range only used for detection of plain DNS Query message (i.e. not subject to ciphering). The usage is described in TS 23.548 [130]. |
|  | Protocol Description | Indicates service protocol used by the flow (NOTE 8), (NOTE 9). |  |
|  | Expedited Transfer Indication | For PDR matching. Contains the value of the “Expedited Transfer Indication” in the downlink media flow. Can be set to TRUE or FALSE (NOTE X). |  |
| Packet replication and detection carry on information | Packet replication skip information NOTE 7 | Contains UE address indication or N19/N6 indication. If the packet matches the packet replication skip information, i.e. source address of the packet is the UE address or the packet has been received on the interface in the packet replication skip information, the UP function neither creates a copy of the packet nor applies the corresponding processing (i.e. FAR, QER, URR). Otherwise the UPF performs a copy and applies the corresponding processing (i.e. FAR, QER, URR). |  |
| NOTE 6 | Carry on indication | Instructs the UP function to continue the packet detection process, i.e. lookup of the other PDRs. |  |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. - UPF "local switch", N6-based forwarding and N19 forwarding is used for different 5G LAN groups. - UPF "local switch" may be used for DNN/S-NSSAI dedicated for PIN. NOTE 2: Either a FAR ID or a MAR ID is included, not both. NOTE 3: The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. NOTE 4: When in the architecture defined in clause 5.34, a PDR is sent over N16a from SMF to I-SMF, the Packet Detection Information may indicate that CN tunnel info is to be locally determined. This is further defined in clause 5.34.6. NOTE 5: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 6: Needed in the case of support for broadcast/multicast traffic forwarding using packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2. NOTE 7: Needed in the case of packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2, to prevent UPF from sending the broadcast/multicast packets back to the source UE or source N19/N6. NOTE 8: Not for PDR matching. In the case of unencrypted traffic, it may be provided to assist PDU Set identification when PDU Set Identification and marking applies to the PDR and/or to assist identification of the last packet of the Data burst in downlink when End of Data Burst identification and marking in downlink applies to the PDR. It may assist determination of the Data Burst Size of the data burst when Data Burst Size determination and marking in downlink applies to the PDR. See TS 26.522 [179]. It may assist determination of the Time to Next Burst when time to next data burst determination and marking in downlink applies to the PDR. See TS 26.522 [179]. NOTE 9: Not for PDR matching. In the case of end-to-end encrypted traffic, it may be provided to indicate how to retrieve media related information as specified in clause 5.37.9. NOTE X: Needed in the case of Expedited Data Transfer with reflective QoS as described in clause 5.37.10.x. |  |  |  |

# * * * End of changes * * * *